ECS3510-28T/52T

Firmware Version 1.5.1.18

INTRODUCTION:

ECS3510-28T/52T supports 24/48 FE ports and 4 Gigabit combo ports, which are comprised of an RJ45 and a SFP.* 10/100/1000 CU built-in associated Mini-GBIC slots

Using IP Clustering for stacking which is a method of grouping switches regardless of geographical location in order to enable centralized management access to the switches through a single management point. This technology provides some of the benefits of stackable switches without requiring direct stacking or connections between switch units. It also allows different kinds of switches to be grouped together, as long as they have the embedded clustering software.

It offers advanced administration through a user-friendly browser interface. QoS and VLAN Supports allows greater scalability and Supports for business-critical applications. Port can be trunked to create a high bandwidth pipe to the network.

It also has comprehensive network management functions, such as Spanning Tree protocol for standard bridging, SNMP, RMON and security features.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

Loader:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.0.6.0 | Formal release | 2014/06/04 |
| Patch Version | 1.0.2.4 | Formal release | 2014/05/30 |
| Previous Version | 1.0.4.0 | Formal release | 2014/02/19 |
| Previous Version | 1.0.2.3 | Formal release | 2013/05/15 |
| Previous Version | 1.0.2.2 | Formal release | 2012/11/15 |
| Previous Version | 1.0.2.1 | Formal release | 2012/10/30 |
| Previous Version | 1.0.0.3 | Formal release | 2012/09/14 |
| Previous Version | 1.0.0.2 | Formal release | 2012/08/23 |
| Previous Version | 1.0.0.1 | Formal release | 2012/06/06 |

Runtime:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Phase 5 |  |  |  |
| Patch Version | 1.5.1.18 | Patch version | 2015/12/30 |
| Patch Version | 1.5.1.17 | Patch version | 2015/10/14 |
| Patch Version | 1.5.1.16 | Patch version | 2015/09/23 |
| Patch Version | 1.5.1.13 | Patch version | 2015/09/8 |
| Patch | 1.5.1.11 | Formal release | 2015/06/25 |
| Test Version | 1.5.1.10 | For test | 2015/05/21 |
| Test Version | 1.5.1.8 | For test | 2015/03/30 |
| Patch Version | 1.5.1.7 | Patch version | 2015/02/13 |
| Test Version | 1.5.1.6 | For test | 2014/12/22 |
| Test Version | 1.5.1.5 | For test | 2014/12/11 |
| Test Version | 1.5.1.4 | For test | 2014/11/14 |
| Current Version | 1.5.1.3 | Formal release | 2014/10/31 |
| Test Version | 1.5.1.2 | NTC test | 2014/09/24 |
| Previous Version | 1.5.0.0 | Formal release | 2014/07/16 |
| Test Version | 1.4.9.2 | NTC test | 2014/07/09 |
| Test Version | 1.4.9.1 | NTC test | 2014/06/24 |
| Test Version | 1.4.9.0 | NTC test | 2014/06/04 |
| Phase 4 |  |  |  |
| Patch Version | 1.4.2.7 | Formal release | 2014/05/30 |
| Patch Version | 1.4.2.6 | Formal release | 2014/04/01 |
| Patch Version | 1.4.2.5 | Formal release | 2014/03/24 |
| Patch Version | 1.4.2.4 | Formal release | 2014/03/10 |
| Patch Version | 1.4.2.3 | Formal release | 2014/02/19 |
| Current Version | 1.4.2.1 | Formal release | 2014/01/23 |
| Test Version | 1.4.2.0 | NTC test | 2014/01/10 |
| Test Version | 1.4.0.2 | NTC test | 2013/12/27 |
| Test Version | 1.4.0.1 | NTC test | 2013/11/25 |
| Previous Version | 1.4.0.0 | Formal release | 2013/10/04 |
| Test Version | 1.2.7.3 | NTC test | 2013/09/30 |
| Test Version | 1.2.7.2 | NTC test | 2013/09/17 |
| Test Version | 1.2.7.0 | NTC test | 2013/08/16 |
| Patch Version | 1.2.6.2 | Formal release | 2013/07/29 |
| Previous Version | 1.2.6.1 | Formal release | 2013/07/04 |
| Previous Version | 1.2.6.0 | Release candidate | 2013/06/17 |
| Test Version | 1.2.5.0 | For testing | 2013/05/16 |
| Previous Version | 1.2.4.2 | Formal release | 2012/12/12 |
| Previous Version | 1.2.4.0 | NTC test | 2012/11/16 |
| Test Version | 1.2.2.1 | NTC test | 2012/11/12 |
| Previous Version | 1.2.2.0 | Formal release | 2012/11/07 |
| Test Version | 1.2.1.2 | NTC test | 2012/10/23 |
| Phase 2 Version | 1.2.0.0 | Formal release | 2012/09/20 |
| Test Version | 1.0.0.7 | NTC test | 2012/09/14 |
| Test Version | 1.0.0.5 | NTC test | 2012/08/23 |
| Phase 1 Version | 1.0.0.1 | Formal release | 2012/06/26 |
| Previous Version | 1.0.0.0 | NTC test | 2012/06/01 |
| Previous Version | 0.0.0.7 | NTC test | 2012/05/03 |
| Previous Version | 0.0.0.4 | Internal test | 2012/04/17 |

HARDware compatibility:

ECS3510-28T: from the 1st version

ECS3510-52T:

- after loader V1.0.2.0

- after runtime V1.2.1.0

BootPROM compatibility:

ALL

Network Management Software Support:

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Accton Product Feature List |  | ECS3510-28T/52T |
| --- | --- | --- |
| Product Configuration |  | 24/28 FE + 4 ComboG |
| Flash Memory |  | 32MB |
| SDRAM |  | 128MB |
| L2 MAC Table |  | 16K |
| Packet Buffer |  | 8Mb |
| PoE |  | No |
| Standalone/Stackable |  | Standalone |
| Layer 2 Features |  |  |
| Link Aggregation | 802.3ad with LACP | YES 1) Total 16 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 4 port/trunk GE |
|  | Cisco EtherChannel Liked | YES 1) Total 16 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 4 port/trunk GE |
|  | Unicast / Multicast traffic Balance over Trunking port | YES 1) Unicast : MAC SA+DA/ SA/DA/SIP+DIP/SIP/DIP |
| VLAN | IEEE 802.1Q Tagged Based | YES |
|  | Port-Based | YES (4K VLANs) |
|  | MAC-Based | YES |
|  | IP-Based | YES |
|  | Private VLAN (Community) | YES |
|  | Traffic Segmentation (Private VLAN Isolated) | YES |
|  | GVRP/GARP | YES |
|  | 802.1v Protocol | YES |
|  | IPv6 VLANs (one type which protocol vlan SUPPORTED) | YES |
|  | voice VLAN | YES |
|  | 4K (4094) VLANs | YES |
|  | VLAN Trunking | YES |
| Spanning Tree | IEEE 802.1D STP | YES |
|  | IEEE 802.1s (MSTP) | YES (16 instances) |
|  | IEEE 802.1w (RSTP) | YES |
|  | Spanning Tree Fast Forwarding | YES |
|  | Loopback Detection | YES |
|  | Auto Edge Port | YES |
|  | BPDU Filter/Guard | YES |
|  | Root Guard | YES |
| Broadcast/Multicast/UnknownUnicast Storm Control |  | YES |
| IGMP Snooping | IGMP Snooping v1/v2 | YES |
|  | IGMP v1/v2 querier support | YES |
|  | IGMP Immediate Leave | YES |
|  | IGMP Filtering/Throttling | YES |
|  | IGMP SNP Leave Proxy | YES |
|  | IGMP v1/v2/v3 Proxy | YES |
|  | IGMPV3 Snooping | NO |
| MVR |  | YES |
| Jumbo Frame |  | 10K |
| QinQ |  | YES |
| select QinQ |  | YES |
| EAPS |  | NO |
| Auto MDI/MDIX |  | Auto |
| Types of GBIC and SFP support (LX, SX, LH, FX) |  | Yes |
| G.8032 (ERPS) |  | YES (phase 2) |
| Non-STP loopback detection |  | YES |
| UDLD |  | YES (phase 2) |
| Digital Diagnostic Monitoring (DDM) |  | YES |
| QoS Features |  |  |
| Number of priority queues |  | 4 queues/port |
| Scheduling for priority queues | WRR Priority scheduling | Yes(Shaped Deficit WRR) |
|  | Strict Priority scheduling | YES |
|  | Hybrid | YES (Shaped Deficit WRR + Strict) |
| COS | 802.1p based COS | YES (per system) |
|  | IP TOS Precedence based COS | No |
|  | IP DSCP based COS | DSCP Only (per system) |
|  | TCP/UDP Port based COS | NO |
| Rate Limiting | Ingress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
|  | Egress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
| DiffServ |  | 1 rate 3 color (yes) 2 rate 3 color (yes) ingress and egress supported |
| Security Features |  |  |
| Static Port Security (MAC-based) |  | YES |
| Dynamic Port Security (MAC-based) |  | YES |
| MAC Limitation per port |  | YES |
| 802.1x (authenticator) | Port-Based | YES |
|  | Port-Based Authentication | YES |
|  | MAC-Based Authentication | YES |
|  | VLAN Assignment | YES |
|  | QoS Assignment (w/o ACL profile) | YES |
|  | QoS assignment w/ ACL profile | Yes |
|  | Guest VLAN | YES |
|  | Supplicant Support | YES |
|  | Supplicator | YES |
|  | EAPOL frames pass-through | Yes |
| MAC Authentication |  | YES |
| WEB Authentication (per port) |  | YES |
| ACL | Number of rules | 512 (both standard and extended modes are supported). |
|  | L2/L3/L4 | YES 1) L2 ACL filter SA/DA/VLAN 2) L3 IP SA/DA, subnet based 3) L4 TCP/UDP port 4) IPv6 |
|  | Mirror | YES |
|  | Ingress | YES |
|  | Egress | NO |
|  | Statistics | YES |
|  | Time-Based (Time Range) | YES |
| RADIUS authentication |  | YES |
| RADIUS authorization and accounting |  | YES |
| TACACS+ Authentication |  | YES |
| TACACS+ authorizaion and accounting |  | YES |
| TACACS+ 3.0 |  | NO |
| HTTPS and SSL (Secured Web) |  | YES |
| Denial of Service Protection(DoS) |  | YES |
| SSH 1.5/V2.0 (Secured Telnet session) |  | YES |
| User name password authentication | Local Authentication | YES |
|  | Remote Authentication via RADIUS/TACACS+ | YES |
| Management Interface Access Filtering | SNMP | YES |
|  | WEB | YES |
|  | TELNET | YES |
| Intrusion Lock (Link Detection) |  | YES |
| MAC filter |  | YES |
| Traffic Segmentation |  | YES |
| Management Features |  |  |
| Cisco Like CLI |  | YES |
| CLI banner |  | YES |
| CLI filtering |  | NO |
| Terminal setting |  | YES |
| Multiple Management IP Interface |  | YES |
| Web Based Management |  | YES |
| Telnet | Client | YES |
|  | Server | YES |
| Software Download/Upgrade | TFTP | YES (dual images) |
|  | Xmodem / Ymodem | YES (Runtime, Loader) |
|  | Diag support TFTP download | No |
|  | FTP | YES |
|  | HTTP | YES (single image) |
| Dual Image |  | YES |
| Configuration Download/Upload | TFTP | YES (Multiple Copies) |
|  | HTTP | YES (Multiple Copies) |
| Auto upgrade | TFTP | YES |
| Auto upgrade | FTP | YES |
| SNMP | v1 | YES |
|  | v2c | YES |
|  | v3 | YES |
| RMON | RMON I (1,2,3,9 group) | YES |
|  | RMON 2 | YES |
| MIB I/II | RFC 1213 | YES |
| BOOTP | Client | YES |
|  | Relay | No |
| DHCP | Client | YES |
|  | Relay | No |
|  | Snooping | YES |
|  | Snooping Option 82 | Yes |
|  | Dynamic Provision (via Option 66,67) | YES |
|  | Option 82 | YES |
|  | Relay option 82 | YES |
|  | Server | No |
| IP Source Guard |  | YES |
| Port Mirroring |  | YES |
| Remote Port Mirroring (RSPAN) |  | YES |
| Event/Error Logging | Syslog (Local Flash) | YES 1) 512 entries 2) 256Kbyte Flash |
|  | Remotelog (RFC3164) | YES 1) Cisco-like 2) Support up to 5 Servers |
|  | SMTP (E-mail Notification) | YES |
| OAM | IEEE 802.3ah | YES (phase 2) |
|  | IEEE 802.1ag (CFM) | YES (phase 2) |
|  | Activate/Deactivate EFM OAM per port | YES (phase 2) |
|  | Y.1731 | YES (phase 2) |
| DNS | Client | YES |
|  | Proxy | YES |
| Remote Ping |  | YES |
| SNTPv4 (RFC2030) |  | YES |
| NTP |  | YES |
| IP Clustering |  | YES (36 members) |
| LLDP(802.1ab) | Link Layer Discovery Protocol | YES |
|  | LLDP-MED (VoIP related) | YES |
|  | IEEE 802.3at | NO |
| UPnP |  | NO |
| MAC Flush |  | YES |
| sFlow |  | NO |
| Dynamic ARP inspection (DAI) |  | YES |
| Auto Traffic Control (ATC) |  | YES |
| PPPoE Intermediate Agent |  | YES |
| VLAN mirror |  | YES |
| MAC Based mirror |  | YES |
| ACL mirror |  | YES |
| ACL Statistics |  | YES |
| ATC (SW rate limit) |  | YES |
| Delay reload |  | YES |
| CLI “show debug” |  | NO |
| CLI “show tech” |  | Yes |
| CLI “show interfaces brief” |  | YES |
| Cable Diagnostics / TDR |  | Yes |
| Port Utilization (kbits/sec, Pkts/sec, % Util in recent 300secs) |  | YES |
| IPv6 Management (Telnet Server/ICMP v6) |  | YES |
| Green Ethernet |  | YES (Combo ports only) |
| PoE |  | NO |
| Monitor Environment | Power Status | NO |
| IPv6 Features |  |  |
| IPv4/IPv6 Dual Protocol Stack |  | YES |
| IPv6 Address Types | Unicast | YES |
|  | Multicast | This is only used internally. Not configurable. |
| ICMPv6 |  | YES |
| ICMPv6 Redirect (Host) |  | YES |
| IPv6 Path MTU Discovery |  | YES |
| IPv6 Neighbor Discovery | Router Discovery | YES |
|  | Duplicate Address Detection | YES |
|  | Static Cache Entry | NO |
|  | Parameter Discovery | YES |
|  | Prefix Discovery | YES |
|  | Address Resolution | YES |
|  | Unreachable Neighbor Detection | YES |
|  | Redirect (Host) | YES |
|  | Snooping | YES (phase 2) |
| Stateless Autoconfiguration |  | YES |
| Stateful auto configuration over IPv6 |  | YES |
| Manual Configuration |  | YES |
| Generic Prefix |  | NO |
| SNMP over IPv6 |  | YES |
| HTTP over IPv6 |  | YES |
| SSH over IPv6 |  | YES |
| IPv6 Telnet Support |  | YES |
| IPv6 DNS Resolver |  | YES |
| IPv6 RADIUS Support |  | NO |
| IPv6 TACACS+ Support |  | NO |
| IPv6 Syslog Support |  | YES |
| IPv6 SNTP Support |  | YES |
| IPv6 SMTP Support |  | NO |
| IPv6 TFTP Support |  | YES |
| Remote IPv6 Ping |  | YES |
| Ping over IPv6 |  | YES |
| Trace Route over IPv6 |  | YES |
| IPv6 sFlow |  | NO |
| DHCP6 | Client | YES |
|  | Relay | NO |
|  | Snooping | YES (phase 2) |
|  | Server | NO |
| MVR6 |  | YES (phase 2) |
| IPv6 Source Guard |  | YES (phase 2) |
| RA Guard |  | YES |
| IPv6 ND Snooping |  | YES (phase 2) |
| IPv6 FTP Support |  | NO |
| MLD Snooping |  | YES (phase 2) |
| Security Features |  |  |
| IPv6 ACL | Number of rules: |  |
|  | L2/L3/L4 | Yes (but need to test) |
| IPv6 QoS |  |  |
| IPv6 DSCP remapping CoS |  | NO |

Installation and Configuration Notes:

In general, the ECS3510-28T/52T Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS3510-28T/52T Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

To download new firmware form a TFTP server, enter the IP address of the TFTP server, select “opcode” as the file type, then enter the source and destination file names. To start the new firmware, enter the “reload” command or reboot the system.

CLI commands:

Console#copy tftp file

TFTP server IP address: 192.168.1.23

Choose file type:

1. config:  2. opcode:  : 2

Source file name: opcode.bix

Destination file name: opcode.bix

\Write to flash programming.

Programming flash started.

Success.

Console#reload

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

LOADER CODE modifications:

Loader V1.0.6.0

| ECS3510-28T-00380 | Enhancement : change bootrom password |
| --- | --- |

Loader V1.0.2.4

| . ECS3510-28T-00416 | inject the packets when bootup will cause discard issue |
| --- | --- |

Loader V1.0.4.0

| .ECS3510-28T-00253 | Loader: backdoor passwd doesn't be encrypted |
| --- | --- |

Loader V1.0.2.3

| . | The last port may abnormal, if that port has packet traffic in boot up. |
| --- | --- |

Loader V1.0.2.2

| ECS3510-28T-00175 | (ECS3510-52T) LED: Port#42 LED always can't be light-up |
| --- | --- |

Loader V1.0.2.1

| ECS3510-28T-00115 | Enhancement: support BID=1 for ECS3510-52T |
| --- | --- |

Loader V1.0.0.3

| Loader | Add PCI API for BID 1 |
| --- | --- |

Loader V1.0.0.2

| Loader | Update DRAM register value & support BID 1 |
| --- | --- |

Loader V1.0.0.1

|  | Loader: fix port 16 can linkup in startup |
| --- | --- |

Runtime CODE modifications:

Runtime V1.5.1.18

| ES3510MA-FLF-38-00923 | fail to set ARP ACL src/dst ip/mask via SNMP |
| --- | --- |
| ES3510MA-FLF-38-00943 | STP didn't send BPDU when traffic-segmentation downlink include port1 (Initialize the rx_ifindex to 0) |

Runtime V1.5.1.17

| ECS3510-28T-00618 | Port-Mirror - Port monitors 2 ports and show any command about 10 times will cause system crash. |
| --- | --- |

Runtime V1.5.1.16

| ECS3510-28T-00606 | Injecting massive L2 packets cause memory leakage and reboot by watchdog |
| --- | --- |

Runtime V1.5.1.13

| ES3528MV2-FLF-EC-00612 | IPSG binding table are not flushed when DHCPSNP binding update |
| --- | --- |
| ES3528MV2-FLF-EC-00613 | Switch crash after configure port security |
| ES3528MV2-FLF-EC-00614 | port security max-mac-count problem |
| ECS3510-28T-00544 | DHCP relay option 82 (per port + per vlan mode) |
| ES3510MA-FLF-38-00918 | GVRP not support 4K VLAN. |
| ES3510MA-FLF-38-00920 | ES3510MA syslog enhancement |
| ES3528MV2-FLF-EC-00616 | remove all VID from port interface |
| ES3510MA-FLF-38-00924 | eap packet will be dropped when the first user authenticates successfully and the second user is authenticating. |

Runtime V1.5.1.11

| ECS3510-28T-00536 | change qos mapping table from global to per port setting |
| --- | --- |
| ECS3510-28T-00557 | auto upgrade version misidentification. |
| ECS3510-28T-00559 | software checking port up/down, it will spend more time |
| ECS4620-28T-00292 | IPv4AddressConfig: DUT may get crash when we send command "ip address dhcp" or "no ip address" with IP got from DHCP server. |

Runtime V1.5.1.10

| ECS3510-28T-00543 | remove spanning tree MSTP mode then switch will hang in boot process |
| --- | --- |
| ES3528MV2-FLF-EC-00610 | IPSG looses binding when receive next DHCP request packets |

Runtime V1.5.1.8

| ECS3510-28T-00534 | Enabled ip dhcp dynamic-provision by factory defaults |
| --- | --- |
| ECS3510-28T-00537 | IPSG max-binding set to 1 will drop the packets exist in binding entry |
| ECS3510-28T-00539 | The packet will lost about 2% in highest strict queue. |
| ECS3510-28T-00540 | When test cable-diag all ports in the same time, will test fail and lose auto- nego |
| ECS3510-28T-00541 | qos map phb-queue configure will not be save |
| ES3510MA-FLF-38-00899 | traffic-segmentation didn't work when enable DAI function |

Runtime V1.5.1.7

| ES3528MV2-FLF-EC-00604 | enable SSH server and generate key cause CPU rising periodically |
| --- | --- |
| ECS3510-28T-00526 | IPSG binding table interface display didn't align |
| ECS3510-28T-00527 | IPSG mac mode static binding display unit number 0 in config file |
| ECS3510-28T-00528 | When enable the dot1q-tunnel system-tunnel-control then vlan-trunking will function failure. |
| ECS3510-28T-00529 | switch still could learn the mac-address when port security max-mac-count is 0 |
| ES3528MV2-FLF-EC-00609 | MVR function cause memory leakage |

Runtime V1.5.1.6

| ECS3510-28T-00522 | New command : PPPoE intermediate-agent format-type vendor-id value(CLI only) |
| --- | --- |

Runtime V1.5.1.5

| ES3510MA-FLF-38-00786 | DAI works incorrectly with IPSG static binding |
| --- | --- |
| ECS3510-28T-00523 | [Problem] "ip source-guard binding filter-source static" doesn't work properly. |
| ECS3510-28T-00525 | no limit for DHCPSNP per port binding entry |
| ES3528MV2-FLF-EC-00601 | Cluster:If cluster commander manage the member through web. DUT would be crash. |

Runtime V1.5.1.4

| ES3510MA-FLF-38-00865 | ipv6 does not work with trunk port issue |
| --- | --- |

Runtime V1.5.1.3

| ECS3510-28T-00446 | MVR:It display error msg in CLI and source port didn't receive all traffic when a port move from a domain MVR VLAN to another |
| --- | --- |
| ECS3510-28T-00490 | DHCP relay : switch should not forward boradcast DHCP packet if DHCP server existsI. |
| ECS3510-28T-00497 | IPSG: IPSG max-binding and static-mac didn't set via MIB because MIB file didn't have these node. |
| ECS3510-28T-00498 | IPSG: It display WEB_GROUP task in CLI when binding filter source set static via web |
| ECS3510-28T-00499 | IPSG: It display incorrectly via WEB when setting IP address of static ACL binding |
| ECS3510-28T-00502 | ECS3510-28T network access mac-auth stop working |
| ECS3510-28T-00503 | IPSG: On Web, select Binding Filter Source "All" or "Static" and click "Apply” button, DUT will occur crash. |
| ECS3510-28T-00504 | IPSG: The IP address display on Static Binding List is incorrect. |
| ECS3510-28T-00505 | IPSG: The static binding entry did not be deleted on Web. |
| ECS3510-28T-00506 | IPSG: On Web, click the "Revert" button, the Static Binding List will add entries. |
| ECS3510-28T-00507 | IPSG: The Static Binding List of MAC table is over to maximum set. |
| ECS3510-28T-00508 | The DU will dump exception message and crash, during copy startup to running and login the WEB page. |
| ECS3510-28T-00510 | WEB:It didn't display any information for CPU utilization task via WEB |
| ECS3510-28T-00511 | CPUGuard: CPU guard shall be able to set via WEB |
| ECS3510-28T-00513 | CPUGuard:It shall display log when enable CPU guard and disable CPU guard trap. |
| ECS3510-28T-00514 | CPUGuard:DUT didn't save to keep high watermark setting after rebooting. |
| ECS3510-28T-00515 | Appscan: DUT happen L2MCAST_GROUP task when appscan execute mvr_assiciate_profile_add_show.htm |
| ES3528MV2-FLF-EC-00593 | CLI:Did some config then put it over night, then DUT displayed exception and reboot continously. |

Runtime V1.5.1.2

| ECS3510-28T-00479 | CPUGuard:It didn't set CPU guard parameters via CLI. |
| --- | --- |
| ECS3510-28T-00469 | [Enhancement]CPU guard |
| ECS3510-28T-00493 | ECS3510-28T snmpset causes switch crash |
| ECS3510-28T-00362 | ECS3510-28T network-access port-mac-filter works incorrectly with port security |
| ECS3510-28T-00475 | Syslog: to support save logfile in TLV format. |
| ECS3510-28T-00450 | MIB:DUT shall support to set and display MAC-learning for VLAN by MIB |
| ECS3510-28T-00492 | Enhancement: MAC learning disable per VLAN |
| ECS3510-28T-00261 | IGMP Snooping learned 1023, MVR domain 1 learned 1023 and MVR domain 2 can learned 1023 entries at the same time |
| ECS3510-28T-00494 | [IPSG] command is not complete |
| ECS3510-28T-00457 | After Disable Class ID at vlan interface, DUT DHCP packet still with option 60, class id = Edge-Core. |
| ECS3510-28T-00428 | ECS4210-28T/ECS3510-28T DHCP SNP trust port without setting DAI trust may fowards ARP packets to others ports. |
| ECS3510-28T-00488 | IGMP-Snooping: Static Group is not existed after saving running config and restart. |
| ECS3510-28T-00486 | IGMP-Snooping: Static Mrouter is not existed after saving running config and restart. |
| ECS3510-28T-00487 | IGMP-Snooping: Static Mrouter is not existed after saving running config and restart. |
| ECS3510-28T-00388 | Network access mode auth cannot work with IPSG mac mode |
| ECS3510-28T-00485 | IGMP-Snooping: Multicast data with unmatched groups of join report should be dropped. |
| ECS3510-28T-00484 | MVR6 enhancement: each profile can configure overlap group range |
| ECS3510-28T-00397 | DHCP relay option 82 doesn't work |
| ECS3510-28T-00394 | DHCP relay doesn't work |
| ECS3510-28T-00354 | IPSG redesign |
| ECS3510-28T-00482 | MVR enhancement: each profile can configure overlap group range |
| ECS3510-28T-00446 | MVR:It display error msg in CLI and source port didn't receive all traffic when a port move from a domain MVR VLAN to another |
| ECS3510-28T-00480 | fail to upgrade loader via snmp |
| ECS3510-28T-00409 | IPv6: DUT sends wrong number DAD packets as configured. |
| ECS3510-28T-00343 | IPSG mac mode doesn't work if port security is enabled. |
| ECS3510-28T-00374 | DHCP SNP: On CLI, "show dhcp snooping", the "Vlan" display is incorrect. |

Runtime V1.5.0.0

| ECS3510-28T-00473 | DHCPDynamicProvision:DUT wll hang up after booting configuration file. |
| --- | --- |
| ECS3510-28T-00431 | PVID may be configured to a VLAN which a port does not join on hybrid or trunk mode. If the VLAN is inexistent, the VLAN is auto. created |

Runtime V1.4.9.2

| ECS3510-28T-00469 | [Enhancement]CPU guard |
| --- | --- |
| ECS3510-28T-00475 | Syslog: to support save logfile in TLV format. |
| ECS3510-28T-00368 | QinQ: the switch dosen't change priority when set "sw priority default " on the access port |
| ECS3510-28T-00474 | Tacacs+ Login authentication behaivor is different than ES3528M, ES3510, ECS4510-28F. |
| ECS3510-28T-00463 | PortHistory: The packets of Discards did not be counted. |
| ECS3510-28T-00466 | WEB:Remote log page on WEB display incorrect. |
| ECS3510-28T-00453 | WEB/interface:the column of port list shall align with other columns, shall not overlap. |
| ECS3510-28T-00452 | WEB/ATC:the column action of ATC shall align with other columns, shall not overlap. |
| ECS3510-28T-00442 | Interface: The port list panel display is not align. |
| ECS3510-28T-00439 | DHCP SNP: On Web, the "Lease Time (seconds)" in binding table position is not in a line. |
| ECS3510-28T-00441 | DHCP SNP: On Web, the text box of "Circuit ID Value" display is incorrect. |
| ECS3510-28T-00440 | DHCP SNP: On Chrome v35.0.1916.114 m, the "DHCP Snooping Information Option Sub-option Format" display is not in a line. |
| ECS3510-28T-00438 | DHCP Relay: The text box of the Server IP Address position is wrong. |
| ECS3510-28T-00467 | WEB:WEB fields display incorrect. |
| ECS3510-28T-00445 | LoaderPasswd:It shall display "Password is changed successfully." when DUT succeed to change loader password. |
| ECS3510-28T-00461 | CLI/MVR:DUT didn't remove it when using no mvr type source/receiver. |
| ECS3510-28T-00447 | MVR:ONLY the source port of domain 1 could received traffic data when source port mode is "Always Forward". |
| ECS3510-28T-00464 | IPSG mac mode doesn't flush mac addresses which learnt by port security |
| ECS3510-28T-00258 | DHCP reply packets will be drop on diable dhcp snooping vlan when the bootp flag is unicast. |
| ECS3510-28T-00459 | ARP: The "Configure General" did not display anything on Web. |
| ECS3510-28T-00468 | WEB:DAI WEB page display incorrect. |

Runtime V1.4.9.1

| ES3528MV2-FLF-EC-00563 | [Enhancement] Customer requires a switch for dynamic provision. |
| --- | --- |
| ECS3510-28T-00391 | ProtocolVLAN:DUT doesn't provide pppoe protocol type items on web. |
| ECS3510-28T-00422 | Enhancement: Create a switch to separate the IPSG binding table form DHCP snooping binding table. |
| ECS3510-28T-00432 | [New command] "switchport allowed vlan vlan-list" |
| ECS3510-28T-00025 | IPv6Log:syslog message only display ipv4 network. when configure remote server as IPv6 address server. |
| ECS3510-28T-00420 | WEB/SNMP staPortBpduGuard Status on Port |
| ECS3510-28T-00437 | PortHistory: History Name input over 12 charasters, it did not display completely. |
| ECS3510-28T-00436 | PortHistory: After click "delete" button, the entry did not deleted on Web. |
| ECS3510-28T-00444 | Trunk: The packets did not forward between Trunks. |

Runtime V1.4.9.0

| ECS3510-28T-00418 | [enhacement] support DCHP Option 12 |
| --- | --- |
| ECS3510-28T-00417 | [enhancement] Support dhcp relay relay hops as like DLINK switch |
| ECS3510-28T-00423 | [enhacement] Change MVR Source Port Mode Dynamic as default |
| ECS3510-28T-00407 | Enhancement: add "write" command to same as copy command |
| ECS3510-28T-00389 | [web enhancement] support switching pps and kbps |
| ECS3510-28T-00413 | [web freezed] to apply IPSG static entry via web, it causes web freeze and cpu 100% |
| ECS3510-28T-00410 | [mldsnp]when receive report or leave on a router port, it will not forward to other router port |
| ECS3510-28T-00399 | mvr proxy switching sends igmp report with source ip 0.0.0.0 |
| ECS3510-28T-00353 | Failed to enable ingress MAC access-group on eth 1/12. |
| ECS3510-28T-00424 | [enhancement] show snmp community string with * during provisioning when the user press P |
| ECS3510-28T-00406 | Enhancement: MAC learning disable per VLAN |
| ECS3510-28T-00429 | Enhancement: Add squashfs compress function for runtime image |
| ECS3510-28T-00419 | [enhancement] WEB/SNMP staPortBpduGuard Status on Port |
| ECS3510-28T-00427 | Enhancement: add history counter |
| ECS3510-28T-00411 | the command mvr/mvr6 immediate leave by-host-ip doesn't exist on trunk port |
| ECS3510-28T-00362 | ECS3510-28T network-access port-mac-filter works incorrectly with port security |
| ES3510MA-FLF-38-00792 | [Enhancement] add MAC address of TC sender to log |
| ES3528MV2-FLF-EC-00502 | VLAN: PVID can be set on VLAN which a port does not join; a new command can set VLAN membership directly without add/remove. |

Runtime V1.4.2.7

| ECS3510-28T-00369 | Create network-access port-mac-filter by SNMP |
| --- | --- |
| ECS3510-28T-00412 | DHCP rely function have different behavior between ECS3510-28T and ES3528M |
| ES3528MV2-FLF-EC-00499 | queue mode WRR is always shown on web interface |
| ES3510MA-FLF-38-00766 | management interface sends dhcp request with wrong format for renew IP address |
| ES3510MA-FLF-38-00776 | diffServPortPolicyMapIndex can be written well but while reading always returns value “0” |
| ES3510MA-FLF-38-00779 | ES3510MA MVR by port SNMP bug |
| ES3528MV2-FLF-EC-00497 | ES3528Mv2 MIB doesn't support "egress" ACL |

Runtime V1.4.2.6

| ECS3510-28T-00398 | When DHCP snooping enable, IPSG static binding doesn’t work. |
| --- | --- |
| ECS3510-28T-00402 | Dhcp snooping will drop DHCP ack packet which fragment flag = 1. |
| ECS3510-28T-00404 | When enable IP source guard, IPSG will block ARP packet. |
| ES3510MA-FLF-38-00785 | ES3510MA v1.4.2.1_sd IPv6 dhcp client can't obtian ip address after port move |
| ES3510MA-FLF-38-00783 | switch forward solicit packet with check sum error |

Runtime V1.4.2.5

| ECS3510-28T-00393 | After ARP flooding about 1500 pps, user unable to connect to switch via snmp, telnet,web. |
| --- | --- |
| ES3528MV2-FLF-EC-00512 | [Enhancemnet] support clear log ram/flash on web |

Runtime V1.4.2.4

| ES3528MV2-FLF-EC-00518 | OSPF/RIP/VRRP packets cannot go through while IGMP or MVR is enabled |
| --- | --- |

Runtime V1.4.2.3 ; Loader V1.0.2.3

| ECS3510-28T-00394 | DHCP relay option 82 doesn’t work |
| --- | --- |
| ES3510MA-FLF-38-00781 | PPPoE IA forward PADR with vlan tag to client port , even vlan is untag member of client port |

Runtime V1.4.2.1 ; Loader V1.0.2.3

| ECS3510-28T-00387 | Enhancement: not reserve these data packet (MAC based mirror, IP Subnet based VLAN & MAC based VLAN) ACL, so user has more ACL can configure |
| --- | --- |
| ECS3510-28T-00352 | [web] ECS3510-28t should not display temperature list |
| ECS3510-28T-00386 | Enhancement: CPU protection to prevent management loss by heavy traffic attack |
| ECS3510-28T-00381 | Enhancement Per process cpu usage monitoring |
| ECS3510-28T-00356 | lost management because of injecting IGMP packet. |
| ES3528MV2-FLF-EC-00513 | add timezone with the same command like ES3528MO |

Runtime V1.4.2.0 ; Loader V1.0.2.3

| ECS3510-28T-00338 | LBD - Modify the SA and inject the loopback packets via TG1 with VLAN, the link port will be shutdown. |
| --- | --- |

Runtime V1.4.0.2 ; Loader V1.0.2.3

| ECS3510-28T-00363 | ECS3510-28T network access mac authentication works incorrectly and shows error messages |
| --- | --- |
| ECS3510-28T-00360 | [enhance] user can config the maximum of FDB lookup depth. |
| ECS3510-28T-00379 | Enhancement : Protocol VLAN support PPPoE type |
| ECS3510-28T-00357 | [snmp] lost management because of 4K vlan |
| ECS3510-28T-00358 | lost management because of injecting OSPF hello packet |
| ECS3510-28T-00382 | Enhancement Uptime of ports cannot be read via snmp |
| ECS3510-28T-00385 | Enhancement: support IPSG_STRICT_MODE and enlarge ACE/ACL number. |
| ECS3510-28T-00383 | Protocol VLAN display problem |
| ECS3510-28T-00364 | ECS3510-28T Can't configure mac-vlan with MASK |
| ECS3510-28T-00365 | when switch receive fragment packet will log as DHCP packets |

Runtime V1.4.0.1 ; Loader V1.0.2.3

| ECS3510-28T-00354 | IPSG redesign |
| --- | --- |
| ECS3510-28T-00117 | Tacacs:Web UI cannot use tacacs admin privilege level to access DUT. |
| ECS3510-28T-00347 | mac-address-table count strange behavior |
| ECS3510-28T-00355 | MIB: fix private MIB file compiler error |
| ECS3510-28T-00200 | Help:The "Default Domain Name" length can be set 127 on Web, SNMP and CLI, but help page is 1~68. |
| ECS3510-28T-00288 | enhancement igmpsnp vlan configuration command and running cfg |
| ECS3510-28T-00344 | Port security - mac learning issue |
| ECS3510-28T-00351 | dhcp snooping option 82 (Malformed packet), client can't obtain ip address |
| ECS3510-28T-00349 | [Web interface] storm control problem, no permit to set threshold |
| ECS3510-28T-00185 | IPV6SG: The help information of "IPv6 Source Guard" did not describe correctly on help page. |
| ECS3510-28T-00332 | Help: On web, Dynamic Binding List did not describe "Type". |
| ECS3510-28T-00205 | Help:line console login timeout default hould not be disable in help page |
| ECS3510-28T-00336 | Help:Several mvr & mvr6 help page information are not correct. |
| ECS3510-28T-00302 | IGMP Snooping:Forwarding Priority on help page range incorrect |
| ECS3510-28T-00331 | WEB Help [ ARP Inspection ]: There is No descripton about " Allow Zeros " |
| ECS3510-28T-00327 | Help: The IPSG vlan range shall be modify to 4094. |
| ECS3510-28T-00340 | [Help page]VLAN: Information of VLAN ID range is incorrect on help page. |
| ECS3510-28T-00335 | Help:Several igmp snooping help page information are not correct. |
| ECS3510-28T-00350 | Enhancement: ATC threshold support 500 pps |
| ECS3510-28T-00348 | No way to configure storm control bit/sec in web interface |
| ECS3510-28T-00297 | mac notification fail when NA |
| ECS3510-28T-00235 | IPv6-AddrConfig: fe81::1234 can be set to link-local address via WebUI. |

Runtime V1.4.0.0 ; Loader V1.0.2.3

| ECS3510-28T-00319 | there are 4k lines for 4k vlan. please compress them. |
| --- | --- |
| ECS3510-28T-00334 | MVR:Use maximum length to be mvr profile name, when "show mvr profile" CLI will have not aligned status. |
| ECS3510-28T-00333 | MVR6:Use maximum length to be mvr6 profile name, when "show mvr6 profile" CLI will have not aligned status. |

Runtime V1.2.7.3 ; Loader V1.0.2.3

| ECS3510-28T-00132 | IPv6log:DUT can set invalid ipv6 link-local address as logging host. |
| --- | --- |
| ECS3510-28T-00322 | IPSG mac mode : client can still access network if applying "no ip source guard static entry" |
| ECS3510-28T-00293 | enabling IPSG mac mode should not flush dhcp snp binding table |
| ECS3510-28T-00291 | Applying "clear ip dhcp snooping binding" should clear mac-table at the same time |
| ECS3510-28T-00290 | if switch learns new entry in dhcp snp binding, it should flush block record |
| ECS3510-28T-00311 | LLDP - The device no "Port Name" parameters of remote device information, should remove from web UI |
| ECS3510-28T-00305 | DHCPv6: DUT is DHCPv6 client, it can not get IP from Server. |
| ECS3510-28T-00288 | enhancement igmpsnp vlan configuration command and running cfg |
| ECS3510-28T-00297 | mac notification fail when NA |
| ECS3510-28T-00318 | Enhancement: Improve show running-config time when create many vlan |

Runtime V1.2.7.2 ; Loader V1.0.2.3

| ECS3510-28T-00299 | DHCP SNP: DHCP SNP rate limit did not supported on MIB. |
| --- | --- |
| ECS3510-28T-00292 | IPSG(mac mode) doesn't work properly after rebooting. |
| ECS3510-28T-00300 | [Issue]: SFP can not link up on R0A Hardware Version. |
| ECS3510-28T-00313 | LLDP - The "SNMP Notification" parameters shall be enabled after default setting. |
| ECS3510-28T-00314 | LBD - Checking length of LDPDU, the value is 00-10. |
| ECS3510-28T-00309 | LBD - Disable the loopback detection of port channel 1, the Oper State shall be disable. |
| ECS3510-28T-00308 | LBD - After factory setting, the Oper State shall be disable of Port. |
| ECS3510-28T-00307 | LBD - Disable the loopback detection of DUTPA, the Oper State shall be disable. |
| ECS3510-28T-00317 | LBD - The default value of recovery time shall be 60 second, |
| ECS3510-28T-00260 | [Strange message] the Age-Out is index0,vid1,mac 0:30:f1:47:5a:62, index:57 |
| ECS3510-28T-00310 | CLI: The debug message that is about to MAC address age-out is shown on CLI. |
| ECS3510-28T-00304 | IPv6 ND Snooping:Wait for several minutes, DUT will display "the Age-Out is index0,vid1,mac 0:22:15:4b:f0:e1, index:1" msg. |
| ECS3510-28T-00301 | RefinePortSecurity:"Learned-PSEC" addresses learned by Port Security function did not saved into start-up config |
| ECS3510-28T-00232 | [enhancement] configure subnet mask friendly |

Runtime V1.2.7.0 ; Loader V1.0.2.3

| ECS3510-28T-00296 | [enhacement] support CLI multiple priviledges |
| --- | --- |
| ECS3510-28T-00282 | no permit for using cable-diagnostics on WEB |
| ECS3510-28T-00294 | DAI syslog packet message : unknowm type? |
| ECS3510-28T-00295 | TCAM problem : entries used by user is not correct |
| ECS3510-28T-00231 | UI: kill admin user account, but it will create again after reboot. |

Runtime V1.2.6.2 ; Loader V1.0.2.3

| ECS3510-28T-00289 | ECS3510-28T: client cannot get ip address because of dhcp snp option 82 |
| --- | --- |

Runtime V1.2.6.1 ; Loader V1.0.2.3

| ECS3510-28T-00272 | Protocol group id not sorting |
| --- | --- |
| ECS3510-28T-00277 | igmp snooping priority doesn't support Cos bit 7 |
| ECS3510-28T-00285 | Can't read the information about dynamic VLAN via SNMP |
| ECS3510-28T-00268 | Debug typo expiry |
| ECS3510-28T-00257 | front pannel display issue in web page |
| ECS3510-28T-00241 | LACP: The LED light of Web page doesn't display when the port is link up. |
| ECS3510-28T-00284 | ECS3510-28T support mvr proxy-switching, but no "MVR Configure Global" web page |
| ECS3510-28T-00255 | (Enhancement) -- IP IGMP Query-drop vlan<VID> |
| ECS3510-28T-00254 | IGMPSNP: Can't clear dynamic groups manually. |
| ECS3510-28T-00246 | The DUT will show error message "L_MM_Free():invalid buffer" when to execute "copy startup-config running-config" command |
| ECS3510-28T-00252 | CLI: Unexpected messages would be showed when copy startup-config to running-config |
| ECS3510-28T-00249 | IGMP report form mvr receiver had been dropped by IGMP filter, but mvr drop counter didn't increase |
| ECS3510-28T-00256 | MSTP instances can not be read by SNMP |
| ECS3510-28T-00236 | IPv6SrcGuard: DUT can not be accessed via http, telnet, and SNMP after creating static entry via SNMP. |
| ECS3510-28T-00245 | DAI: DUT cannot save this filter setting to running configuration. |
| ECS3510-28T-00242 | IPv6SrcGuard: Enabled port could still forward mismatched packets when source guard entry is generated via DHCP. |
| ECS3510-28T-00247 | SYSLOG: SYSLOG format is incorrect. |
| ECS3510-28T-00239 | MLDv2: The group entry still exist after receive TO_INCLUDE(4) or BLOCK(6). |
| ECS3510-28T-00237 | MLDSNP: CLI prompt debug message after MLD group expired. |

Runtime V1.2.6.0 ; Loader V1.0.2.3

| ECS3510-28T-00226 | IGMPSnooping: Allocate too much memory when all ports join all 4094 VLANs case. And the device will reboot automatically. |
| --- | --- |
| ASF4512MP-FLF-EC-00429 | CLI command "Show interface transceiver" fails after the SFP has been inserted for more than 248 days |

Runtime V1.2.5.0 ; Loader V1.0.2.3

| Loader | The last port may has problem when packet traffic in startup |
| --- | --- |
|  | Add filter for PVST packets |
|  | IGMP RADIUS authentication |
| ECS3510-28T-00228 | Modify default LLDP parameters in ECS3510-28T to be the same as it is now in ES3528M switch |
| ECS3510-28T-00227 | Enhancement: adding command “ip arp inspection validate dst-mac src-mac ip allow zeros” like in Cisco |
| ECS3510-28T-00225 | mac-address-table aging time problem |
| ECS3510-28T-00224 | change design to let MVR receiver port not to drop multicast data |
| ECS3510-28T-00215 | remove egress service policy command |
| ECS3510-28T-00211 | ipv6 dhcp snooping vlan ? - incomplete command prompt |
| ECS3510-28T-00210 | show ip arp inspection vlan ? - incomplete command prompt |
| ECS3510-28T-00202 | ipv6 nd snooping auto-detect retransmit interval ? -> command prompt missing "seconds" |
| ECS3510-28T-00201 | ipv6 nd snooping vlan ? -> command prompt missing vlan range |
| ECS3510-28T-00209 | ipv6 nd snooping vlan ? - incomplete command prompt |
| ECS3510-28T-00223 | CLI internal error when configuring igmpsnp port channel max-group |
| ECS3510-28T-00216 | DHCPSNP attack message: add source mac address of rogue dhcp server |
| ECS3510-28T-00221 | Enhancement: let threshold value can be different for boardcast, multicast & unknow unicast |
| ECS3510-28T-00217 | ping can not respond when there are many MAC learning packets |
| ECS3510-28T-00218 | MVR: when a port join same group for multiple vlan, leave report for one vlan will take the port out of forwarding bitmap. |
| ECS3510-28T-00219 | Ingress-filter can not work and increase of CPU utilization |
| ECS3510-28T-00206 | Enhancement: add MVR parameter commands and IGMP immediate-Leave Check Source IP |
| ECS3510-28T-00213 | ECS3510-52T flowcontrol doesn't work on SFP port (auto-negotiation mode) |
| ECS3510-28T-00214 | Add trap and log for BPDU guard |
| ECS3510-28T-00192 | CoS: The capability of CoS did not work correctly while the congestion happened on the different chip. |

Runtime V1.2.4.2 ; Loader V1.0.2.2

| ECS3510-28T-00196 | Performance: The DUT can not forward packets correctly while tests RFC-2889 Partially Meashed and Fully Meashed. |
| --- | --- |

Runtime V1.2.4.0 ; Loader V1.0.2.2

| ECS3510-28T-00204 | Can't Login Web Via HTTPS |
| --- | --- |
| ECS3510-28T-00181 | Web Help:It should add the description of loopback detection about action and shutdown interval in help page. |
| ECS3510-28T-00178 | IPv6 Source Guard:The "IPv6" display on Web and Help page is not inconsistent. |
| ECS3510-28T-00185 | IPV6SG: The help information of "IPv6 Source Guard" did not describe correctly on help page. |

Runtime V1.2.2.1 ; Loader V1.0.2.2

|  | After enable ip source-guard on port, i can't bind service-policy to port |
| --- | --- |
|  | Refine html build environment |

Runtime V1.2.2.0 ; Loader V1.0.2.2

| ECS3510-28T-00175 | (ECS3510-52T) LED: Port#42 LED always can't be light-up |
| --- | --- |
| ECS3510-28T-00149 | MVR6 MIB:It didn't set mvr6 vlan via snmp. |
| ECS3510-28T-00190 | CoS: The DUT only supported "4" Queue ID but it could set "8" weights which assigned to queues via CLI. |
| ECS3510-28T-00198 | MAC-BasedVLAN:Add normal MAC to VLAN-MAC,CLI will show exception message and Web UIcan't be accessed any more. |
| ECS3510-28T-00193 | Web : IE6 can't login the DUT web management interface by ipv6 address.(Firefox/Chrome are OK) |
| ECS3510-28T-00135 | MLDSNP Web:It shall be able to clear dynamic mld group via web. |
| ECS3510-28T-00179 | Remote-Ping : DUT ping a hostname with 33 length, console will be display crash information. |
| ECS3510-28T-00188 | IPV6SG: While users converted the "NDSNP" binding entry into a STATIC binding entry, the DUT will display Exception and Hang. |
| ECS3510-28T-00187 | VLAN : the mutiple VLAN ID cloud Not be removed in CLI at the same time |
| ECS3510-28T-00184 | Customerinfo - In MG-SOFT, did not found "systemDescription" and "systemObjectID" node. |
| ECS3510-28T-00143 | IGMPSNP Web:Time format of the expire shall display HH:MM:SS in web, not MM:SS. |
| ECS3510-28T-00177 | UserAuthRemote:DUT can login authentication by TACACS server via CLI and Telnet,but it can not login via Web. |

Runtime V1.2.1.2 ; Loader V1.0.2.0

| ECS3510-28T-00115 | Enhancement: support BID=1 for ECS3510-52T |
| --- | --- |
| ECS3510-28T-00098 | Enhancement: disable vlan translation |
| ECS3510-28T-00114 | CodeSonar: remove compiler warning messgae |
| ECS3510-28T-00094 | Enhancement: adjust system header file for ECS3510-52T |
| ECS3510-28T-00116 | Enhancement: modify SFP & thermal driver to support ECS3510-52T |
| ECS3510-28T-00174 | DHCP: enable ip dhcp snooping will cause client can not get IP |
| ECS3510-28T-00173 | Enhancement: add web UI for ECS3510-52T when BID=1 |
| ECS3510-28T-00158 | DHCPv6 Snooping Option37:On SNMP, it does not support DHCPv6 Snooping Option. |
| ECS3510-28T-00164 | IPv6SourceGuard: The DUT did not support the function of IPv6 Source Guard on MIB. |
| ECS3510-28T-00129 | ERPS Web:It shall display warning message correctly when ring port has been used. |
| ECS3510-28T-00172 | IGMPSNP MIB: It didn't get the value of igmpSnoopingPortNumGroup. |
| ECS3510-28T-00136 | MLDSNP Web:It shall display complete group address via web. |
| ECS3510-28T-00128 | EPRS Web:It shall not allow to change any erps setting when ERPS has been enabled. |
| ECS3510-28T-00122 | TraceRoute Web:It didn't trace route via web because there have no related page. |
| ECS3510-28T-00127 | ERPS Web:It shall not allow to set again when forced/manual switch has been triggered |
| ECS3510-28T-00131 | ERPS Web:It shall succeed to set or show correct message swhen remove one ring port and set major domain at the same time. |
| ECS3510-28T-00130 | ERPS Web:It shall display warning message correctly when the version is 1 and using force-switch/manual-switch cmd. |
| ECS3510-28T-00125 | TraceRoute6: DUT didn't indicate correctly address format when using ipv6 link-local address to trace. |
| ECS3510-28T-00161 | IPV6SG: Could not set the static IPv6 Source Guard binding entries by using hyphen and comma with the VLAN/Port list. |
| ECS3510-28T-00151 | MVR6 MIB:It shall be able to use mvr6ForwardingPriority node to set or get via snmp. |
| ECS3510-28T-00150 | MVR6 MIB:It didn't get the value of mvr6DomainGroupStaticPorts. |
| ECS3510-28T-00149 | MVR6 MIB:It didn't set mvr6 vlan via snmp. |
| ECS3510-28T-00147 | MVR MIB:It didn't set mvr vlan via snmp. |
| ECS3510-28T-00146 | MVR MIB:It didn't get and set mvrDomainUpstreamSourceIp correctly. |
| ECS3510-28T-00145 | MVR MIB:It didn't get the value of mvrDomainGroupStaticPorts. |
| ECS3510-28T-00121 | Security > IPv6 Source Guard > Dynamic Binding web page => change "IP Address" to "IPv6 Address" |
| ECS3510-28T-00120 | binding ACL to egress port removed from cli, so should also be removed from web |
| ECS3510-28T-00165 | DHCPv6 Snooping:The lifetime has not expired, but the binding table will disappear. |
| ECS3510-28T-00134 | IPV6SG: The user could set over the MAX-binding entries of IPv6 Source Guard on a port interface. |
| ECS3510-28T-00074 | CoS: The descriptions of CoS trust-mode were incorrect when set Cos trust-mode on port interface via CLI. |

Runtime V1.2.0.0 ; Loader V1.0.0.3

|  | Update version from V1.0.0.7 to V1.2.0.0 |
| --- | --- |

Runtime V1.0.0.7 ; Loader V1.0.0.3

| Loader | Add PCI API for BID 1 |
| --- | --- |
| ECS3510-28T-00113 | CodeSonar: {ERPS_MGR_GetDomainIdByName {Uninitialized Variable} |
| ECS3510-28T-00144 | MLDSNP MIB:It didn't set static group in the port via snmp. |
| ECS3510-28T-00153 | MVR6 Help:It shall describe clearly about mvr6 parameters in help page. |
| ECS3510-28T-00141 | DHCPv6: The DHCPv6 Solicit message did not include a "Client Identifier" option on the DHCPv6 packets. |
| ECS3510-28T-00168 | Use SNMP Setting ACL Cause Exception |
| ECS3510-28T-00118 | CLI: Configure Terminal History Size as 0 and run any command will cause CLI Hanged. |
| ECS3510-28T-00107 | DUT will hang up when update the runtime version without cold start |
| ECS3510-28T-00140 | CLI: Clear ACL Hardware Counter has direction - egress (ACL not support Egress now, so it shall be removed) |
| ECS3510-28T-00162 | ipv6addr:ip default gateway can set as IPv6 address. |
| ECS3510-28T-00124 | TraceRoute6:DUT will lookup ipv4 address when using traceroute6 cmd to trace domain name. |
| ECS3510-28T-00119 | VLAN translation removed from cli, so should also be removed from web |
| ECS3510-28T-00123 | Vlan-translation:On help page has vlan-translation information, but DUT does not support. |
| ECS3510-28T-00139 | MLDSNP Help:It shall add the description of MLDSNP in help page. |
| ECS3510-28T-00126 | ERPS Help:It shall describe all information of ERPSv1 and ERPSv2 in help page. |
| ECS3510-28T-00142 | IPv6SG: The DUT did not support the description of IPv6 Source Guard no help page. |
| ECS3510-28T-00103 | Help Page:reload-at range of year is incorrect. |
| ECS3510-28T-00109 | MLDSNP:It didn't forward to mrouter port when DUT received MLD query. |
| ECS3510-28T-00108 | MLDSNP:It will flood to all ports of the same vlan when MLD query checksum is wrong. |
| ECS3510-28T-00111 | MLDSNP:Multicast data will not forward to mrouter port when mrouter port set after mld snoopoing enabled. |
| ECS3510-28T-00110 | MLDSNP:It shall work correctly when unknown multicast mode is "flood" or "to router port". |

Runtime V1.0.0.5 ; Loader V1.0.0.2

| Loader | Update DRAM register value & support BID 1 |
| --- | --- |
| ECS3510-28T-00027 | Protocol VLAN : The Priority vlaue range setting in the WEB UI Not match in the CLI |
| ECS3510-28T-00099 | Enhancement: remove some ACL command because chip is not support egress rule |
| ECS3510-28T-00086 | Enhancement: add G.8032, Y.1731, 802.1ag & 802.3ah and UDLD functions |
| ECS3510-28T-00100 | Enhancement: Add Daylight Saving Time (DST) function |
| ECS3510-28T-00098 | Enhancement: disable vlan translation |
| ECS3510-28T-00080 | HelpPage: When the user launched the help page of PPPoE Intermedia on Web UI, it will display "404 Not Found". |
| ECS3510-28T-00095 | Enhancement: add web UI for all phase 2 new functions |
| ECS3510-28T-00092 | Enhancement: add IPv6 source guard function |
| ECS3510-28T-00097 | Enhancement: Add ND snooping function |
| ECS3510-28T-00090 | Enhancement: add DHCPv6 option 37 function |
| ECS3510-28T-00089 | Enhancement: add DHCPv6 snooping function |
| ECS3510-28T-00091 | Enhancement: add MVR6 function |
| ECS3510-28T-00085 | correction to grammar - show qos map phb-queue |
| ECS3510-28T-00093 | Enhancement: add IPv6 MLD snooping |
| ECS3510-28T-00039 | IPv6SNMP: DUT can not set snmp server as IPv6 address. |
| ECS3510-28T-00082 | Help Page:Vlan Translation does not support on help page. |
| ECS3510-28T-00084 | RSPAN:When command "Console(config-vlan)#vlan 2-10 m e rspan" ,DUT will crash. |
| ECS3510-28T-00081 | ATC : BC and MC shloud not enable on the same port at the same time |
| ECS3510-28T-00075 | Port Filter: In Web Ui Does Not Have Option "All-Client",But in CLI does |
| ECS3510-28T-00017 | IPv6: The DUT could set over the MAX range on IPv6 hop-limit via CLI. |
| ECS3510-28T-00061 | CLI: Corrections to spelling - set IPv6 mtu in the interface config mode of VLAN |
| ECS3510-28T-00083 | IPv6: Remove "ipv6 hop-limit" command for L2 project |

Runtime V1.0.0.1 ; Loader V1.0.0.1

|  | Update web Logo |
| --- | --- |

Runtime V1.0.0.0 ; Loader V1.0.0.1

|  | Loader: fix port 16 can linkup in startup |
| --- | --- |
| ECS3510-28T-00016 | Flash: system is lucked when update runtime image |
| ECS3510-28T-00042 | DHCP Relay Option82:DHCP Relay Option 82 does not work correctly. |
| ECS3510-28T-00032 | IPv6address:Set valid ipv6 address and reboot DUT. original ipv6 address will become invalid. |
| ECS3510-28T-00038 | WEB: The http upgrade opcode code and config file on WEB UI display error. |
| ECS3510-28T-00034 | IP DHCP Relay Option 82 implemented in cli but not in web |
| ECS3510-28T-00071 | DHCP Relay Option 82:DHCP Relay Option does not support on WEB and Help page. |
| ECS3510-28T-00073 | SNMP: The setup panel of SNMP disappeared and did not switch the page correctly when user added SNMPv3 remote user on Web UI. |
| ECS3510-28T-00014 | Enhancement: Add diagnostic features |
| ECS3510-28T-00063 | MVR:Get object:mvrSourcePortmode value via mib, it displays "2", but object value description only (0)dynamic and (1)forward. |
| ECS3510-28T-00022 | Auto Traffic Control (ATC) : WEB Help page missing |
| ECS3510-28T-00077 | Dynamic Arp Inspection : Rate limit value is affect on the trust port. |
| ECS3510-28T-00055 | Web: It should add "expire" field of igmpsnp mrouter table in web. |
| ECS3510-28T-00059 | Web:It always display "disable" when igmp query drop is enabled or disabled |
| ECS3510-28T-00054 | Help Page:The default configuration of MRD should match with help page. |
| ECS3510-28T-00065 | Help:MVR robustness-value range is 1-255 and default is 2, but help page range is 1-10 and default value is 1. |
| ECS3510-28T-00044 | SelectiQinQ: The DUT does not support the description of Select QinQ on help page. |
| ECS3510-28T-00036 | LLDP: Default state of SNMP Notification/ MED Notification are incorrect on help page. |
| ECS3510-28T-00062 | Help Page:It should add the description of help page about forwarding entry.The description of some fields lack. |
| ECS3510-28T-00060 | Help Page:The default configuration of igmpsnp status should match with help page. |
| ECS3510-28T-00070 | RSPAN: The descriptions of RSPAN session and RSPAN page were incorrect on the help page of Web UI. |
| ECS3510-28T-00076 | CoS: The ID range (0-7) of priority queue is incorrect on help page. |
| ECS3510-28T-00030 | IPv6telnet: Default silent time does not have enable. |
| ECS3510-28T-00078 | GVRP :Not capture GVRP PDU containing JoinEmpty event |
| ECS3510-28T-00066 | IPv6mtu:Reset DUT IPv6 MTU. DUT fragment will incorrect. |
| ECS3510-28T-00069 | MVR:Enable mvr and igmp snooping at the same time , only mvr can work correctly, igmp snooping can't work. |
| ECS3510-28T-00053 | IGMPFilter:DUT didn't forward multicast traffic correctly when setting igmp throttle or filter. |
| ECS3510-28T-00051 | IGMPSnooping:DUT will not forward multicast traffic to this port when setting static group to one port. |
| ECS3510-28T-00049 | MVR:Can't get/write the value of object mvrProxySwitching and mvrRobustnessValue via mib browser. |
| ECS3510-28T-00064 | Web:It should be first choice if the default igmp filter access mode of is "Deny" |
| ECS3510-28T-00058 | Web: System > IP (Add Address) => revise "Click this button to restart DHCP service." |
| ECS3510-28T-00024 | MIB:MIB file can not work correctly. |
| ECS3510-28T-00040 | DUT has packet loss on Throughput test with packet size=64, 128, 256, 512, 1024, 1518, 10240 bytes.(both 10h and 100h) |
| ECS3510-28T-00018 | CLI: The function of "terminal length" did not work on the DUT. |
| ECS3510-28T-00067 | ATC : DUT will hang-up when DUT config the auto-traffic-control and receive the Muticast packets |
| ECS3510-28T-00050 | MVR:Delete static trunk interface, mvr page(configure static group member) will dispaly "404 not Found". |
| ECS3510-28T-00048 | Web: ACL Mirror Port only support Port#25~28 can be selected on Web. (CLI can select all the 28 Ports) |
| ECS3510-28T-00047 | Web: Web shows "404 Not Found" when change ACL Configure Interface Action pages from "Show Hardware Counter" to "Configure". |
| ECS3510-28T-00046 | ACL: ACL Hardware Counter information on Web is incorrect. Some information are in the wrong place. |
| ECS3510-28T-00043 | MVR6: Product spec. doesn't support mvr6 function. But we can find mvr6 fucntion on web and mib. |
| ECS3510-28T-00056 | PortSecurity: Dump Exception happened when the DUT learned secured MAC over MAX MAC and triggered the shutdown action. |
| ECS3510-28T-00052 | ATC : DUT will hang-up when DUT config the auto-traffic-control and receive the Broadcast packets |
| ECS3510-28T-00029 | MIB file name is incorrect. |
| ECS3510-28T-00041 | Fully Meshed performance: Always Port#1 has packet loss when doing the Fully Meshed performance testing. |
| ECS3510-28T-00019 | LLDP: The DUT could not configure the Civic Address of LLDP-MED Location on Web panel. |
| ECS3510-28T-00037 | RSTP:When the BPDU guard is enabled on this port and connect another DUT,crash will be occured. |
| ECS3510-28T-00021 | LLDP: The DUT displayed the information of LLDP-MED even if the link partner did not advertise the LLDP-MED capabilities. |
| ECS3510-28T-00026 | IPv6remotePing:Packet size range is not the same in Web UI and Console. |
| ECS3510-28T-00033 | IPv6Dhcp:default ipv6 dhcp does not enable. |
| ECS3510-28T-00002 | Enhancement: Add loader code |
| ECS3510-28T-00013 | Enhancement: change trunk to 8 and MSTP instance to 16 |

Runtime V0.0.0.7 ; Loader V0.0.0.6

| ECS3510-28T-00016 | Flash: system is lucked when update runtime image |
| --- | --- |
| ECS3510-28T-00006 | Enhancement: support combo port and SFP for new HW |
| ECS3510-28T-00015 | Remove sflow command and web UI because this chip doesn't support |

Runtime V0.0.0.4 ; Loader V0.0.0.4

| ECS3510-28T-00004 | System driver: Port LED and combo port can not work |
| --- | --- |
| ECS3510-28T-00005 | Driver: add SFP & I2C driver |
| ECS3510-28T-00003 | CLI: command "reload" can not correct reboot |

Known Issue

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

- VLAN mirroring or MAC-based mirroring can not coexist with Port mirroring. While VLAN mirroring and MAC-based mirroring can be coexisted, but they must use the same target port.

- MAC Chip limit: unicast, multicast, broadcast and storm control all use one same threshold, although each one can enable/disable independent. If user need to use different threshold for each one, he must use rule (diffserv) to do it.

- PHY chip limit: Cable Test on Power Saving is highly inaccurate. If you want to get accurate cable test, please turn off power saving mode first.

- PHY chip limit: GE port can do cable diagnostic in linkup, but FE & GE ports can only do cable diagnostic in linkdown without partner

- PHY chip limit: Power saving (green Ethernet) is work for GE port only.

- MAC chip limit: There is no HW loopback detection without STP (loop-free detection)

- MAC chip limit: Can not support multiple uplink ports on one session

- No mac-learning command for CLI and webUI, the manual is incorrect.

- After phase5, when you add one user level 15, you can remove both admin user and guest user. But, we will create them again in next bootup. You should use “no user admin” to disable admin account if you don’t need admin account.

- In Phase 5, we add port history counter function. But, chip don’t support input/output discard packet counter. So, its values are always 0.

- After Phase 5, we add MAC learning disable by VLAN. Because this project uses software learn. If user use this function (ex. Disable MAC learning on VLAN 10), then all VLAN (10) packets will all trap to CPU. Although we have CPU protection which will drop burst packets, it may cause other control packets to CPU.