Firmware Version 1.2.2.33

INTRODUCTION:

ECS4120 series Gigabit Access Switch consists of ECS4120-28T, ECS4120-52T which configuration is 24/48 10/100/1000 Base-T ports, with additional 4 10G SFP+ ports while ECS4120-28F which configuration is 20 GE SFP & 4 Combo GE ports with additional 4 10G SFP+ ports.

Using IP Clustering for stacking which is a method of grouping switches regardless of geographical location in order to enable centralized management access to the switches through a single management point. This technology provides some of the benefits of stackable switches without requiring direct stacking or connections between switch units. It also allows different kinds of switches to be grouped together, as long as they have the embedded clustering software.

It offers advanced administration through a user-friendly browser interface. QoS and VLAN Supports allows greater scalability and Supports for business-critical applications. Port can be trunked to create a high bandwidth pipe to the network.

It also has comprehensive network management functions, such as Spanning Tree protocol for standard bridging, SNMP, RMON and security features.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Firmware | Version | File Name |
| --- | --- | --- |
| Software | 1.2.2.33 | ecs4120-run_v1.2.2.33.bix |

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Patch Version | 1.2.2.33 | Patch version | 2020/10/30 |
| Patch Version | 1.2.2.32 | Patch version | 2020/7/31 |
| Patch Version | 1.2.2.31 | Patch version | 2020/7/2 |
| Patch Version | 1.2.2.30 | NTC Version | 2020/3/20 |
| Patch Version | 1.2.2.29 | Patch version | 2020/3/5 |
| Patch Version | 1.2.2.28 | Patch version | 2019/12/19 |
| Patch Version | 1.2.2.27 | Patch version | 2019/11/18 |
| Patch Version | 1.2.2.26 | NTC Version | 2019/10/18 |
| Patch Version | 1.2.2.25 | Patch version | 2019/8/22 |
| Patch Version | 1.2.2.24 | NTC Version | 2019/3/21 |
| Patch Version | 1.2.2.23 | NTC Version | 2019/2/11 |
| Patch Version | 1.2.2.22 | NTC Version | 2019/1/23 |
| Patch Version | 1.2.2.21 | NTC Version | 2019/1/10 |
| Patch Version | 1.2.2.20 | NTC Version | 2019/1/3 |
| Patch Version | 1.2.2.19 | NTC Version | 2018/12/24 |
| Patch Version | 1.2.2.19 | NTC Version | 2018/12/24 |
| Patch Version | 1.2.2.18 | NTC Version | 2018/12/18 |
| Patch Version | 1.2.2.17 | NTC Version | 2018/12/13 |
| Patch Version | 1.2.2.16 | NTC Version | 2018/12/10 |
| Patch Version | 1.2.2.15 | NTC Version | 2018/11/27 |
| Patch Version | 1.2.2.14 | NTC Version | 2018/11/16 |
| Patch Version | 1.2.2.13 | NTC Version | 2018/10/30 |
| Patch Version | 1.2.2.12 | Patch version | 2018/10/30 |
| Patch Version | 1.2.2.10 | NTC Version | 2018/10/5 |
| Patch Version | 1.2.2.9 | NTC Version | 2018/10/1 |
| Patch Version | 1.2.2.5 | Patch Version | 2018/9/28 |
| Patch Version | 1.2.2.8 | NTC Version | 2018/9/26 |
| Patch Version | 1.2.2.7 | NTC Version | 2018/9/25 |
| Patch Version | 1.2.2.5 | NTC Version | 2018/9/13 |
| Patch Version | 1.2.2.4 | NTC Version | 2018/9/12 |
| Patch Version | 1.2.2.3 | NTC Version | 2018/8/31 |
| Patch Version | 1.2.2.2 | NTC Version | 2018/8/17 |
| Patch Version | 1.2.2.1 | NTC Version | 2018/8/8 |
| Patch Version | 1.0.2.46 | Patch Version | 2018/7/20 |
| Patch Version | 1.0.2.45 | NTC Version | 2018/7/13 |
| Patch Version | 1.0.2.44 | NTC Version | 2018/6/8 |
| Patch Version | 1.0.2.43 | Patch Version | 2018/6/1 |
| Patch Version | 1.0.2.42 | NTC Version | 2018/5/25 |
| Patch Version | 1.0.2.41 | NTC Version | 2018/5/18 |
| Patch Version | 1.0.2.40 | NTC Version | 2018/5/2 |
| Patch Version | 1.0.2.39 | NTC Version | 2018/4/26 |
| Patch Version | 1.0.2.38 | NTC Version | 2018/4/20 |
| Patch Version | 1.0.2.37 | Patch Version | 2018/3/16 |
| Patch Version | 1.0.2.36 | Patch Version | 2017/12/ |
| Patch Version | 1.0.2.35 | NTC Version | 2017/11/8 |
| Patch Version | 1.0.2.34 | Patch Version | 2017/10/ |
| Patch Version | 1.0.2.33 | Patch Version | 2017/10/13 |
| Patch Version | 1.0.2.32 | NTC Version | 2017/9/21 |
| Patch Version | 1.0.2.31 | NTC Version | 2017/9/8 |
| Patch Version | 1.0.2.29 | Patch Version | 2017/8/3 |
| Patch Version | 1.0.2.28 | Patch Version | 2017/8/3 |
| Patch Version | 1.0.2.27 | Patch Version | 2017/7/26 |
| Patch Version | 1.0.2.26 | Patch Version | 2017/7/7 |
| Previous Version | 1.0.2.25 | NTC test | 2017/6/30 |
| Previous Version | 1.0.2.24 | Patch Version | 2017/6/22 |
| Previous Version | 1.0.2.23 | NTC test | 2017/6/9 |
| Previous Version | 1.0.2.22 | NTC test | 2017/5/12 |
| Previous Version | 1.0.2.21 | NTC test | 2017/2/8 |
| Previous Version | 1.0.2.20 | Patch Version | 2017/1/24 |
| Previous Version | 1.0.2.19 | NTC test | 2017/1/16 |
| Previous Version | 1.0.2.18 | Patch Version | 2017/1/6 |
| Previous Version | 1.0.2.17 | NTC test | 2016/12/19 |
| Previous Version | 1.0.2.15 | Patch Version | 2016/10/18 |
| Previous Version | 1.0.2.14 | NTC test | 2016/7/9 |
| Previous Version | 1.0.2.13 | NTC test | 2016/7/9 |
| Previous Version | 1.0.2.12 | NTC test | 2016/6/23 |
| Previous Version | 1.0.2.11 | NTC test | 2016/6/13 |
| Previous Version | 1.0.2.10 | NTC test | 2016/4/29 |
| Previous Version | 1.0.2.14 | NTC test | 2016/7/9 |
| Previous Version | 1.0.2.8 | NTC test | 2016/3/14 |
| Test Version | 1.0.2.7 | test | 2016/2/1 |
| Previous Version | 1.0.2.6 | NTC tested | 2016/1/22 |
| Previous Version | 1.0.2.0 | NTC tested | 2015/11/24 |

HARDware compatibility:

ES4528BH2-HPoE-EC

BootPROM compatibility:

ALL

Installation and Configuration Notes:

In general, the ECS4120 series Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS4120 series Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

To download new firmware form a TFTP server, enter the IP address of the TFTP server, select “opcode” as the file type, then enter the source and destination file names. To start the new firmware, enter the “reload” command or reboot the system.

CLI commands:

Console#copy tftp file

TFTP server IP address: 192.168.1.23

Choose file type:

1. config:  2. opcode:  : 2

Source file name: opcode.bix

Destination file name: opcode.bix

\Write to flash programming.

Programming flash started.

Success.

Console#reload

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Runtime V1.2.2.33

| ECS4120EC-23 | ECS4120 Series reboots by software watchdog L2MUX_GROUP （add workaround function） |
| --- | --- |
| ECS4120EC-65 | # Add reset the port configuration feature（support CLI and Web） |
| ECS4120EC-69 | # web support IPV6 Source Guard SIP and SDP mode at same port. |
| ECS4120EC-75 | # After command "no inclusion-vlan", the switch reboot by HW Watchdog due to SW Watchdog hang. |
| ECS4120EC-67 | # customized opcode to support special option82 circuit format-mib/web |

Runtime V1.2.2.32

| ECS4120EC-64 | # customized opcode to support special option82 circuit format |
| --- | --- |
| ECS4120EC-11 | # support IPV6 Source Guard SIP and SDP mode at same port |

Runtime V1.2.2.31

| ECS4120EC-23 | # ECS4120 Series reboots by software watchdog L2MUX_GROUP # （add debug message:1.add watchdog for Amtrl3_Group 2.add trace log for Amtrl3_Group if the Group watchdog timeout.3. add debug message for ipal_netlink,record the error message when always fail to recvmsg.4. optimize watchdog log which can dump gdb message for all threads） |
| --- | --- |
| ECS4120EC-25 | # [ERPS] when ERPS status changes, the console will very slow response and CPU will rises.（just optimize the RPL Owner side） |
| ECS4120EC-20 | NTP Server configure loss when reboot switch |
| ECS4120EC-34 | # [GVRP] CPU rises to 100% due to thread GVRP_GROUP. |

Runtime V1.2.2.30

| ECS4120EC-18 | # rollback VNPT enhance for standalone branch(rollback 4 EPR change ES4528BH2-HPoE-0815-EC-00948 /ES4528BH2-HPoE-0815-EC-00949/ ECS4120EC-1/ECS4120EC-4) |
| --- | --- |
| ECS4120EC-8 | ECS4120-28F/28F-I diag led test error |

Runtime V1.2.2.29

| ECS4120EC-1 | Add command to modify interface type for information option tr101 format. |
| --- | --- |
| ECS4120EC-2 | ERPS blocking port will forward PVST packets when enable dot1q-tunnel globally |
| ECS4120EC-4 | PPPoE Snooping add interface-type and vendor tag keep function |

Runtime V1.2.2.28

| ES4528BH2-HPoE-0815-EC-00966 | ACL:cannot bind acl on the egress port |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00950 | DHCP snooping not bind when same MAC, different VLAN get different IP |
|  | Add debug method for 3M issue (switch reboots by software watchdog L2MUX_GROUP) |

Runtime V1.2.2.27

| ES4528BH2-HPoE-0815-EC-00949 | [Enhancement] Enhance PPPoe IA to Support PPPoe Snooping |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00948 | VLAN translation cannot separate ingress & egress direction |
| ES4528BH2-HPoE-0815-EC-00946 | [Enhancement] Support DHCPSNP filter-only mode |
| ES4528BH2-HPoE-0815-EC-00944 | Keeping "show interface transceiver" will trigger the software watchdog. |
| ES4528BH2-HPoE-0815-EC-00942 | MAC ACL couldn't block the BPDU packet. |
| ES4528BH2-HPoE-0815-EC-00932 | [debug] The debug mode of the DHCPSNP missed the ACK packet information. |

Runtime V1.2.2.26; Loader V0.0.3.2/V0.0.4.2

| ES4528BH2-HPoE-0815-EC-00945 | system: show system garbled log and warm restart misjudentment |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00939 | ECS4120-28F-I and ECS4120-28Fv2-I apply new thermal plan |  |
| ES4528BH2-HPoE-0815-EC-00937 |  | ECS4120 doesn't support the "ip dhcp client hostname" function but has empty command |
| ES4528BH2-HPoE-0815-EC-00936 |  | HTTP-Download Config will get old config |

Loader V0.0.3.1

| ES4528BH2-HPoE-0815-EC-00902 | Switch reverts to the factory default setting when upgrading the opcode(for 0.0.3.0 version) |
| --- | --- |

Runtime V1.2.2.25; Loader V0.0.4.1

| ES4528BH2-HPoE-0815-EC-00930 | Port utilization info is incorrect on 10G port |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00927 |  | [Enhance] AAA TACACS: No system-auto privilege authorization following login authentication. |
| ES4528BH2-HPoE-0815-EC-00922 |  | Enabled both VLAN Trunking and Ingress Filtering, DAI trusted port does not forward unknown-VLAN ARP packets. |
| ES4528BH2-HPoE-0815-EC-00918 |  | Web ERPS with Multiple instance. |
| ES4528BH2-HPoE-0815-EC-00902 |  | Switch reverts to the factory default setting when upgrading the opcode |
| ES4528BH2-HPoE-0815-EC-00865 |  | Wrong description on CLI command. |

Runtime V1.2.2.24; Loader V0.0.4.0/0.0.3.0 (latest code from server)

| ES4528BH2-HPoE-0815-EC-00917 | User Layer 2 user entry to trap BPDU CPU intead of rule. |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00915 |  | switch will forward root port BPDU packets to Designate port (Fixed by EPR917). |
| ES4528BH2-HPoE-0815-EC-00914 |  | ERPS Trap OID is not consistent between MIB definition and actual packet's. |
| ES4528BH2-HPoE-0815-EC-00913 |  | delete/add arp entry cause inter vlan routing problem |
| ES4528BH2-HPoE-0815-EC-00904 |  | [DAI & vlan-trunking] switch will not forward the unknow VLAN ARP packet. |
| ES4528BH2-HPoE-0815-EC-00912 |  | mfg_runtime loaded but caused the dut fail boot up in acc recovery. (mfg_runtime only) |
| ES4528BH2-HPoE-0815-EC-00822 |  | DHCP Server: Failed to write excluded-address setting to running configuration. |
| ES4528BH2-HPoE-0815-EC-00909 |  | [DHCP relay] switch only send the DHCP packet to the last DHCP relay server even the last server does not exist.. |
| ES4528BH2-HPoE-0815-EC-00908 |  | MAC address of vlans are which are not in the allowed vlans list , are still being learned on the eth ports |

Runtime V1.2.2.23; Loader V0.0.4.0/0.0.3.0 (latest code from server)

| ECS4120-Stacking-00143 | RSTP: DUT does not reject to configure RSTP mode as spanning-tree is disabled |  |
| --- | --- | --- |
| ECS4120-Stacking-00144 |  | Runtime size generate via acc recovery is large in Dir (mfg_runtime) |
| ES4528BH2-HPoE-0815-EC-00900 |  | ERPS: After shutdown and no shutdown this trunk member ports, DUT cannot forward some VLAN-group data correctly. |
| ES4528BH2-HPoE-0815-EC-00905 |  | ERPS: When we configure vlan_group and Ring over maximum number, it doesn't return "Maximum number reached" in error |
| ECS4120-Stacking-00145 |  | loopback fail in Diag. |
| ES4528BH2-HPoE-0815-EC-00711 |  | numerous ARP packets impact the IGMP control packets |
| ES4528BH2-HPoE-0815-EC-00864 |  | Console locked is caused by loading attached config file when copy startup-config to running-config. (with LLDP-med function) |

Loader V0.0.4.0

| ES4528BH2-HPoE-0815-EC-00893 | [enhance] new flash partition for recovery and runtime partition |
| --- | --- |

Runtime V1.2.2.22; Loader V0.0.3.0 (code base v1.2.2.21)

| ES4528BH2-HPoE-0815-EC-00903 | static IPv6 Source Guard with full IPv6 address display problem. |
| --- | --- |

Runtime V1.2.2.21; Loader V0.0.3.0 (code base v1.2.2.19)

| ES4528BH2-HPoE-0815-EC-00897 | Static mac add/remove by SNMP failed. |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00895 |  | Dying-gasp: Dying-gasp trap is not working on authNoPriv and authPriv. |
| ES4528BH2-HPoE-0815-EC-00892 |  | DHCP-SNP: Add another entry with lower prefix than existing entry, DUT does not forward ipv6 traffic with lower prefix settin. |
| ES4528BH2-HPoE-0815-EC-00891 |  | IPv6 Source guard: When delete some sdp entry, other entry cannot work.. |
| ES4528BH2-HPoE-0815-EC-00750 |  | command FAN-Speed force-full does not work for 28FV2 |
| ES4528BH2-HPoE-0815-EC-00669 |  | DHCP-SNP: Enable DHCP-SNP and learning maximum MAC enrties, DUT cannot forward DHCP packet to trust port. |

Runtime V1.2.2.20; Loader V0.0.3.0 (code base v1.2.2.18)

| ES4528BH2-HPoE-0815-EC-00891 | IPv6 Source guard: When delete some sdp entry, other entry cannot work.. |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00892 |  | DHCP-SNP: Add another entry with lower prefix than existing entry, DUT does not forward ipv6 traffic with lower prefix settin. |

Runtime V1.2.2.19; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00884 | Dying gasp trap suport SNMPv3. |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00830 |  | The STP loopback detection fails after port shutdown / no shutdown. |
| ES4528BH2-HPoE-0815-EC-00817 |  | The ERPS Open Ring doesn’t change to Idle state after WTR time. (EPR816 fixed indirectly by EPR817) |
| ECS4120-Stacking-00128 |  | Loop detection: After we blobal enable loop detection in Master switch; CLI is halted and DUT rebooting itself.. |
| ECS4120-Stacking-00121 |  | Wrong password is saved to Mozilla Firefox web browser |

Runtime V1.2.2.18; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00886 | IPv6 Source guard: The same ipv6 sdp entry with prefix is replaced by another full ipv6 sip entry. |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00885 |  | IPv6 Source guard: The same ipv6 sdp entry with prefix 32 is replaced by another lower prefix (16) entry. |
| ES4528BH2-HPoE-0815-EC-00883 |  | IPv6 Source guard: When we try to delete one inexistent entry from DUT, but it remove other existing entry. |
| ES4528BH2-HPoE-0815-EC-00880 |  | ZTPv6: When we configure mutiple IPV6 interface, DUT fail to get this bootfile. |
| ES4528BH2-HPoE-0815-EC-00877 |  | [Source ::1 of the snmp traps ,it will be drop by NNMi Server, refinement for corner testing case. |
|  |  | Correct the CLI privilege issue caused by merged EPR 850: IPV6 prefix Guard |

Runtime V1.2.2.17; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00881 | Port description option enhancement missing Web Gui |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00877 |  | [Source ::1 of the snmp traps ,it will be drop by NNMi Server |
| ES4528BH2-HPoE-0815-EC-00876 |  | fail to set static ipv6 source guard binding for ipv6 prefix |
| ES4528BH2-HPoE-0815-EC-00875 |  | [WEB] Modify WEB for IPSGv6 and DHCPSNPv6 changes. For EPR850. Refined |
| ES4528BH2-HPoE-0815-EC-00873 |  | trap with hostname & remove IPv6 in variable binding |
| ES4528BH2-HPoE-0815-EC-00829 |  | Support DHCP op82 remote-id "port description" only (without any remote-id prefix), SNMP |
| ES4528BH2-HPoE-0815-EC-00793 |  | [Enhancement] Support to download exception log and watchdog log. |
| ES4528BH2-HPoE-0815-EC-00899 |  | Add missed function to initial dying gasp for SNMP process. |

Runtime V1.2.2.16; Loader V0.0.3.0

| ECS4120-Stacking-00131 | [Enhancement]Always set IFG to 20 for ingress port rate limit, just make code same as other model |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00818 |  | [Enhancement] add the "flooding behavior" command for DHCPSNP |
| ES4528BH2-HPoE-0815-EC-00820 |  | VRRP change status will cause memory leakage. |
| ES4528BH2-HPoE-0815-EC-00850 |  | [enhance] IPV6 prefix Guard request in SNMP |
| ES4528BH2-HPoE-0815-EC-00871 |  | TWAMP spec “Server-Greeting” message count field is at 262144, not 1024 |
| ES4528BH2-HPoE-0815-EC-00868 |  | [enhance] Add bid 15 for esc4120-28Fv2 based on board id 12 |
| ES4528BH2-HPoE-0815-EC-00875 |  | [WEB] Modify WEB for IPSGv6 and DHCPSNPv6 changes. For EPR850. |

Runtime V1.2.2.15; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00862 | access to wrong array index for swctrl_station_move_trap_per_port |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00853 |  | When the problematic transceiver is plugged in, it displayed the previous transceiver information, not checksum-error |
| ES4528BH2-HPoE-0815-EC-00850 |  | [enhance] IPV6 prefix Guard request in CLI |
| ES4528BH2-HPoE-0815-EC-00849 |  | ERPS: After shutdown and no shutdown Ring port in maximum instance setting; Port cannot receive all VLAN traffic. |
| ES4528BH2-HPoE-0815-EC-00808 |  | ERPS WTR timer should support range 1 - 12 minutes |
| ES4528BH2-HPoE-0815-EC-00790 |  | DHCP Snooping trust port will remove the option 82.. |
| ES4528BH2-HPoE-0815-EC-00731 |  | [Enhance] ERPS with Multiple Instance MIB |
| ECS4120-Stacking-00122 |  | Remove duplicate warning message while performing Dynamic DHCP-provision |

Runtime V1.2.2.14; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00860 | [WEB] TACACS supports IPv6 |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00851 |  | Modify the place of spanning-tree command in CLI running. |
| ES4528BH2-HPoE-0815-EC-00848 |  | Tacacs support over ipv6 |
| ES4528BH2-HPoE-0815-EC-00843 |  | SSH: The DSA option should be removed on WEB page |
| ES4528BH2-HPoE-0815-EC-00840 |  | [Enhancement] SW WatchDog monitor GVRP_GROUP |
| ES4528BH2-HPoE-0815-EC-00839 |  | IPv6 address prefix >64 for 4120 CLI |
| ES4528BH2-HPoE-0815-EC-00836 |  | [TACACS] Since authorization is supported, it's not necessary to grant privilege automatically after authentication. |
| ES4528BH2-HPoE-0815-EC-00831 |  | when port description length less then 8, web gui/snmp display problem |
| ES4528BH2-HPoE-0815-EC-00829 |  | Support DHCP op82 remote-id "port description" only (without any remote-id prefix) |
| ES4528BH2-HPoE-0815-EC-00828 |  | DHCPSNP option82 remote-id configure delimiter as 0(null), then Port Description field will not exist |
| ES4528BH2-HPoE-0815-EC-00835 |  | ECS4120-28F Inserting SGMII RJ45 SFP at port 1-24 when boot-up, affect config. |
| ES4528BH2-HPoE-0815-EC-00872 |  | ECS4120-28P PoE powering up PD for short period when warm rebooting even though the port is disable |

Runtime V1.2.2.13; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00836 | [TACACS] Since authorization is supported, it's not necessary to grant privilege automatically after authentication. |  |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00832 |  | [TACACS] Keep authentication behaviour. |
| ES4528BH2-HPoE-0815-EC-00827 |  | Enter the “break” hotkey then switch will print out the “SysRq” message on terminal.. |
| ES4528BH2-HPoE-0815-EC-00825 |  | MIB ,Getting the error while uploading in NNM |
| ES4528BH2-HPoE-0815-EC-00819 |  | (MSTP_issue)MST configuration of spanning-tree is deleted by device after reboot in v1.2.2.20 |
| ES4528BH2-HPoE-0815-EC-00786 |  | DAI works incorrectly with IPSG static binding |
| ES4528BH2-HPoE-0815-EC-00796 |  | DHCPv6: After change IPV6 with dhcp mode from AutoConfig, DUT cannot get DHCP ip address.. |
| ES4528BH2-HPoE-0815-EC-00761 |  | [XFER] Enhance XFER copy status message for UI. |
| ES4528BH2-HPoE-0815-EC-00731 |  | [Enhance] ERPS with Multiple Instance |
| ES4528BH2-HPoE-0815-EC-00728 |  | TWAMP prob and responder Support |
| ES4528BH2-HPoE-0815-EC-00643 |  | [Enhance] SSH: Upgrade SSH to version 7.8p1 |
| ES4528BH2-HPoE-0815-EC-00533 |  | After LOIC-SSH attack, switch will lose management |
| ES4528BH2-HPoE-0815-EC-00176 |  | StaticRoute: Static route doesn't support MIB.. |

Runtime V1.2.2.12; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00824 | Load runtime to ECS4120-28F, the port status always shows link down. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00823 | shutdown port via port security then DRIVER_GROUP_TX counter increase and cause watchdog reboot |

Runtime V1.2.2.10; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00812 | prevent ECS4120-28Fv2-I(BID=12) downgrade firmware to v1.2.2.8 and below. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00811 | system description is incorrect on new hardware ECS4120-28Fv2-I(BID=12) |
| ES4528BH2-HPoE-0815-EC-00810 | NTP: support to add or delete NTP IPv4/IPv6/hostname servers in SNMP private MIB |
| ES4528BH2-HPoE-0815-EC-00797 | ZTPv6: After restart this ZTPv6 for 10 times, DUT failed to get IPV6 address. |
| ES4528BH2-HPoE-0815-EC-00795 | DHCPv6: After restart DHCPv6 client on DUT continuously, system halted and show exception. |
| ES4528BH2-HPoE-0815-EC-00753 | ZTPv6: After DUT fail to get bootfile, CLI show exception. |

Runtime V1.2.2.9; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00807 | The ERPS convergence time more than 600ms, shall be less than 50ms |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00801 | new hw with board id 12 that 2 of 10G ports can not link-up and LED can not di[play |

Runtime V1.2.2.8; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00733 | [Enhancement] Support sticky mac behavior refinement. |
| --- | --- |

Runtime V1.2.2.7; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00800 | [Enhance] DHCP server |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00799 | switch still download config from TFTP server automatically even ip dhcp dynamic-provision is disabled. |
| ES4528BH2-HPoE-0815-EC-00798 | Master unit rebooted with an error message and changed its state to Slave after boot up which impacts Automation test |
| ES4528BH2-HPoE-0815-EC-00794 | After firmware upgrade and reboot, debug message print out continually. |
| ES4528BH2-HPoE-0815-EC-00792 | [Enhancement] sflow enabled/disabled syslog |
| ES4528BH2-HPoE-0815-EC-00791 | [enhance] error coding for QinQ default service MIB |
| ES4528BH2-HPoE-0815-EC-00783 | remove OSPF related OID from MIB file, since 4120 does not support OSPF |
| ES4528BH2-HPoE-0815-EC-00780 | [LLDP] Define default value for LLDP IPv6 address |
| ES4528BH2-HPoE-0815-EC-00769 | We can't get the maximum management ip if we let the vlan interface get ipv6 interface by autoconfig. |
| ES4528BH2-HPoE-0815-EC-00743 | 802.1ag-CFM-1170: TG1 will received tagged packets when Transmit LTM with MEP_2 destination address from DUT1PA |
| ES4528BH2-HPoE-0815-EC-00729 | [enhance] remove the setting of /127 Management vlan interface IPv6/v6 support |
| ES4528BH2-HPoE-0815-EC-00684 | MLDv1/v2 report/leave problem(33:33:00:00:00:01-02), L2/L3 table issue |
| ES4528BH2-HPoE-0815-EC-00353 | MLDSNP: Multicast data can not flood to other ports even MLDSNP is disabled. |

Runtime V1.2.2.6; Loader V0.0.3.0 (obslete)

|  | New stacking HW with new port mapping |
| --- | --- |

Runtime V1.2.2.5 rev; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00807 | The ERPS convergence time more than 600ms, shall be less than 50ms |
| --- | --- |

Runtime V1.2.2.5; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00789 | add bid 12 model name as ECS4120-28Fv2-I and Web display |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00788 | change log level to alert when soft TX disable is detected. |
| ES4528BH2-HPoE-0815-EC-00787 | SFP port can't link up |
| ES4528BH2-HPoE-0815-EC-00781 | The lifetime of the IPv6 address shows wrong |

Runtime V1.2.2.4; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00785 | NTP: support show commands to display more detailed information from NTP daemon |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00784 | [Enhance] Gets port status to link-down on GPON module insertion, which causes link-up first. |
| ES4528BH2-HPoE-0815-EC-00779 | [enhance] add retry for soc_i2c_read_word_data() & soc_i2c_write_word_data(). |
| ES4528BH2-HPoE-0815-EC-00778 | [Enhancement] Porting PortSationMoveAction related codes for sticky mac. |
| ES4528BH2-HPoE-0815-EC-00773 | the switch does not request ipv6 address after configure ipv6 address DHCP |
| ES4528BH2-HPoE-0815-EC-00767 | Sflow: The enable/disable is only display trap:263&264, no ecs4120TrapsPrap node.. |
| ES4528BH2-HPoE-0815-EC-00753 | ZTPv6: After DUT fail to get bootfile, CLI show exception. |
| ES4528BH2-HPoE-0815-EC-00735 | switch will forward the packet, even if the vlan state is suspend. |
| ES4528BH2-HPoE-0815-EC-00734 | [Enhancement] Extend the characters of remote-id and circuit-id for DHCPSNP option82 and PPPoEIA. |
| ES4528BH2-HPoE-0815-EC-00733 | [Enhancement] Support sticky mac. |

Runtime V1.2.2.3; Loader V0.0.3.0

| ES4528BH2-HPoE-0815-EC-00704 | [Enhancement] add sw-tx-disable workaround. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00732 | Can't apply ERPS settings when unchecking the box "Admin Status ENABLE. |
| ES4528BH2-HPoE-0815-EC-00745 | Thermal plan apply on ECS4120-28V2-I new layout and ECS4120-28F-I. |
| ES4528BH2-HPoE-0815-EC-00746 | [12Tv2] Source photonic GPON transceiver (SPS-34-24T-HP-TDFO-RR) EEPROM info. partially missing or overlapped. |
| ES4528BH2-HPoE-0815-EC-00749 | TACACS: The Global key would cover the server key. |
| ES4528BH2-HPoE-0815-EC-00750 | FAN-Speed force-full command does not work. |
| ES4528BH2-HPoE-0815-EC-00751 | NTP: The DUT doesn't send the NTP client packets when enable uthenticate and save/reboot. |
| ES4528BH2-HPoE-0815-EC-00755 | ImageDownload: Response "Unsupported version number. Not updated.", but announce "Success" |
| ES4528BH2-HPoE-0815-EC-00760 | authorization exec didn't work well. |
| ES4528BH2-HPoE-0815-EC-00762 | ZTP can’t autorun. |
| ES4528BH2-HPoE-0815-EC-00763 | [Enhancement] add I2C operation without offset |
| ES4528BH2-HPoE-0815-EC-00764 | When the share key is over 48, we will face watchdog reboot. |
| ES4528BH2-HPoE-0815-EC-00765 | acctonruntime=1 add into default uboot environment. |
| ES4528BH2-HPoE-0815-EC-00766 | The diag LED is always light red in ECS4120-28T. |
| ES4528BH2-HPoE-0815-EC-00768 | After ipv6 dhcp-provision get and apply the configuration, the setting "ipv6 address dhcp" won't work well. |
| ES4528BH2-HPoE-0815-EC-00771 | ECS4120-28F is unable restart ipv6 dynamic provision process after it's failed to get config from TFTP server. |
| ES4528BH2-HPoE-0815-EC-00772 | Information is inconsistent "show run interface vlan id 1" and "show run". |
| ES4528BH2-HPoE-0815-EC-00774 | VRRP change status will cause mac-learning invalid. |
| ES4528BH2-HPoE-0815-EC-00775 | It always displays "AUTOCONFIG" in show ipv6 interface even it's DHCP mode. |
| ES4528BH2-HPoE-0815-EC-00776 | Fixed need to press any key to trigger ZTP process. |
| ES4528BH2-HPoE-0815-EC-00777 | Fixed need to press any key to trigger ZTP process - another test procedures |

Runtime V1.2.2.2; Loader V0.0.2.7

| ES4528BH2-HPoE-0815-EC-00756 | MIB: There is no SFTP on MIB selection. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00752 | SYS_ADPT_MAX_NBR_OF_SWITCH_PORT_PER_CHIP is wrong. |
| ES4528BH2-HPoE-0815-EC-00748 | new model “ECS4120-28Fv3-I”for bid 12 that support stacking (can not tested). |
| ES4528BH2-HPoE-0815-EC-00747 | Static Mac-address-table entry won't work on lacp interface after reboot |
| ES4528BH2-HPoE-0815-EC-00744 | When we want to set "clock timezone-predefined" by telnet, then switch crash. |
| ES4528BH2-HPoE-0815-EC-00740 | [Enhancement] sflow enabled/disabled trap. |
| ES4528BH2-HPoE-0815-EC-00739 | Set the snmp version 3 inform, we will face exception. |
| ES4528BH2-HPoE-0815-EC-00738 | NTPv4 have long latency issue. |
| ES4528BH2-HPoE-0815-EC-00736 | [WEB] Enhance CGI for range list. |
| ES4528BH2-HPoE-0815-EC-00721 | STP state will flapping (non-fwd/fwd) when switch received the SyncE packet. |
| ES4528BH2-HPoE-0815-EC-00715 | EFM OAM remote-loopback test is abnormal. |

Runtime V1.2.2.1; Loader V0.0.2.7

| ES4528BH2-HPoE-0815-EC-00741 | Avoid new HW to load the old runtime image that is not supported new board id. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00737 | Get error message when we configure ntp server and client. |
| ES4528BH2-HPoE-0815-EC-00732 | Can't apply ERPS settings when unchecking the box "Admin Status ENABLE" on web GUI. |
| ES4528BH2-HPoE-0815-EC-00730 | [Enhancement] DUID Type 2 (DUID-EN) Support for DHCPv6 client |
| ES4528BH2-HPoE-0815-EC-00729 | [Enhancement] /31 Management vlan interface IPv4 support |
| ES4528BH2-HPoE-0815-EC-00727 | [Enhancement] new board id for ECS4120-28Fv2 /ECS4120-28Fv2-I for Web display |
| ES4528BH2-HPoE-0815-EC-00726 | Reachability to Multiple TACACS server |
| ES4528BH2-HPoE-0815-EC-00725 | [Enhancement] TransceiverAbsentTrap & TransceiverPresentTrap. |
| ES4528BH2-HPoE-0815-EC-00724 | [Enhancementment]ERPS trap. |
| ES4528BH2-HPoE-0815-EC-00720 | NTP: The DUT will crash when configure the NTP server with IPv6 address and save/reboot. |
| ES4528BH2-HPoE-0815-EC-00719 | NTP: Need to remove the polling time information on NTP version 4. |
| ES4528BH2-HPoE-0815-EC-00718 | [Enhancement] Raw data have the mechanism for the bad data bit-flip for uboot |
| ES4528BH2-HPoE-0815-EC-00712 | [Enhancement] use enhancement backdoor password |
| ES4528BH2-HPoE-0815-EC-00708 | [Enhancement] LLDP IPv6 Support |
| ES4528BH2-HPoE-0815-EC-00706 | Can't trigger the swSmtpConnFailure trap.. |
| ES4528BH2-HPoE-0815-EC-00679 | DHCPv6 options and ZTP for IPv6: |
| ES4528BH2-HPoE-0815-EC-00655 | system name was changed when configure interface vlan ip address |
| ES4528BH2-HPoE-0815-EC-00586 | IPv6-AddrConfig-0670: No static neighbors exist in ipv6 neighbour table after adding 10 static neighbor. |

Runtime V1.0.2.46; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00723 | [Enhancement] ECS4120-28Fv2 instead of ECS4120-28Fv3. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00717 | Can’t upload the new MIB for ECS4120 v1.0.2.45 which support dyingGaspMgt |
| ES4528BH2-HPoE-0815-EC-00716 | [Enhancement] SFTP |
| ES4528BH2-HPoE-0815-EC-00714 | enable IGMPSNP proxy-reporting and inject numerous IGMP report/leave cause exception and reboot by watchdog |
| ES4528BH2-HPoE-0815-EC-00713 | FS support the bit flip check in runtime image periodically |
| ES4528BH2-HPoE-0815-EC-00699 | [Enhancement] Add smart pair Web page |
| ES4528BH2-HPoE-0815-EC-00484 | WebUI: "Redundant Power Status" shall be removed because it does not support. |

Runtime V1.0.2.45; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00710 | [Enhancement] add exception log |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00709 | switch can't dump exception log & watchdog log via backdoor TFTP upload |
| ES4528BH2-HPoE-0815-EC-00707 | [Enhancement] new board id for ECS4120-28Fv3-I. |
| ES4528BH2-HPoE-0815-EC-00705 | [WEB] There is no place to configure ERPS VLAN-inclusion or exclusion. |
| ES4528BH2-HPoE-0815-EC-00703 | Configure UserAuth info via SNMP fail |
| ES4528BH2-HPoE-0815-EC-00702 | switch remark DSCP value when configure trust-mode to dscp |
| ES4528BH2-HPoE-0815-EC-00701 | [WEB] Administration > ERPS, Configure Details page may be blank. |
| ES4528BH2-HPoE-0815-EC-00700 | Have to remove SNMP TRAP swPowerStatusChangeTrap since hardware limitation. |
| ES4528BH2-HPoE-0815-EC-00698 | IGMPSNP: Priority in multicast data can not be changed when change initial priority 0 to 1~7. |
| ES4528BH2-HPoE-0815-EC-00697 | MLDSNP: The behavior is wrong when change unknown-multicast mode. |
| ES4528BH2-HPoE-0815-EC-00696 | failed to bind egress ACL when apply output service-policy first |
| ES4528BH2-HPoE-0815-EC-00695 | [Enhancement] Enable the smart pair |
| ES4528BH2-HPoE-0815-EC-00694 | [Enhancement] Enable ERPS for open ring & VLAN excluded/included on ring port |
| ES4528BH2-HPoE-0815-EC-00691 | Smart pair - rx counter > tx counter (smart pair link restore) |
| ES4528BH2-HPoE-0815-EC-00688 | SmartPair: Port shutdown causing longer failover time. failover is > 50ms if applying shutdown command |
| ES4528BH2-HPoE-0815-EC-00670 | ECS4120-28Fv2/ECS4120-28Fv2-I does not support media-type sfp-preferred-auto for combo port (21-22). |
| ES4528BH2-HPoE-0815-EC-00664 | MVR: The multicast traffic that from non-source port is forwarded to the other source port even there is no group subscribe. |
| ES4528BH2-HPoE-0815-EC-00656 | [Enhancement] Dying Gasp on ECS4120-28Fv2 & ECS4120-28Fv2-I |
| ES4528BH2-HPoE-0815-EC-00644 | SW Enhance NTPv4 |
| ES4528BH2-HPoE-0815-EC-00625 | MLDSNP: DTU forwards double queries to other ports. |
| ES4528BH2-HPoE-0815-EC-00563 | IGMP snooping v3:after disable and enable igmp snooping, DUT not forward the mc traffic after receive igmpv3 IS_in report. |
| ES4528BH2-HPoE-0815-EC-00559 | IGMP-Snooping-1180:DUT did not work well with static/dynamic group. |
| ES4528BH2-HPoE-0815-EC-00153 | IGMP-SNP: We can't enable igmp snooping immediate leave by host via SNMP.. |

Runtime V1.0.2.44; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00695 | [Enhancement] Disable the smart pair |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00694 | [Enhancement] Disable ERPS for open ring & VLAN excluded/included on ring port |
| ES4528BH2-HPoE-0815-EC-00693 | BCM task get into infinite loop and consume over 90% CPU process. |
| ES4528BH2-HPoE-0815-EC-00689 | MLD Snooping: When group is left, it will cause other group with same DA not forward |
| ES4528BH2-HPoE-0815-EC-00685 | IGMPSNP: When IGMPSNP priority is changed, multicast data can not be forwarded to register |
| ES4528BH2-HPoE-0815-EC-00683 | ipv6 multicast group which isn't learn on MLDSNP table isn't flooded to other ports. |
| ES4528BH2-HPoE-0815-EC-00682 | MLD snooping: do not flood ipv6 multicast traffic even disabling MDL snooping. |
| ES4528BH2-HPoE-0815-EC-00677 | After reboot LAG not calculate on MSTP |
| ES4528BH2-HPoE-0815-EC-00642 | MVR: Inject IGMP report overnight, DUT will auto reboot. |
| ES4528BH2-HPoE-0815-EC-00641 | IGMP-Snooping: The unregistered group will be forwarded. |
| ES4528BH2-HPoE-0815-EC-00624 | DHCP-SNP: The log format is incorrect as DUT failed to find egress port. |
| ES4528BH2-HPoE-0815-EC-00542 | Root Cost for MST 0 instance display incorrect. |
| ES4528BH2-HPoE-0815-EC-00249 | MLDSNP: Total entries always show 1. |
| ES4528BH2-HPoE-0815-EC-00173 | Web: Please remove "Redundant Power Status" from WebUI. (It's not displayed on CLI) |
| ES4528BH2-HPoE-0815-EC-00113 | Web: WebUI would logout automatically after idle 5 minutes. |
| ES4528BH2-HPoE-0815-EC-00062 | CLI: It shows "Not present" on "Redundant Power Status" when show version/system. |

Runtime V1.0.2.43; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00687 | ERPS/Smart pair convergence value is > 1.5s if backup port is port 1 (primary port is 2 - 8) |
| --- | --- |

Runtime V1.0.2.42; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00676 | [Enhancement]Force port link-down be detected if rx los interrupt is detected |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00675 | after reboot will fail to config remote-id and circuit-id of option82 and PPPoEIA with space in string |
| ES4528BH2-HPoE-0815-EC-00671 | IP address in arp inspection log are showing up reversed |
| ES4528BH2-HPoE-0815-EC-00666 | MLD Snooping will let ping6 lose connect. |
| ES4528BH2-HPoE-0815-EC-00662 | MVR: MVR priority does not work fine when priority is 7. |
| ES4528BH2-HPoE-0815-EC-00652 | SmartPair: The time of Rx Lose is over then 50 ms that is not match customer requirement. |
| ES4528BH2-HPoE-0815-EC-00645 | Log message in "show log flash" is cleared in case unplug power cord. |
| ES4528BH2-HPoE-0815-EC-00638 | MVR: The behavior is wrong in maximum MVR group situation. |
| ES4528BH2-HPoE-0815-EC-00637 | MLDSNP-proxy: SA and SIP are wrong when DUT sent the report to mrouter port. |
| ES4528BH2-HPoE-0815-EC-00595 | After kill WTDOG_PROC, show log ram is incorrect (refined) |
| ES4528BH2-HPoE-0815-EC-00532 | TACACS accounting:dut didnot suport "aaa authorization commands " setting or "show authorization Commands *" |
| ES4528BH2-HPoE-0815-EC-00526 | When switch enable DHCP Snooping & IP source-guard, DHCP renew cause packet loss(ECS4100-28T) |

Runtime V1.0.2.41; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00673 | max mldsnp groups is 1023 |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00672 | MLDSNPV2 IS exclude group not learning |
| ES4528BH2-HPoE-0815-EC-00665 | After enable mld snooping with ECS2100 mld querier, mld join report can't forward to mrouter port |
| ES4528BH2-HPoE-0815-EC-00663 | CFM: When MEP receive some error CCM frames, the error/RX CCM count is incorrect. |
| ES4528BH2-HPoE-0815-EC-00659 | WebAuth: The webauth function is failure. |
| ES4528BH2-HPoE-0815-EC-00654 | HP-NNMI MIB compiler error. |
| ES4528BH2-HPoE-0815-EC-00653 | add missing value for fileCopyDestOperType in MIB file |
| ES4528BH2-HPoE-0815-EC-00648 | 1QVLAN: DUT does not reject to delete the native VLAN by WEB, it cause PC cannot ping to DUT. |
| ES4528BH2-HPoE-0815-EC-00636 | MLDSNP-proxy: Received query from mrouter port would be flooded to other ports when IPv6 is enabled. |
| ES4528BH2-HPoE-0815-EC-00632 | IP SourceGuard: The IPSG ACL mode cannot work. |
| ES4528BH2-HPoE-0815-EC-00631 | If using Console login failed, the syslog could not send the syslog trap. |
| ES4528BH2-HPoE-0815-EC-00606 | IGMP-SNP: When IGMP group reach maximum 1024 groups, the multicast data flood. |
| ES4528BH2-HPoE-0815-EC-00595 | After kill WTDOG_PROC, show log ram is incorrect |
| ES4528BH2-HPoE-0815-EC-00553 | LLDP web:LLDP remote port/trunk "port id" information display in web is not correct |
| ES4528BH2-HPoE-0815-EC-00551 | CFM: The transmission interval of LBM and LTM is incorrect.. |
| ES4528BH2-HPoE-0815-EC-00550 | CFM: DUT should not learn aged LTR to linktrace cache. |
| ES4528BH2-HPoE-0815-EC-00549 | CFM: DUT should not reply tagged LTR as it is untagged VLAN member. |
| ES4528BH2-HPoE-0815-EC-00499 | loopback detection detects traffic loop even port is DOWN |
| ES4528BH2-HPoE-0815-EC-00465 | CFM: DUT should reject to set this same MEP_ID with remote MEP. |
| ES4528BH2-HPoE-0815-EC-00460 | Web-1QVLAN-0020:delete all vlan didnot successfully |
| ES4528BH2-HPoE-0815-EC-00329 | WEB-1QVLAN: The membership mode should not be tagged if there was no other VLAN joint. |
| ES4528BH2-HPoE-0815-EC-00628 | [Enhancement] Support ERPS for open ring & VLAN excluded/included on ring port |
| ES4528BH2-HPoE-0815-EC-00317 | enabled proxy-switching still reply the IGMP querier when MVR source port receive the join report |

Runtime V1.0.2.40; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00268 | MLDSNP: DUT sent 2 G-S query even robustness value was changed to 5. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00269 | MLDSNP: Registered group is deleted before it's expire time timeout. |
| ES4528BH2-HPoE-0815-EC-00327 | WEB-1QVLAN: The behavior in deleting VLANs from WEB encounter errors. |
| ES4528BH2-HPoE-0815-EC-00353 | MLDSNP: Multicast data can not flood to other ports even MLDSNP is disabled. |
| ES4528BH2-HPoE-0815-EC-00357 | Duplicate DHCP discover packet and maximum value of retransmission interval is not right |
| ES4528BH2-HPoE-0815-EC-00361 | DHCP dynamic-provision does not work on the other VLANs except the default VLAN 1. |
| ES4528BH2-HPoE-0815-EC-00459 | RSTP path cost will be fixed when static port-channel link up/down. |
| ES4528BH2-HPoE-0815-EC-00590 | SNMPWALK but get error about sysResourceMgt |
| ES4528BH2-HPoE-0815-EC-00615 | Default configure each interface will be set to "ethernet cfm port-enable" |
| ES4528BH2-HPoE-0815-EC-00618 | interface vlan IP(vlan 96) did not send the arp reply.. |
| ES4528BH2-HPoE-0815-EC-00629 | [Enhancement] Support Active/Standby static LAG |
| ES4528BH2-HPoE-0815-EC-00630 | DDoS: The default priority is not modifying when the port had once the authenticated. |

Runtime V1.0.2.39; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00355 | ECS4100-52T sends LLDP packet with no “port description, system name and system description” field after receiving Huawei s |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00362 | Huawei switch S2300 can't recognize ECS4100's LLDP packet with "dot1-tlv protocol identity". |
| ES4528BH2-HPoE-0815-EC-00369 | MSTP: Change the instance priority, the traffic will stop and recover the issue after 6 mins. (same as ES4528BH2-HPoE-0815-EC-00369/373) |
| ES4528BH2-HPoE-0815-EC-00612 | After disable then enable vlan on Web, vlan interface link down. |
| ES4528BH2-HPoE-0815-EC-00617 | ipv6 mld proxy-reporting have no function |
| ES4528BH2-HPoE-0815-EC-00620 | mld snooping : different member port inject the same join group packet, only the first one will be trapped to CPU |
| ES4528BH2-HPoE-0815-EC-00622 | STP Alternate port does not discard VLAN tagged BPDU packets |

Runtime V1.0.2.38; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00319 | endian problem is present in the LuaSocket library |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00379 | NTP-0040:dut did not handle NTP multiple servers well. |
| ES4528BH2-HPoE-0815-EC-00498 | Static port-channel cause loopback detection enable global failed |
| ES4528BH2-HPoE-0815-EC-00520 | CLI-Terminal-show any command about 10 times and set SNMP location will cause system crash.. |
| ES4528BH2-HPoE-0815-EC-00614 | MVR/IGMPSNP function to cause memory leak. |
| ES4528BH2-HPoE-0815-EC-00619 | ECS4120 series drop OSPF/RIP/VRRP packets when IGMPSNP enable and disable then MVR enable. |

Runtime V1.0.2.37; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00577 | WebUI: transceiver-monitor cannot enable trap messages |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00604 | ECS4120 series drop OSPF/RIP/VRRP packets when IGMPSNP or MVR is enabled.. |
| ES4528BH2-HPoE-0815-EC-00611 | [Enhancement] Use HW watchdog to monitor SW watchdog. |
| ES4528BH2-HPoE-0815-EC-00613 | ipv6 mld query-drop command have no function |

Runtime V1.0.2.36; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00456 | [Enhancement] Maximum number of IGMP snooping group support to 1024. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00459 | RSTP path cost will be fixed when static port-channel link up/down. |
| ES4528BH2-HPoE-0815-EC-00525 | static mac was deleted when switch enabled IPSG mac mode and receive DHCP release packet |
| ES4528BH2-HPoE-0815-EC-00528 | [Enhancement] Banner support 2048 characters |

| ES4528BH2-HPoE-0815-EC-00456 | [Enhancement] Maximum number of IGMP snooping group support to 1024. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00459 | RSTP path cost will be fixed when static port-channel link up/down. |
| ES4528BH2-HPoE-0815-EC-00525 | static mac was deleted when switch enabled IPSG mac mode and receive DHCP release packet |
| ES4528BH2-HPoE-0815-EC-00528 | [enhanement] Banner support 2048 characters |
| ES4528BH2-HPoE-0815-EC-00530 | Switch did not write the ARP entry when user access to switch via craft port. |
| ES4528BH2-HPoE-0815-EC-00531 | TACACS accounting:dut didnot suport "show authorization exec" |
| ES4528BH2-HPoE-0815-EC-00534 | remove accton string when boot up |
| ES4528BH2-HPoE-0815-EC-00536 | Traffic control min threshold is 500 pps, we want minimal value possible be lower, in example down to 10 pps |
| ES4528BH2-HPoE-0815-EC-00539 | Europe summer time end date is incorrect |
| ES4528BH2-HPoE-0815-EC-00541 | change spanning-tree mode will make the traffic stop. |
| ES4528BH2-HPoE-0815-EC-00543 | Fail to telnet EC switch(ECS4120) from Cisco device |
| ES4528BH2-HPoE-0815-EC-00544 | sflow packet agent address display wrong. |
| ES4528BH2-HPoE-0815-EC-00545 | switch could still learn the mac-address which belong none-vlan member even if enable the ingress filtering. |
| ES4528BH2-HPoE-0815-EC-00547 | sflow sample packet doesn't include IPv4 information. |
| ES4528BH2-HPoE-0815-EC-00548 | Failed to set the TACACS+ server for aaa accounting dot1x. |
| ES4528BH2-HPoE-0815-EC-00546 | [Enhancement] ECS4120 support 2.5G transceiver on the 10G uplink port |
| ES4528BH2-HPoE-0815-EC-00566 | LACP: The ingress Filter is failed when meets specific packets |
| ES4528BH2-HPoE-0815-EC-00588 | [Enhancement] Support read Total/Dynamic/Static mac count by SNMP |
| ES4528BH2-HPoE-0815-EC-00599 | [Enhancement] unify the design of configuration for jumbo frame |

Runtime V1.0.2.35; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00449 | FAN control for new model 28F-I ,28F-V2 and 28FV2-I. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00529 | Disable LBD trigger a LBD recover log. |

Runtime V1.0.2.34; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00455 | [Enhancement] support accounting of commands.. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00521 | Update MIB file for create time range from SNMP |
| ES4528BH2-HPoE-0815-EC-00524 | MSTP recovery time is too long 30~50 sec. |

Runtime V1.0.2.33; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00364 | DoS: Use LOIC to attack port 22, CLI has no response and DUT reboot by watchdog. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00448 | PPPoEIA can't work with QinQ |
| ES4528BH2-HPoE-0815-EC-00518 | switch dropped RSVP PATH packets. |

Runtime V1.0.2.32; Loader V0.0.2.6

|  | build en-us image (English Web interface) and remove zh-cn in this version. |
| --- | --- |
|  | Update the oid of 28Fv2–I model in adpt.h (RD internal) |

Runtime V1.0.2.31; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00445 | STP TCN log format change, and don’t log TC |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00450 | [Enhancement] Add board id 9 for 28Fv2-I |
| ES4528BH2-HPoE-0815-EC-00451 | [IGMP filter] the multicast traffic does not match rule, but it still will be forwarded.. |
| ES4528BH2-HPoE-0815-EC-00466 | Link state of LACP trunk member port changes between up and down repeatedly |
| ES4528BH2-HPoE-0815-EC-00511 | SNMP: The value of OID "lldpRemManAddrIfId" remove ".0" at the end. |
| ES4528BH2-HPoE-0815-EC-00512 | 28FV2 enhance function via rx loss singal detection |

Runtime V1.0.2.29; Loader V0.0.2.6

|  | build en-us image (English Web interface) and remove zh-cn in this version. |
| --- | --- |

Runtime V1.0.2.28; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00444 | enhance: add zh-cn for EC |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00447 | 10G port does not link-up issue on ECS4120-28F(V1.0.2.26).. |

Runtime V1.0.2.27; Loader V0.0.2.6

|  | 10G port does not link-up issue on ECS4120-28F(V1.0.2.26).. |
| --- | --- |

Runtime V1.0.2.26; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00441 | new oid from EC for each ECS4120 model. |
| --- | --- |

Runtime V1.0.2.25; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00431 | There is a typo on the DHCP L3 Relay help page. |
| --- | --- |

Runtime V1.0.2.24; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00359 | After reload , CPU Utilization incorrect bigger than 800M |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00411 | Non-UDP multicast packet will flooding |
| ES4528BH2-HPoE-0815-EC-00386 | new 28F Diag Suuport for LED , reset button test (support ECS4120-28Fv2) |

Runtime V1.0.2.23; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00381 | loopback detection log messages are incorrect |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00392 | SNMPWALK error message |
| ES4528BH2-HPoE-0815-EC-00393 | Interface information should be removed if it’s in L3 relay mode. |
| ES4528BH2-HPoE-0815-EC-00395 | Port security port move on DHCP SNP issue. |
| ES4528BH2-HPoE-0815-EC-00396 | WEB: DUT lose L3 DHCP-Relay description in Help page.. |
| ES4528BH2-HPoE-0815-EC-00397 | WEB: DUT should display correct L2/L3 Relay description in WEB page.. |
| ES4528BH2-HPoE-0815-EC-00398 | DHCP Relay: DUT should support to switch this L2/L3 Relay by WEB. |
| ES4528BH2-HPoE-0815-EC-00399 | ECS4120 mini console port should be removed from webpage. |
| ES4528BH2-HPoE-0815-EC-00401 | Inject the LLDP packet, then dashboard and show remote device information will empty. |
| ES4528BH2-HPoE-0815-EC-00382 | [Enhancement]Support new board with board id 7(28F add i2C interface) |

Runtime V1.0.2.22; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00363 | DHCPv6 client can't get ip address when enable IPv6 source guard |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00367 | FW: LACP(When DUT has enable LACP and connect other switch at port 28, ping will fail.). |
| ES4528BH2-HPoE-0815-EC-00370 | snmpEngineTime didn't reset the value even switch reboot. |
| ES4528BH2-HPoE-0815-EC-00371 | [Telnet] After tool "LOIC" TCP:23port attack, modified thread pool for telnetd & tnshd. |
| ES4528BH2-HPoE-0815-EC-00375 | [WEB] Need to support panel for board ID 8 |
| ES4528BH2-HPoE-0815-EC-00378 | [Enhancement] switching B/W L2 DHCP relay and L3 DHCP relay |
| ES4528BH2-HPoE-0815-EC-00380 | remove "private-vlan" feature |
| ES4528BH2-HPoE-0815-EC-00391 | SNMP_TD crash |

Runtime V1.0.2.21; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00332 | STP : DUT2 console is down when spanning tree mode is stp |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00336 | IP-Cluster: Commander auto reboots after it logs off member switch. |
| ES4528BH2-HPoE-0815-EC-00338 | IGMP-SNP: After remove vlan, an exception occurs. |
| ES4528BH2-HPoE-0815-EC-00339 | TACACS+: The DUT will crash and reboot automatically when delete the tacacs key and login the local account. |
| ES4528BH2-HPoE-0815-EC-00348 | memory leakage when injecting specific lldp packets |

Runtime V1.0.2.20; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00335 | ECS4120-28F combo port stop forwarding when shutdown eth1/22. |
| --- | --- |

Runtime V1.0.2.19; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00314 |  | SNMP: Use SNMP get some MIB object, DUT return incorrect value. |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00347 | rate limit input works incorrect. |  |
| ES4528BH2-HPoE-0815-EC-00354 | Mib compile error |  |

Runtime V1.0.2.18; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00331 |  | switch didn't forward General Query to client. |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00333 | switch stops sending syslog trap to server |  |
| ES4528BH2-HPoE-0815-EC-00334 | IP-Cluster: IP Cluster Web Management Failed |  |
| ES4528BH2-HPoE-0815-EC-00340 | switch still send schedule report to all MVR source port |  |
| ES4528BH2-HPoE-0815-EC-00343 | Spanning-tree: We cannot enable the STP mode from WEB. |  |
| ES4528BH2-HPoE-0815-EC-00345 | RADIUS-Acct: The console and Web of accounting information that is mismatch |  |

Runtime V1.0.2.17; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00344 |  | enhance: new board id 8 for ECS4120-28F-I |
| --- | --- | --- |
|  | New thermal plan for Industry spec. |  |

Runtime V1.0.2.15; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00321 |  | remove transceiver info and option from WEB GUI |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00326 | For 28-SFP : Remove transceiver-monitor, transceiver-threshold, transceiver-threshold-auto from CLI |  |

Runtime V1.0.2.14; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00245 |  | [Enhanced] policy map support set ip dscp |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00284 | IGMP profile : display issue & configuration can't be saved |  |
| ES4528BH2-HPoE-0815-EC-00293 | ERPS: Data can't be forwarded while cross chip. |  |
| ES4528BH2-HPoE-0815-EC-00298 | EFM-OAM: There is no oamMgt MIB node. |  |
| ES4528BH2-HPoE-0815-EC-00300 | EFM-OAM: The OAM PDU formate is incorrect. |  |
| ES4528BH2-HPoE-0815-EC-00301 | EFM-OAM: EFM-OAM is not compatible with other device. |  |
| ES4528BH2-HPoE-0815-EC-00302 | EFM-OAM: The OAM dying-gasp bit is always true. |  |
| ES4528BH2-HPoE-0815-EC-00303 | TACACS+: The DUT will reboot automatically when user logins the DUT by TACACS account, but the TACACS key is mismatch. |  |
| ES4528BH2-HPoE-0815-EC-00311 | L2SW: DUT cannot forward packets to destination through different chip.. |  |

Runtime V1.0.2.13; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00295 |  | WEB: DUT should support the CFM Help page. |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00297 | Enhancement OAM IEEE 802.3ah EFM. |  |

Runtime V1.0.2.12; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00266 |  | Add ERPS and CFM. |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00273 | change EEE default to disable. |  |

Runtime V1.0.2.11; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00270 |  | ECS4120-28F media-type sfp-forced 100fx after reboot will linkup as 1000full.. |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00272 | ECS4120-28T/F 10G port(1/25-28) could not linkup on 1G speed with ECS4210 series 1G SFP port.. |  |

Runtime V1.0.2.10; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00080 |  | RMON2: This trap host cannot be configured successfully. |
| --- | --- | --- |
| ES4528BH2-HPoE-0815-EC-00155 | LLDP: DUT can't sent shutdown TLV after port shutdown. |  |
| ES4528BH2-HPoE-0815-EC-00202 | correction to grammar for reload command |  |
| ES4528BH2-HPoE-0815-EC-00203 | correction to spelling for "show access-list tcam-utilization" command |  |
| ES4528BH2-HPoE-0815-EC-00206 | HTTPs: The https certificate information is incorrect. |  |
| ES4528BH2-HPoE-0815-EC-00207 | The DUT will crash and reboot when telnet attack of DDOS |  |
| ES4528BH2-HPoE-0815-EC-00209 | correction to grammar, change "regularity" to "regularly" |  |
| ES4528BH2-HPoE-0815-EC-00210 | Interface > Port > History => interval has no unit of measure |  |
| ES4528BH2-HPoE-0815-EC-00213 | HelpPage: The help page of system will show "404 not found". |  |
| ES4528BH2-HPoE-0815-EC-00214 | IP Source Guard: The default maximum binding number is incorrect.. |  |
| ES4528BH2-HPoE-0815-EC-00217 | LLDP: Some typo need to bo modify in help page.. |  |
| ES4528BH2-HPoE-0815-EC-00219 | Dashboard: Switch events will be overlapped when multiple levels are existed. |  |
| ES4528BH2-HPoE-0815-EC-00220 | DHCP Relay: DUT should not forward DHCP-Request repeatedly as it is Relay agent. |  |
| ES4528BH2-HPoE-0815-EC-00224 | DHCP-Client: It shows maximum DHCP client ID could be 64, but actually it only accept 32 digits. |  |
| ES4528BH2-HPoE-0815-EC-00240 | non-native vlan mgmt. IP will dropped when VLAN-1 was “ip address dhcp”. |  |
| ES4528BH2-HPoE-0815-EC-00242 | PPPoE-IA: We can't configure remote-id delimiter by WEB. |  |
| ES4528BH2-HPoE-0815-EC-00246 | wrong format for igmpSnoopProfileRangeTable |  |
| ES4528BH2-HPoE-0815-EC-00250 | MLDSNP: Implement statistics on WebUI. |  |
| ES4528BH2-HPoE-0815-EC-00251 | MLDSNP: Some show commands caused DUT to crash and reboot. |  |
| ES4528BH2-HPoE-0815-EC-00252 | WEB: Some items between WEB/Help/Site Map are mismatch. |  |
| ES4528BH2-HPoE-0815-EC-00253 | The more the help page will showing the "404 not found" message. |  |
| ES4528BH2-HPoE-0815-EC-00254 | SNMP: We can't access private MIB node by MG-Soft. |  |
| ES4528BH2-HPoE-0815-EC-00255 | IGMP-SNP: When DUT receive leave message in maximum group contidition, all data flooded. |  |
| ES4528BH2-HPoE-0815-EC-00256 | ARP: Add the static arp entry, the DUT will still send ARP packets. |  |
| ES4528BH2-HPoE-0815-EC-00257 | switch default value : ingress-filtering should be disabled |  |
| ES4528BH2-HPoE-0815-EC-00258 | Interface > Port > History --> no unit of measure |  |
| ES4528BH2-HPoE-0815-EC-00259 | ECS4120-28F 1G SFP failed to linkup on 1G port |  |
| ES4528BH2-HPoE-0815-EC-00260 | remove IEEE 802.1ag (CFM) and ERPS |  |
| ES4528BH2-HPoE-0815-EC-00263 | [Fan] Implement new thermal plan |  |

Runtime V1.0.2.9; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00042 | enhancement 8032 ERPS |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00140 | 802.1X-VLAN: The DUT does not assign the guest Vlan by 802.1X authentication.. |
| ES4528BH2-HPoE-0815-EC-00163 | ARP: Can't create the static ARP entry. |
| ES4528BH2-HPoE-0815-EC-00222 | switch can't learn arp correctly when DAI enabled |
| ES4528BH2-HPoE-0815-EC-00225 | The help page of web will show the "404 not found" message.. |
| ES4528BH2-HPoE-0815-EC-00226 | IP Source Guard: The display parameters and iterm don't arrange in binding table neatly. |
| ES4528BH2-HPoE-0815-EC-00227 | Help: RSPAN session ID shall be only 1. |
| ES4528BH2-HPoE-0815-EC-00229 | IGMP-SNP: When multicast group reach to maximum value, most of data flooded. |
| ES4528BH2-HPoE-0815-EC-00230 | IPv6-MLD-SNP: Send IPv6 MLD SNP Query packet with FE80::4 will cause DUT crash. |
| ES4528BH2-HPoE-0815-EC-00231 | IGMP-Snooping: Self querier expired time should not be count down if Querier is disabled. |
| ES4528BH2-HPoE-0815-EC-00232 | WEB: DUT lose the "Relay" Help page.. |
| ES4528BH2-HPoE-0815-EC-00233 | DHCP Snooping: The display parameters and iterm don't arrange in interface-list neatly. |
| ES4528BH2-HPoE-0815-EC-00234 | WEB: DUT lose the "DHCP Snooping" Help page, and it return error. |
| ES4528BH2-HPoE-0815-EC-00235 | DAI: We can't specify target IP/MAC in ARP ACL. |
| ES4528BH2-HPoE-0815-EC-00236 | IGMP-SNP(help): The maximum multicast groups value in help is incorrect. |
| ES4528BH2-HPoE-0815-EC-00237 | DUT console hang on when MGMT port setting ip address. |
| ES4528BH2-HPoE-0815-EC-00238 | LLDP(help): Remove ECS4510 related from help page. |
| ES4528BH2-HPoE-0815-EC-00241 | Client-PC can't authorize with Radius if we are not configure static IP at VLAN-1. |

Runtime V1.0.2.8; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00054 | VLAN-Translation: The help page of web will show the "404 not found" message. |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00059 | DAI: After setting ARP inspection can't get the gateway mac address. |
| ES4528BH2-HPoE-0815-EC-00063 | Web: Got "404 Not Found" after click Interface>RSTP>? |
| ES4528BH2-HPoE-0815-EC-00074 | ECS4120 dynamic QoS support egress rate-limiting |
| ES4528BH2-HPoE-0815-EC-00097 | IGMP-SNP: When multicast entry over 1023, the multicast data will be flooded. |
| ES4528BH2-HPoE-0815-EC-00100 | RSTP:: When the release mode set to manual, the loop-detection action cannot work. |
| ES4528BH2-HPoE-0815-EC-00125 | Time-Range: Perodic rule can't be saved properly. |
| ES4528BH2-HPoE-0815-EC-00134 | Spanning-tree: When we loop the Ports of DUT, this spanning-tree status is wrong. |
| ES4528BH2-HPoE-0815-EC-00135 | Spanning-tree: When DUT detect spanning-tree loop-detection, it cannot detect traffic loops which happened on other ports. |
| ES4528BH2-HPoE-0815-EC-00145 | CoS: Change phb-queue is not working. |
| ES4528BH2-HPoE-0815-EC-00161 | Boot: POST_Main: READ EPLD_VERSION INFORMATIN fail, I2C device 0x2f write error |
| ES4528BH2-HPoE-0815-EC-00177 | StaticRoute: IPv6 routing entry doesn't be showed on WebUI. |
| ES4528BH2-HPoE-0815-EC-00184 | Time-Range: Periodic weekdays time range entry would disappear after save the config and reboot the device. |
| ES4528BH2-HPoE-0815-EC-00187 | CLI: (ECS4120-28P) It's always displayed "No SFP inserted" on ports 1-24 when SFP is inserted indeed. |
| ES4528BH2-HPoE-0815-EC-00188 | L2SW: The copper port LED is incorrect. |
| ES4528BH2-HPoE-0815-EC-00208 | IP Source Guard: We cannot create maximum binding entries on DUT. |
| ES4528BH2-HPoE-0815-EC-00216 | IGMP-Filter: MVR can't work with IGMP filter. |
| ES4528BH2-HPoE-0815-EC-00228 | change the default value from EC |

Runtime V1.0.2.7; Loader V0.0.2.6

|  | Fix the diag LED test issue |
| --- | --- |

Runtime V1.0.2.6; Loader V0.0.2.6

|  | Add Web help page |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00027 | does not support switchport mode access |
| ES4528BH2-HPoE-0815-EC-00043 | Pv6 ACL does not support next header & not support tcp/udp port No |
| ES4528BH2-HPoE-0815-EC-00049 | extend IPv6 ACL add SIP. |
| ES4528BH2-HPoE-0815-EC-00051 | NTP: The DUT doesn't send the request packets of ntp. |
| ES4528BH2-HPoE-0815-EC-00055 | CLI: Boot information shows one error message. |
| ES4528BH2-HPoE-0815-EC-00057 | IP Source Guard: After we enable IPSG, DUT cannot forward security IP traffic. |
| ES4528BH2-HPoE-0815-EC-00061 | HW: Diag led is lit red on 28T/52T/28P, and lit amber on 28F |
| ES4528BH2-HPoE-0815-EC-00076 | DHCP Snooping : follow new design. Independent DHCPSNP tab |
| ES4528BH2-HPoE-0815-EC-00077 | IPSG : support independent binding table, ACL mode, MAC mode ,static-filtering |
| ES4528BH2-HPoE-0815-EC-00081 | DHCP-Snooping: After we remove port member from trunk, DUT show unknown information. |
| ES4528BH2-HPoE-0815-EC-00082 | DHCP-SNP: DUT should not drop unicast DHCP-Request packets. |
| ES4528BH2-HPoE-0815-EC-00084 | DHCP-SNP: When the DHCP-Client move to another port, DUT create wrong DHCP-SNP binding. |
| ES4528BH2-HPoE-0815-EC-00085 | IP Source Guard: When we enable DHCP-SNP and IPSG with MAC mode; DUT should not discard all DHCP-Request packets. |
| ES4528BH2-HPoE-0815-EC-00094 | DHCP-SNP: When DUT receive unicast DHCP-Reply packets, DUT does not create DHCP-SNP entry.. |
| ES4528BH2-HPoE-0815-EC-00095 | DHCP-SNP: DUT should limit the DHCP Snooping maximum binding number; and it should drop unnecessary DHCP-Request packets. |
| ES4528BH2-HPoE-0815-EC-00108 | ACL: 802.3 filter in MAC ACL in egress directon is useless.. |
| ES4528BH2-HPoE-0815-EC-00109 | ACL: IP ACL is useless when packets are filtered by MAC ACL first. |
| ES4528BH2-HPoE-0815-EC-00115 | IGMP-Filter: Create IGMP profile with multiple range entry, it will merge to incorrect entry. |
| ES4528BH2-HPoE-0815-EC-00116 | IGMP-SNP: IGMP snooping statistics query displayed incorrect value. |
| ES4528BH2-HPoE-0815-EC-00123 | DHCP-SNP: DUT cannot learn maximum binding.. . |
| ES4528BH2-HPoE-0815-EC-00131 | MLDSNP: ICMPv6 checksum in GS query sent from DUT is incorrect. |
| ES4528BH2-HPoE-0815-EC-00137 | 802.1X: The supplicant does not authenticated when port-operation mode is single host or mac-base. |
| ES4528BH2-HPoE-0815-EC-00138 | IPV4 Address: DUT cannot send out unicast DHCP-Request packets. |
| ES4528BH2-HPoE-0815-EC-00141 | SMTP: DUT sent incorrect mail level after reboot. |
| ES4528BH2-HPoE-0815-EC-00143 | 802.1X: The eapol-pass-through doesn't work |
| ES4528BH2-HPoE-0815-EC-00149 | CLI: There is no debugging message when run "show debug ip dhcp snooping" |
| ES4528BH2-HPoE-0815-EC-00152 | IP Source Guard: DUT should reject to create IPSG entry over maximum binding limit. |
| ES4528BH2-HPoE-0815-EC-00154 | LLDP-MED: Non PoE device (52T) sent Power-via-MDI TLV. |
| ES4528BH2-HPoE-0815-EC-00157 | RSPAN: Got "DEV_SWDRV_SetRspanVlanTag(): Line = 16077 , |
| ES4528BH2-HPoE-0815-EC-00160 | DHCP-SNP: When one client get IP, but DUT should not create over 9000 entries. |
| ES4528BH2-HPoE-0815-EC-00165 | ARP: The Proxy-ARP doesn't work. |
| ES4528BH2-HPoE-0815-EC-00166 | ImageMgmt: The old image version will upgrade automatically when configure the auto upgrade settings |

Runtime V1.0.2.0; Loader V0.0.2.6

| ES4528BH2-HPoE-0815-EC-00020 | ECS4120 switch hangs up because of port security or show mac-table |
| --- | --- |
| ES4528BH2-HPoE-0815-EC-00022 | If this model does not support location-LED, please remove this command |
| ES4528BH2-HPoE-0815-EC-00023 | does not support show spanning-tree brief |
| ES4528BH2-HPoE-0815-EC-00024 | does not support show run i e 1/x |
| ES4528BH2-HPoE-0815-EC-00025 | ECS4210 should remove craft interface from ECS4120-28T |
| ES4528BH2-HPoE-0815-EC-00026 | Should remove this command, because it’s fan-less design. |
| ES4528BH2-HPoE-0815-EC-00029 | Support usbdisk ? If No, please remove command “umount" . |
| ES4528BH2-HPoE-0815-EC-00030 | does not support cable-diag |
| ES4528BH2-HPoE-0815-EC-00031 | QinQ: TPID is set on the interface, but it actually should be set on global. . |
| ES4528BH2-HPoE-0815-EC-00033 | Non-spanning tree loopback detection |
| ES4528BH2-HPoE-0815-EC-00044 | IPv6 standard ACL support source IP only |
| ES4528BH2-HPoE-0815-EC-00046 | IGMP-SNP: The value is incorrect when we set proxy query address by SNMP. |
| ES4528BH2-HPoE-0815-EC-00047 | IGMP-SNP: Perform SNMP walk for igmpSnoopMulticastGroupPorts/igmpSnoopMulticastGroupStatus can't be stop. |
| ES4528BH2-HPoE-0815-EC-00048 | Enhancement support ipv4 & ipv6 static routing |
| ES4528BH2-HPoE-0815-EC-00052 | WebAuth:Cannot open the web authentication page when user login the web management from https protocol. |
| ES4528BH2-HPoE-0815-EC-00053 | Web: Front panel and System Switch page are wrong on WebUI. |
| ES4528BH2-HPoE-0815-EC-00058 | UserAuth-Remote: The DUT will dump error message and reboot automatically |
| ES4528BH2-HPoE-0815-EC-00064 | Performance: Packet loss when run 2 pairs 10G over 8K |
| ES4528BH2-HPoE-0815-EC-00065 | ATC: Forwarding rate is not 128kpps(default Alarm Clear Threshold) by default when broadcaset ATC is triggered. . |
| ES4528BH2-HPoE-0815-EC-00066 | LLDP:The LLDP local device information is incorrent. . |
| ES4528BH2-HPoE-0815-EC-00069 | DHCP relay option 82 (per port + per vlan mode).. |
| ES4528BH2-HPoE-0815-EC-00078 | MSTP instance extended to 64 |
| ES4528BH2-HPoE-0815-EC-00079 | RMON: The oversize and jabbers counter cannot work. |
| ES4528BH2-HPoE-0815-EC-00086 | RSPAN: First session source port setting will be copied to other sessions when show running/startup. |
| ES4528BH2-HPoE-0815-EC-00087 | StormCtrl: Broadcast control is not accurate when it's 1518 bytes broadcast stream.. |
| ES4528BH2-HPoE-0815-EC-00088 | MAC-VLAN: CLI cannot create MAC-VLAN entry with mask, but WEB can configure mask. . |
| ES4528BH2-HPoE-0815-EC-00089 | MAC-VLAN: After we create MAC-VLAN entry with mask, DUT cannot forward all MAC-VLAN traffic. . |
| ES4528BH2-HPoE-0815-EC-00110 | LLDP: The TLV value of system capabilities are incorrect. |
| ES4528BH2-HPoE-0815-EC-00111 | Web: Media-type shows unknown characters |
| ES4528BH2-HPoE-0815-EC-00112 | Web: Switch information is always show "undefined". |
| ES4528BH2-HPoE-0815-EC-00116 | IGMP-SNP: IGMP snooping statistics query displayed incorrect value. |
| ES4528BH2-HPoE-0815-EC-00117 | MSTP: Current maximum instance of MSTP extended is 33(create 32 entries + 0 instance) |
| ES4528BH2-HPoE-0815-EC-00121 | MIB: No such object when get cpuCurrentUti.0 from MIB-browser |
| ES4528BH2-HPoE-0815-EC-00124 | CableDiag: No the test item of cable-diagnostics on web management |
| ES4528BH2-HPoE-0815-EC-00126 | PoE: High power command shall be removed. . |
| ES4528BH2-HPoE-0815-EC-00129 | CoS: CLI can't stop display queue mode after press other key. |
| ES4528BH2-HPoE-0815-EC-00150 | LACP: After save configuration and power off/on DUT, runtime will be disappeared.. |
| ES4528BH2-HPoE-0815-EC-00156 | RateLimit: Ratelimit is not working on trunk (52T). |

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

|  | Support one group for PORT MIRROR, and support one group for RSPAN. PORT MIRROR and RSPAN is mutually exclusive. |
| --- | --- |
|  | SFP(ECS4120-28F) model, there is no SFP information from port 1 to port 24. |
|  | not support sfp-preferred-auto in ECS4120 28F |
|  | IGMP group is 255 but ASIC vendor data sheet described is 256 because 1 group is reserved by default. |
|  | efm oam remote-loopback test but not receive the loopback test packet when spanning tree status is block. |
|  | 10G port with ET4502-DAC-5M and 3M link DOWN with 10Gbps, 1M do not have this issue |
|  | Due to HW limitation, ERPS is work as excepted on ECS4120-28FV2 or ECS4120-28FV2-I for fail recovery time ([ITU-T G.808.1]) for a failure on a ring link shall be less than 50 ms). It can not work on the copper port. It also can not support on the SFP module that do not support the RX Loss signial, and the DAC cable also can not be supported. |
|  | Due to ASIC limitation, mac-vlan don't support mask option |
| ES4528BH2-HPoE-0815-EC-00327 | The browser need IE 9+, Chrome 54+, Firefox 52+ or Opera 41+ for this case: WEB-1QVLAN: The behavior in deleting VLANs from WEB encounter errors |
| ES4528BH2-HPoE-0815-EC-00741 | avoid new HW to load the old runtime image that is not supported new board id, (v1.2.2.1 can not download the image version less than v1.0.2.45) |
|  | Dying gasp HW supported on ECS4120-28Fv2-I (newLayout, stacking) bid 12 |