ES4510MA-PD-FLF-EC

Firmware Version 1.0.2.5

INTRODUCTION:

ES4510MA-PD-FLF-EC supports 10 10/100/1000 Base-T ports with 2 Gigabit fiber ports, which are comprised of an RJ45 and a SFP.* 10/100/1000 CU built-in associated Mini-GBIC slots

Using IP Clustering for stacking which is a method of grouping switches regardless of geographical location in order to enable centralized management access to the switches through a single management point. This technology provides some of the benefits of stackable switches without requiring direct stacking or connections between switch units. It also allows different kinds of switches to be grouped together, as long as they have the embedded clustering software.

It offers advanced administration through a user-friendly browser interface. QoS and VLAN Supports allows greater scalability and Supports for business-critical applications. Port can be trunked to create a high bandwidth pipe to the network.

It also has comprehensive network management functions, such as Spanning Tree protocol for standard bridging, SNMP, RMON and security features.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

Loader:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.0.0.6 | Formal release | 2014/04/01 |
| Current Version | 1.0.0.5 | Formal release | 2013/10/04 |
| Test Version | 1.0.0.4 | Release candidate | 2013/08/07 |

Runtime:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.0.2.5 | Patch, TS test | 2015/06/09 |
| Previous Version | 1.0.2.4 | Patch, TS test | 2015/04/23 |
| Previous Version | 1.0.2.3 | Patch, TS test | 2015/04/07 |
| Previous Version | 1.0.2.2 | Release candidate | 2013/08/07 |
| Previous Version | 1.0.2.0 | Release candidate | 2013/07/18 |
| Test Version | 1.0.1.2 | NTC test | 2013/05/20 |
| Test Version | 1.0.0.2 | RD test | 2013/03/22 |

HARDware compatibility:

Note:

Port 1~8 support PoE as Powered Devices (PD) with 802.3at compliant.  Default is enable to connect PSE device only (If the partner is not PSE, port will be shutdown.)  To disable this function, you can use “ no power-source-check” command.

If you connect PoE to port 1~8 over 1 port, hardware will only use one port power, but we don’t know which port. It’s decided by hardware.  When you disconnect the port which hardware use its power, hardware will change to another PD port immediately.  So, switch still can work well.

But, when you just connect one PoE to port 1~8, hardware need time to do hand shacking.  Before it’s done, you should not disconnect other PD port.  Otherwise, it may cause switch power down if hardware use that port power.  To make sure hardware finish hand shaking, you can use “show power-source-status” to check that port display 802.3af or 802.3at.

Port 10 support PoE as Power Sourcing Equipment (PSE) with 802.3af compliant.  Default is disable.  It will be enabled when DC power is ON or one of port 1~8 has 802.3at power.

BootPROM compatibility:

ALL

Network Management Software Support:

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Accton Product Feature List |  | ES4510MA-PD-FLF-EC |
| --- | --- | --- |
| Product Configuration |  | 10GE + 2 Fiber |
| Flash Memory |  | 32MB |
| SDRAM |  | 128MB |
| L2 MAC Table |  | 16K |
| PoE |  | YES |
| Standalone/Stackable |  | Standalone |
| Layer 2 Features |  |  |
| Link Aggregation | 802.3ad with LACP | YES 1) Total 6 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk GE |
|  | Cisco EtherChannel Liked | YES 1) Total 6 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk GE |
|  | Unicast / Multicast traffic Balance over Trunking port | YES 1) Unicast : MAC SA+DA/ SA/DA/ SIP+DIP/SIP/DIP |
| VLAN | IEEE 802.1Q Tagged Based | YES |
|  | Port-Based | YES (4K VLANs) |
|  | MAC-Based | YES |
|  | IP-Based | YES |
|  | Private VLAN | YES (Traffic Segmentation per port/VLAN by chip) |
|  | GVRP | YES |
|  | 802.1v Protocol | YES |
|  | IPv6 VLANs | YES |
|  | voice VLAN | YES |
|  | 4K (4092) VLANs | YES |
|  | VLAN Trunking | YES |
| Spanning Tree | IEEE 802.1D | YES |
|  | IEEE 802.1s | YES (32 instances) |
|  | IEEE 802.1w | YES |
|  | Spanning Tree Fast Forwarding | YES |
|  | Loopback Detection | YES |
|  | Auto Edge Port | YES |
|  | BPDU Filter/Guard | YES |
|  | Root Guard | YES |
|  | BPDU Forward | YES |
| Broadcast/Multicast/UnknownUnicast Storm Control |  | YES (no Broadcast, B+M, B+M+DLF limit) |
| IGMP Snooping | IGMP Snooping v1/v2 | YES |
|  | IGMP v1/v2 querier support | YES |
|  | IGMP Immediate Leave | YES |
|  | IGMP Filtering/Throttling | YES |
|  | IGMP SNP Leave Proxy | YES |
|  | IGMP v1/v2/v3 Proxy | YES |
|  | IGMPV3 Snooping | NO |
|  | Query Drop | YES |
| MVR |  | YES (groups - admin: 1024, active: 255 in phase 2) |
| Jumbo Frame |  | 10K |
| QinQ |  | YES |
| select QinQ |  | YES |
| EAPS |  | NO |
| Auto MDI/MDIX |  | Auto |
| Types of GBIC and SFP support (LX, SX, LH, FX) |  | YES |
| G.8032 (ERPS) | V2 | YES (support 6 rings) |
| Non-STP loopback Detection |  | YES |
| UDLD |  | YES |
| Digital Diagnostic Monitor (DDM) |  | YES |
| Flow Control |  | YES |
| G.8032 (ERPS) |  | YES |
| Y.1731 |  | YES |
| 802.1ag (CFM) |  | YES |
| 802.3ah (OAM) | Customer Premises Equipment (CPE) | YES |
| QoS Features |  |  |
| Number of priority queues |  | 4 queues/port |
| Scheduling for priority queues | WRR Priority scheduling | YES (Shaped Deficit WRR) |
|  | Strict Priority scheduling | YES |
|  | Hybrid | YES (Shaped Deficit WRR + Strict) |
| COS | 802.1p based COS | YES (per system) |
|  | IP TOS Precedence based COS | NO |
|  | IP DSCP based COS | DSCP Only (per system) |
|  | TCP/UDP Port based COS | YES (no command) |
| Rate Limiting | Ingress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
|  | Egress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
| DiffServ |  | YES |
| Security Features |  |  |
| Static Port Security (MAC-based) |  | YES |
| Dynamic Port Security (MAC-based) |  | YES |
| MAC Limitation per port |  | YES |
| 802.1x (authenticator) | Port-Based | YES |
|  | MAC-Based | YES |
|  | VLAN Assignment | YES |
|  | QoS Assignment (w/o ACL profile) | YES |
|  | QoS assignment w/ ACL profile | YES |
|  | Guest VLAN | YES |
|  | Supplicant Support | YES |
|  | Supplicator | YES |
|  | EAPOL frames pass-through | YES |
| MAC Authentication |  | YES |
| WEB Authentication |  | YES |
| ACL | Number of rules | 512 (extended mode 256 rules) |
|  | L2/L3/L4 | YES 1) L2 SA/DA MAC, VLAN based 2) L3 IP SA/DA, subnet based 3) L4 TCP/UDP port 4) IPv6 |
|  | Mirror | YES |
|  | Ingress | YES |
|  | Egress | NO |
|  | Statistics | YES |
|  | Time-Based (Time Range) | YES |
| AAA | RADIUS authentication | YES |
|  | RADIUS authorization and accounting | YES |
|  | TACACS+ Authentication | YES |
|  | TACACS+ authorizaion | YES |
|  | TACACS+ accounting | YES |
| HTTPS and SSL (Secured Web) |  | YES |
| Denial of Service Protection (DoS) |  | YES |
| SSH 1.5/V2.0 (Secured Telnet session) |  | YES |
| User name password authentication | Local Authentication | YES |
|  | Remote Authentication via RADIUS/TACACS+ | YES |
| Management Interface Access Filtering | SNMP | YES |
|  | WEB | YES |
|  | TELNET | YES |
| Intrusion Lock (Link Detection) |  | YES |
| MAC filter |  | YES |
| Traffic Segmentation |  | YES |
| PPPoE Intermediate Agent |  | Yes |
| Management Features |  |  |
| Cisco Like CLI |  | YES |
| CLI Banner |  | YES |
| CLI filtering and terminal setting |  | NO (will add in future) |
| Multiple Management IP Interface |  | YES |
| Web Based Management |  | YES |
| Telnet | Client | YES |
|  | Server | YES (8) |
| Software Download/Upgrade | TFTP | YES (dual images) |
|  | Xmodem / Ymodem | YES (Runtime, Loader) |
|  | Diag support TFTP download | NO |
|  | FTP | YES |
|  | HTTP | YES (dual images) |
| Dual Image |  | YES |
| Configuration Download/Upload | TFTP | YES (Multiple Copies) |
|  | HTTP | YES (Multiple Copies) |
| Auto upgrade | TFTP | YES |
|  | FTP | YES |
| SNMP | v1 | YES |
|  | v2c | YES |
|  | v3 | YES |
| RMON | RMON I (1,2,3,9 group) | YES |
|  | RMON 2 | YES |
| MIB I/II | RFC 1213 | YES |
| BOOTP | Client | YES |
|  | Relay | NO |
| DHCP | Client | YES |
|  | Relay | NO |
|  | Snooping | YES |
|  | Snooping Option 82 | YES |
|  | Dynamic Provision (via Option 66,67) | YES |
|  | Option 82 | YES |
|  | Relay option 82 | YES |
|  | Server | No |
| IP Source Guard |  | YES (user can config max. binding entry) |
| Port Mirroring |  | YES |
| Remote Port Mirroring |  | YES |
| Event/Error Logging | Syslog (Local Flash) | YES 1) 512 entries 2) 256Kbyte Flash |
|  | Remotelog (RFC3164) | YES 1) Cisco-like 2) Support up to 5 Servers |
|  | SMTP (E-mail Notification) | YES |
| DNS | Client | YES |
|  | Proxy | YES |
| Remote Ping |  | YES |
| SNTP |  | YES |
| NTP |  | Yes |
| IP Clustering |  | YES (36 members) |
| LLDP(802.1ab) | Link Layer Discovery Protocol | YES |
|  | LLDP-MED | YES |
|  | IEEE 802.3at | NO |
| UPnP |  | NO |
| Banner |  | YES |
| MAC Flush |  | YES |
| sFlow |  | NO (chip limitation) |
| Dynamic ARP inspection (DAI) |  | YES |
| VLAN mirror |  | YES |
| MAC Based mirror |  | YES |
| ATC |  | YES |
| Delay reload |  | YES |
| CLI “show debug” |  | NO |
| CLI “show tech” |  | YES |
| CLI “show interfaces brief” |  | YES |
| Cable Diagnostics / TDR |  | YES |
| Port Utilization (kbits/sec, Pkts/sec, % Util in recent 300secs) |  | YES |
| IPv6 Management (Telnet Server/ICMP v6) |  | YES |
| Green Ethernet |  | YES |
| PoE |  | YES (only show status, PD can be disable) |
| Monitor Environment | Power Status | NO |
| IPv6 Features |  |  |
| IPv4/IPv6 Dual Protocol Stack |  | YES |
| IPv6 Address Types | Unicast | YES |
|  | Multicast | This is only used internally. Not configurable. |
| ICMPv6 |  | YES |
| ICMPv6 Redirect (Host) |  | YES |
| IPv6 Path MTU Discovery |  | YES |
| IPv6 Neighbor Discovery | Router Discovery | YES |
|  | Duplicate Address Detection | YES |
|  | Static Cache Entry | NO |
|  | Parameter Discovery | YES |
|  | Prefix Discovery | YES |
|  | Address Resolution | YES |
|  | Unreachable Neighbor Detection | YES |
|  | Redirect (Host) | YES |
| Stateless Auto configuration |  | YES |
| Manual Configuration |  | YES |
| Generic Prefix |  | NO |
| SNMP over IPv6 |  | YES |
| HTTP over IPv6 |  | YES |
| SSH over IPv6 |  | YES |
| IPv6 Telnet Support |  | YES |
| IPv6 DNS Resolver |  | YES |
| IPv6 RADIUS+ Support |  | NO |
| IPv6 TACACS+ Support |  | NO |
| IPv6 Syslog Support |  | YES |
| IPv6 SNTP Support |  | YES |
| IPv6 SMTP Support |  | NO |
| IPv6 TFTP Support |  | YES (CLI not support) |
| Remote IPv6 Ping |  | YES |
| Ping over IPv6 |  | YES |
| Trace Route over IPv6 |  | YES |
| IPv6 sFlow |  | NO |
| MVR6 |  | NO |
| IPv6 Source Guard |  | NO |
| RA Guard |  | YES |
| MLD Snooping |  | NO |
| IPv6 ND Snooping |  | NO |
| IPv6 QoS |  |  |
| IPv6 ACL | Number of rules: |  |
|  | L2/L3/L4 | YES |
| IPv6 DSCP remapping CoS |  | NO |

Installation and Configuration Notes:

In general, the ECS4510-12PD Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS4510-12PD Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

To download new firmware form a TFTP server, enter the IP address of the TFTP server, select “opcode” as the file type, then enter the source and destination file names. To start the new firmware, enter the “reload” command or reboot the system.

CLI commands:

Console#copy tftp file

TFTP server IP address: 192.168.1.23

Choose file type:

1. config:  2. opcode:  : 2

Source file name: opcode.bix

Destination file name: opcode.bix

\Write to flash programming.

Programming flash started.

Success.

Console#reload

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

LOADER CODE modifications:

Loader V1.0.0.6

| ES4510MA-PD-FLF-EC-00097 | Enhancement: Modify DRAM initial for new DRAM chip and clock rate |
| --- | --- |

Loader V1.0.0.5

| ES4510MA-PD-FLF-EC-00093 | Loader: For R01 unit, loader should not pull MPP 7 & 18 to low |
| --- | --- |

Loader V1.0.0.4

| ES4510MA-PD-FLF-EC-00033 | Loader: In bootup time, port 9~12 are in linkup status |
| --- | --- |

Runtime CODE modifications:

Runtime V1.0.2.5 ;

| ES4510MA-PD-FLF-EC-00117 | [enhance] display total flash size in CLI 'dir' command for EC products. |
| --- | --- |
| ES4510MA-PD-FLF-EC-00112 | New features: summer-time & timezone-predefined on system time UI available on CLI, Web, and SNMP |
| ES4510MA-PD-FLF-EC-00111 | 1QVLAN: DUT cannot show all maximum VLANs, and it exit CLI session automatically. |
| ES4510MA-PD-FLF-EC-00110 | remove spanning tree MSTP mode then switch will hang in boot process |
| ES4510MA-PD-FLF-EC-00109 | RMON: Input snmpset command by net-snmp, the system will shows exception log. |
| ES4510MA-PD-FLF-EC-00108 | Stacking: The DUT will dump exception stackframe and crash when user execute the "copy start to running" command. |
| ES4510MA-PD-FLF-EC-00106 | correction to spelling/grammar - show ip igmp authentication |
| ES4510MA-PD-FLF-EC-00104 | IGMP V3 Membership Query cause show run display igmp snooping vlan proxy-address |

Runtime V1.0.2.4 ;

| ES4510MA-PD-FLF-EC-00105 | edit error in config file manually and copy to startup-config will reset switch to factory default |
| --- | --- |

Runtime V1.0.2.3 ;

| ES4510MA-PD-FLF-EC-00102 | Power-source-check by port On/Off |
| --- | --- |
| ES4510MA-PD-FLF-EC-00101 | SFP: modify SFP configuration for non-PHY SFP port |

Runtime V1.0.2.2 ;

| ES4510MA-PD-FLF-EC-00035 | Diag extlb/wsextlb test fails sometimes after execution of intlb |
| --- | --- |
| ES4510MA-PD-FLF-EC-00082 | [Security] Incorrect mac type when DUT receives ICMPV6 Router Solicitation packet |
| ES4510MA-PD-FLF-EC-00048 | PoE:It always show System Operation Status :Off and PoE Power Consumption:0 Watts in global configure page on web. |
| ES4510MA-PD-FLF-EC-00047 | PoE:It doesn't have description about power allocation range on web. |
| ES4510MA-PD-FLF-EC-00046 | PoE:Power Consumption column always displays "0" on web. |
| ES4510MA-PD-FLF-EC-00045 | PoE:Command "show power inline status ethernet 1/10" doesn't provide Power Consumption column information |
| ES4510MA-PD-FLF-EC-00044 | PoE:DUT doesn't provide command to set PoE priority & power allocation on port interface via CLI. |
| ES4510MA-PD-FLF-EC-00051 | PoE:Power allocation doesn't work, even PD need more than the value of power allocation, it also provide power to PD. |
| ES4510MA-PD-FLF-EC-00043 | PoE:Insert to PSEs to DUT port1,2 , disconnect port1, DUT will power off then reboot automatically. |
| ES4510MA-PD-FLF-EC-00087 | System:After disable gvrp global & per port,"show ip igmp snooping mrouter" cause DUT display exception then DUT reboot autom |
| ES4510MA-PD-FLF-EC-00078 | DHCP SNP: After enable option 82, client can not get ip from server. |
| ES4510MA-PD-FLF-EC-00076 | PoE:Click Administrator > PoE > PSE page,select step2. Configure Interface, DUT display exception. |
| ES4510MA-PD-FLF-EC-00027 | SFP Port LINK , Show Interface Brief is Auto-10Full |
| ES4510MA-PD-FLF-EC-00031 | Show IP Traffic almost is empty |
| ES4510MA-PD-FLF-EC-00052 | PoE:CLI doesn't provide command to show poe global config. |
| ES4510MA-PD-FLF-EC-00068 | L2SW: The LED of DUTP11/DUTP12 is not flashing, when transmit packets from DUTP11/DUTP12. |

Runtime V1.0.2.0 ; Loader V1.0.0.3

| ES4510MA-PD-FLF-EC-00016 | [basic function] In runtime pkt will loss, root cause is "Fragments". |
| --- | --- |
| ES4510MA-PD-FLF-EC-00039 | DHCP Snooping Option82:When dhcp client get IP from DHCP Server, DUT would appear error message |
| ES4510MA-PD-FLF-EC-00071 | ACL: Can't configure/show ACL hardware counter via CLI. |
| ES4510MA-PD-FLF-EC-00049 | PoE:The value of PoE Maximum Available Power in global configure page is not correct. |
| ES4510MA-PD-FLF-EC-00074 | NTP cause CPU alarm time counter error |
| ES4510MA-PD-FLF-EC-00042 | Remote Syslog : Syslog host will Not dispaly in WEB GUI after save and reboot |
| ES4510MA-PD-FLF-EC-00038 | Enhancement: Add OID for power-source-check and power-source-status |
| ES4510MA-PD-FLF-EC-00040 | LED:DUT port11,12 doesn't connect , but LED of port11,12 is light. |
| ES4510MA-PD-FLF-EC-00064 | RFC-2889 Partially Meashed:the DUT has the packet loss issue in port#1~8. |
| ES4510MA-PD-FLF-EC-00063 | RFC-2544 Back to Back : the DUT has the packet loss issue in port#1~8. |
| ES4510MA-PD-FLF-EC-00062 | RFC-2544 Packet Lost : the DUT has the packet loss issue in port#1~8. |
| ES4510MA-PD-FLF-EC-00061 | RFC-2544 Latency : the DUT has the packet loss issue in port#1~8. |
| ES4510MA-PD-FLF-EC-00060 | RFC-2544 Throughput : the DUT will have the packet loss issue in port#1~8. |
| ES4510MA-PD-FLF-EC-00065 | RFC-2889 Fully Meashed:the DUT has the packet loss issue in port#1~8. |
| ES4510MA-PD-FLF-EC-00057 | L2SW: when the flow control in the half mode is "Back pressure", another port can not receive all the unicast packets . |
| ES4510MA-PD-FLF-EC-00066 | Storm Control: When the Tx rate above broadcast storm limit setup, the Rx rate does not equal to broadcast threshold. |
| ES4510MA-PD-FLF-EC-00058 | L2SW: flow control do not work at some ports. |
| ES4510MA-PD-FLF-EC-00053 | PoE:DUT doesn't provide power source check configure on web. |
| ES4510MA-PD-FLF-EC-00052 | PoE:CLI doesn't provide command to show poe global config. |
| ES4510MA-PD-FLF-EC-00051 | PoE:Power allocation doesn't work, even PD need more than the value of power allocation, it also provide power to PD. |
| ES4510MA-PD-FLF-EC-00050 | Suggestion:In global configure page on web, no column can be configured, the page name modify to global information is better |
| ES4510MA-PD-FLF-EC-00048 | PoE:It always show System Operation Status :Off and PoE Power Consumption:0 Watts in global configure page on web. |
| ES4510MA-PD-FLF-EC-00047 | PoE:It doesn't have description about power allocation range on web. |
| ES4510MA-PD-FLF-EC-00046 | PoE:Power Consumption column always displays "0" on web. |
| ES4510MA-PD-FLF-EC-00045 | PoE:Command "show power inline status ethernet 1/10" doesn't provide Power Consumption column information |
| ES4510MA-PD-FLF-EC-00044 | PoE:DUT doesn't provide command to set PoE priority & power allocation on port interface via CLI. |
| ES4510MA-PD-FLF-EC-00041 | PoE:DUT doesn't provide time range command after "power inline ?" in port interface. |
| ES4510MA-PD-FLF-EC-00069 | LACP:DUT can't forward packets correctly via lacp interface, it will flood data to the port which provide power to DUT. |
| ES4510MA-PD-FLF-EC-00043 | PoE:Insert to PSEs to DUT port1,2 , disconnect port1, DUT will power off then reboot automatically. |

Runtime V1.0.1.2 ; Loader V1.0.0.2

| ES4510MA-PD-FLF-EC-00033 | Loader: In bootup time, port 9~12 are in linkup status |
| --- | --- |
| ES4510MA-PD-FLF-EC-00025 | Enable stp bpdu-guard cause system hang out |
| ES4510MA-PD-FLF-EC-00037 | Loader: In booting up, the diag. LED is OFF |
| ES4510MA-PD-FLF-EC-00032 | Diagnostic: Add BI log and power cycle functions |
| ES4510MA-PD-FLF-EC-00021 | MIB file error |

Runtime V1.0.0.2 ; Loader V1.0.0.0

| ES4510MA-PD-FLF-EC-00036 | Enhancement: Add squashfs compress function for runtime image |
| --- | --- |
| ES4510MA-PD-FLF-EC-00015 | Enhancement: add diagnostic functions for ECS4510-12PD |
| ES4510MA-PD-FLF-EC-00030 | Strange System log : PSE Port On/Off :TrapVarBinding |
| ES4510MA-PD-FLF-EC-00029 | Queue Weight and Queue Mode Strict-wrr setting cannot work |
| ES4510MA-PD-FLF-EC-00035 | Diag extlb/wsextlb test fails sometimes after execution of intlb |
| ES4510MA-PD-FLF-EC-00022 | Copy running-config file will cause exception after create port channel |
| ES4510MA-PD-FLF-EC-00023 | Set ip igmp max-groups will cause CLI internal error after create port channel |
| ES4510MA-PD-FLF-EC-00024 | Expire time will keep looping under the IGMPSNP group table |
| ES4510MA-PD-FLF-EC-00026 | Diag LED .OFF After Booting diagnostic finish |
| ES4510MA-PD-FLF-EC-00020 | The command "copy tftp to startup-config" will rewrite the file of tftp server. |

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

- Ingress TCP Rate limit: If you need accurate ratio of ingress TCP rate limit, you can use ACL rule plus DiffSev way to manage the bandwidth of input.

- The jumbo frame size is 10240 which is different with manual and html help file. User can not change the size. (ES4510MA-PD-FLF-EC-00179)

- Vlan trunking can be supported on all ports. The manual is incorrect on “VLAN trunking can only be enabled on Gigabit Ethernet port or trunks”

- VLAN mirroring or MAC-based mirroring can not coexist with Port mirroring. While VLAN mirroring and MAC-based mirroring can be coexisted, but they must use the same target port.

- MAC Chip limit: unicast, multicast, broadcast and storm control all use one same threshold, although each one can enable/disable independent. If user need to use different threshold for each one, he must use rule (diffserv) to do it.

- PHY chip limit: Cable Test on Power Saving is highly inaccurate. If you want to get accurate cable test, please turn off power saving mode first.

- PHY chip limit: you must do cable diagnostic in linkup, or link-down without partner PD port 1~8 should disable power-source-check before test cable diagnostic PSE port 10 only can do cable diagnostic in linkup Fiber port 11~12 not support cable diagnostic.

- MAC chip limit: There is no HW loopback detection without STP (loop-free detection)

- MAC chip limit: Can not support multiple uplink ports on one session

- IGMPSNPv3 ASM: we only recognize IGMP V3 report without src_ip_list

- No mac-learning command for CLI and webUI, the manual is incorrect.

- We only support browser IE 7 and above version