Access Switch ECS4660-28F

Phase I Firmware Version 1.2.0.0

INTRODUCTION:

The ECS46600-28F is a cost effective managed Fast Ethernet Layer 3 access Switch. Each ECS46600-28F unit contains 24 10/100 /100 Based-X/100 Based-T ports with two expansion slots and two combo ports. The slots can support expansion modules such as 1000Base-T, 1000Base-SX, 1000Base-LX and 100FX modules.

The base hardware is a 1.5U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common Http browsers such as Netscape Navigator (shall be Version 6.2 or above) or Microsoft’s Internet Explorer (should be Version 5.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the ECS46600-28F from a RJ45 type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out of band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release note prior to the installation or upgrade of this product.

Firmware Specification:

Runtime

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Patch release | 1.2.0.0 | Customer | 7/3/2012 |

Daig

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| NTC release | 1.0.0.1 | NTC | 1/6/2012 |

Loader

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| NTC release | 1.3.2.3 | NTC | 8/19/2011 |

LOADER CODE modifications:

###### V1.3.2.3

1. If it uses too long filename in TFTP image download in Loader environment, command "dir" would work abnormally.

2. When dual firmware master is failed, boot from backup firmware

###### V1.3.2.2

1. fix SNMPv3When DUT warm start(reload),DUT send cold start

###### V1.3.2.1

1. Initial version

DIAGNOSTIC CODE modifications:

###### V0.0.0

1. Initial version

###### V1.0.0.0

1. Formal release

Runtime  CODE modifications:

V1.2.0.0

| ASF4628BBS5-FLF-EC-00244 | SSH: Login via SSH will be failed and CLI will prompt error message sometimes. |
| --- | --- |
| ASF4628BBS5-FLF-EC-00387 | sometimes VIF will disappear when boot up |
| ASF4628BBS5-FLF-EC-00399 | ipv4 ospf/rip will be flooded so that routing table can't be established. |
| ASF4628BBS5-FLF-EC-00400 | ACL: Switch crash when create an ACE with ethertype and time-range |

V1.0.3.1

| ASF4628BBS5-FLF-EC-00289 | VLAN:Delete vlan 2-4094 via CLI & Web will display error message. |
| --- | --- |
| ASF4628BBS5-FLF-EC-00302 | RSPAN:Get object:rspanStatus value via mib, it display "0", but object value description only (1)valid (2)invalid. |
| ASF4628BBS5-FLF-EC-00305 | PIMSM6:It didn't forward multicast traffic correctly when different group belong to different RP. |
| ASF4628BBS5-FLF-EC-00310 | PIMSM6:It will display exception and receiver didn't receive multicast traffic when changing spanning-tree priority. |
| ASF4628BBS5-FLF-EC-00311 | PIMSM6:It always use shortest path when setting ipv6 pim spt-threshold infinity |
| ASF4628BBS5-FLF-EC-00327 | IPv6 SourceGurad:DUT allows to set invalid ipv6 address to ipv6 source guard entry. |
| ASF4628BBS5-FLF-EC-00329 | BGP CLI:It should display correct error message when removing one standard extcommunity-list. |
| ASF4628BBS5-FLF-EC-00330 | BGP CLI:It did not succeed to set max value when setting route-map match. |
| ASF4628BBS5-FLF-EC-00331 | BGP CLI:It didn't succeed to set max value of weight(metric and local-preference) when settting route-map set. |
| ASF4628BBS5-FLF-EC-00332 | BGP CLI:It should display a specfic prefix-list when specify the name. |
| ASF4628BBS5-FLF-EC-00334 | VRRP:DUT will send out vrrp priority=0 when DUT change lower vrrp priority and DUT status is master. |
| ASF4628BBS5-FLF-EC-00335 | MAC: Default MAC-learning is disabled. |
| ASF4628BBS5-FLF-EC-00337 | IPv6 SourceGuard:DUT doesn't provide interface "port-channel" item when add static ipv6 source guard entry, but spec. does. |
| ASF4628BBS5-FLF-EC-00338 | BGP CLI: It will disappear in show ip bgp neighbors after setting "no neighbor peer-group ", but it display in show running. |
| ASF4628BBS5-FLF-EC-00339 | BGP CLI:BGP timers should match with setting or it shall display error message when DUT calculate hold time=3*keepalive time |
| ASF4628BBS5-FLF-EC-00340 | BGP CLI:The bgp send-community is both mode when setting send-community extended. |
| ASF4628BBS5-FLF-EC-00342 | SNMP:DUT can not set SyncE function via SNMP. |
| ASF4628BBS5-FLF-EC-00343 | BGP CLI:It should display correct maximum-prefix entry. |
| ASF4628BBS5-FLF-EC-00344 | BGP CLI:It should not display in show running-config when using "no" neighbor maximum-prefix. |
| ASF4628BBS5-FLF-EC-00345 | BGP CLI:It didn't remove ip as-path access-list. |
| ASF4628BBS5-FLF-EC-00346 | IPv6 SourceGuard:DUT allows user to set "not unicast" mac address to ipv6 source guard entry. |
| ASF4628BBS5-FLF-EC-00347 | BGP CLI:It did not succeed to set max value when setting bgp confederation peer. |
| ASF4628BBS5-FLF-EC-00348 | BGP CLI:It should describe exclude 1 and 2 about bgp hold time range. |
| ASF4628BBS5-FLF-EC-00349 | BGP CLI:It should fiter some specific subnet when setting aggregate-address |
| ASF4628BBS5-FLF-EC-00350 | BGP CLI:It should dislpay correct error message when removing different types of ip community-list. |
| ASF4628BBS5-FLF-EC-00353 | IPv6 SourceGuard:Add static ipv6 source guard entries then save and reboot, configuration will be lost. |
| ASF4628BBS5-FLF-EC-00355 | IPv6 SourceGuard:Add a static ipv6 SG entry on specific port, then add this port to trunk member,it doesn't display correctly |
| ASF4628BBS5-FLF-EC-00356 | Vlan-Translation:We set the vlan-translation of port channel on CLI ,but it does not display on web. |
| ASF4628BBS5-FLF-EC-00358 | IPv6 SourceGuard:Add 5 static ipv6 SG entreis on port1,then change port1's max-binding to 3,it will show wrong information. |
| ASF4628BBS5-FLF-EC-00362 | RSPAN:When command "Console(config-vlan)#vlan 2-10 m e rspan" ,DUT will crash. |
| ASF4628BBS5-FLF-EC-00364 | DHCPv6 Snooping:On SNMP,it does not support DHCPv6 snooping. |
| ASF4628BBS5-FLF-EC-00366 | BGP CLI:It should display all route-map emtries when creating multi-route-map. |
| ASF4628BBS5-FLF-EC-00367 | BGP:It should not filter routes that match this peer when setting to match peer in route-map rule. |
| ASF4628BBS5-FLF-EC-00370 | SyncE: if remove non-syncE Port. DUT will display debug message. |
| ASF4628BBS5-FLF-EC-00372 | DHCPv6 Snooping:When client restart to get ipv6 address with server, DUT will appear debug message. |
| ASF4628BBS5-FLF-EC-00373 | BGP CLI:DUT will display error message when setting some bgp cmd and then reboot after saving configuration. |
| ASF4628BBS5-FLF-EC-00374 | BGP CLI:It should display "peer local", not "peer" when setting to match peer local. |
| ASF4628BBS5-FLF-EC-00375 | BGP CLI:BGP pathlimit range should not be 1~65535. |
| ASF4628BBS5-FLF-EC-00376 | CLI:Time format of mroute stat expire should be 00h:00m:00s, not 00d:00h when starting to send multicast traffic. |

V1.0.3.0

1. Support RA guard

2. Support DHCPv6 snooping

3. Support IPv6 Tunnel (manual/6 to 4 tunnel)

4. Support BGP/BGP4+4.

5. Support IPv6 Source Guard

6. Support Sync-E

EPR fixed list

| ASF4628BBS5-FLF-EC-00114 | Web UI: OSPF information can not display non area 0 entry in default table. |
| --- | --- |
| ASF4628BBS5-FLF-EC-00163 | Web:It should add one page for the information of show ip(v6) mroute and one page for enable/disable ip(v6) multicast-routing |
| ASF4628BBS5-FLF-EC-00205 | IPv6 address config: After setting invalid ipv6 address, CLI can describe this address is invalid,but web can't do it. |
| ASF4628BBS5-FLF-EC-00226 | Init: DUT prompt "SYSDRV_FAN_CHIP_Init fail" during the reboot. (Hard to reproduce.) |
| ASF4628BBS5-FLF-EC-00234 | Web UI: Web UI can accept IPv4 address and IPv6 next hop at the same time. |
| ASF4628BBS5-FLF-EC-00261 | Reload Delay: The valid range of the minutes in the "Reset Mode: In" is inconsistent on CLI and Web |
| ASF4628BBS5-FLF-EC-00285 | Enhancement:Max value of OSPF dead interval should match 4*hello interval. |
| ASF4628BBS5-FLF-EC-00291 | ERPSv2:Mib doesn't provide ERPSv2 function. |
| ASF4628BBS5-FLF-EC-00292 | RSPAN:RSPAN VLAN shall not be VLAN 4093, which is used for cluster VLAN in the switch. |
| ASF4628BBS5-FLF-EC-00294 | RSPAN Web:It didn't configure about RSPAN by web and didn't display any description of RSPAN from help page. |
| ASF4628BBS5-FLF-EC-00295 | PIMSM6:The BSR priority and hash mask length will reverse when changing ipv6 address or reboot. |
| ASF4628BBS5-FLF-EC-00296 | Show Debug:Whether the status of port link up or down, it display will the same(LACP: PortLinkup_Service on....). |
| ASF4628BBS5-FLF-EC-00300 | Vlan-Translation:When we enable VLAN Translation on a port channel group can appear error message. |
| ASF4628BBS5-FLF-EC-00301 | Show Debug:Dynamic trunk is create and enable debug lacp packet and detail,but DUT can not display any debug message. |
| ASF4628BBS5-FLF-EC-00303 | PIMSM6:It didn't display register interface when showing the cmd "show ipv6 mroute". |
| ASF4628BBS5-FLF-EC-00304 | PIMSM6:It display same priority when setting same rp-candidate interface vlan, different group-prefix and different priority. |
| ASF4628BBS5-FLF-EC-00305 | PIMSM6:It didn't forward multicast traffic correctly when different group belong to different RP. |
| ASF4628BBS5-FLF-EC-00306 | Vlan-Translation:Web UI does not support vlan-translation. |
| ASF4628BBS5-FLF-EC-00309 | PIMSM6:Receiver will more receive 20 packets per second. |
| ASF4628BBS5-FLF-EC-00310 | PIMSM6:It will display exception and receiver didn't receive multicast traffic when changing spanning-tree priority. |
| ASF4628BBS5-FLF-EC-00311 | PIMSM6:It always use shortest path when setting ipv6 pim spt-threshold infinity |
| ASF4628BBS5-FLF-EC-00312 | PIMSM6:It should display which outgoing interface is prune state when sending multicast traffic to downstream. |
| ASF4628BBS5-FLF-EC-00313 | Show Debug:Debug information display between "Show" and "Configure" is not match. |
| ASF4628BBS5-FLF-EC-00314 | RSPAN:The monitored traffic should be received correctly when RSPAN destination port is set tag or untag. |
| ASF4628BBS5-FLF-EC-00315 | VRRP Web:It will display "Data is invalid" and didn't display in web when setting vrrp ip address(But it display in CLI) |
| ASF4628BBS5-FLF-EC-00316 | VRRP:DUT didn't send out vrrp packets when it is master but it could check received vrrp packets. |
| ASF4628BBS5-FLF-EC-00317 | OSPFv3:It will display the exception when disable ospfv3. |
| ASF4628BBS5-FLF-EC-00319 | Web:It should add one page to set PIMSM6 parameters. |
| ASF4628BBS5-FLF-EC-00320 | Web:It can set about PIMSM in two places.General page should be incorrect if it follow help page. |
| ASF4628BBS5-FLF-EC-00322 | PIMSM6:It didn't limit register packets number when setting pim register-rate-limit |

V1.0.1.0

1. Support RSPAN

2. Support VLAN translation

3. Support PIM/SM6

4. Support ERPS v2

5. Support show debug(XSTP/LACP)

EPR fixed list

V1.0.0.1

| ASF4628BBS5-FLF-EC-00021 | the HTTPS certificate is not correct for this project |
| --- | --- |
| ASF4628BBS5-FLF-EC-00062 | correction to cli prompt message - radius server 1 host 1.1.1.1 ? |
| ASF4628BBS5-FLF-EC-00069 | corrections to formatting - transceiver-threshold |
| ASF4628BBS5-FLF-EC-00177 | sFlow: sFlow IPv6 agent address is incorrect in the captured sFlow packets. |
| ASF4628BBS5-FLF-EC-00223 | Help Page :Metric of OSPF NSSA Area value is incorrect. |
| ASF4628BBS5-FLF-EC-00224 | CLI: The first word of parameter information for #sh access-group ? is not a capital letter. |
| ASF4628BBS5-FLF-EC-00225 | Storm Control: The default rates of unknown unicast and multicast traffic are inconsistent with the Web help page |
| ASF4628BBS5-FLF-EC-00229 | DHCP: DUT can not get IP address via DHCP that carries router option (option 3) |
| ASF4628BBS5-FLF-EC-00231 | Web: IGMP query interval alarm box information is incorrect. (Query Interval can't equal to Max Response Time) |
| ASF4628BBS5-FLF-EC-00232 | Web: IGMP query interval and Max response time value configuration on Web Management behavior is incorrect. (CLI is correct) |
| ASF4628BBS5-FLF-EC-00233 | RIP: Receive Version of RIP is different between Web UI and Help page. |
| ASF4628BBS5-FLF-EC-00236 | Cos: Queue Mode default setting is different between Web UI and Help page. |
| ASF4628BBS5-FLF-EC-00237 | Web: OSPF show page will disappear after remove one of the processes. |
| ASF4628BBS5-FLF-EC-00240 | Web: Help page Trunk Range on VLAN > Static, Action: Edit Member by Interface part is incorrect. |
| ASF4628BBS5-FLF-EC-00243 | Web: Default value for Unknown Unicase Storm control on help page is mismatch what in reality. |
| ASF4628BBS5-FLF-EC-00244 | PIM:if down stream is local. DUT Vlan status will be forwarding. without sending join report. |
| ASF4628BBS5-FLF-EC-00248 | ERPS: ERPS domain name can't be spaces on CLI, but can be spaces on Web. |
| ASF4628BBS5-FLF-EC-00250 | MAC VLAN: Can't specify vlan 4094 and vlan 4093 with mac address by mib. |
| ASF4628BBS5-FLF-EC-00252 | PIM:DUT can be set non-exist vlan as register source via CLI. |
| ASF4628BBS5-FLF-EC-00253 | Web: VLAN ID range for creating Private VLAN on Help page is mismatch what on Management page. |
| ASF4628BBS5-FLF-EC-00264 | DHCP: DUT can not get IP address via DHCP that carries router option (option 3) |
| ASF4628BBS5-FLF-EC-00265 | Customerinfo:Object Lable =ecs4660-28fMIB (Object ID = 1.3.6.1.4.1.259.10.1.10) description is not correct. |
| ASF4628BBS5-FLF-EC-00266 | Multicast > Multicast Routing > General - missing web pages |

###### V1.0.0.0

###### V.0.9.7.4

| ASF4628BBS5-FLF-EC-00021; the HTTPS certificate is not correct for this project. ASF4628BBS5-FLF-EC-00036; missing "show banner" command. ASF4628BBS5-FLF-EC-00059; SFP ports 1-26 -> non-combo ports should use default media type "sfp-forced". ASF4628BBS5-FLF-EC-00063; corrections to formatting - show network-access. ASF4628BBS5-FLF-EC-00064; correction to grammar - show ip dhcp snooping. ASF4628BBS5-FLF-EC-00065; corrections to grammar/formatting - show ip source-guard binding. ASF4628BBS5-FLF-EC-00066; corrections to grammar - show ip arp inspection configuration. ASF4628BBS5-FLF-EC-00068; corrections to grammar - show dos-protection. ASF4628BBS5-FLF-EC-00070; corrections to grammar - show class map. ASF4628BBS5-FLF-EC-00108; OSPF: Router ID setting and help page help infomation is not the same .. ASF4628BBS5-FLF-EC-00126; UDP Helper: UDP helper address can be configured with 0.0.0.0 in Web UI. ASF4628BBS5-FLF-EC-00147; OSPFv3 : DUT can not forward bi-directional packet from the virtual link connection between two backbone areas.. ASF4628BBS5-FLF-EC-00158; Web UI: Network can be setted by IP address but can't by VLAN when the RIP is disabled globally.. ASF4628BBS5-FLF-EC-00162; Web UI :Metric of OSPF NSSA Area value is incorrect.. ASF4628BBS5-FLF-EC-00180; Route table:CLI cannot type keyboard "A" to display All table list.. ASF4628BBS5-FLF-EC-00182; DHCP: The host pool can be configured with IP address which is not belong to any existed network pool. ASF4628BBS5-FLF-EC-00183; Web UI: When join max OSPF entry. OSPF information format is incorrect.. ASF4628BBS5-FLF-EC-00186; TACACS: "admin" can not get correct privilege when DUT enable TACACS authorization. ASF4628BBS5-FLF-EC-00189; PortSecurity: The DUT did not delete secured MAC address correctly when the action of Port Security was "Shutdown" state.. ASF4628BBS5-FLF-EC-00190; IP Subnet-vlan: The subnet IP of DUT is mapped to other vlan by web, the web won't display correctly.. ASF4628BBS5-FLF-EC-00194; IP Subnet-vlan: If IP subnet vlan is 192.168.x.0 , it can't be set by web with the mask is 255.255.0.0.. ASF4628BBS5-FLF-EC-00208; Web: After inputting passwords in input field and clicking "Login", the the number of dots in input field will be increased.. ASF4628BBS5-FLF-EC-00212; Storm Control: The range of rate is incorrect in the Web help page. ASF4628BBS5-FLF-EC-00213; Voice VLAN: Remaining age with number is not in alignment with NA or Not Start.. ASF4628BBS5-FLF-EC-00214; ACL: Description for the parameters with #sh access-group ? shall not include egress part. (DUT not support Egress binding.). ASF4628BBS5-FLF-EC-00215; Voice VLAN: Telephony OUI example on help page is incorrect. Telephony OUI has rules need to follow.. ASF4628BBS5-FLF-EC-00216; Web: Voice VLAN OUI MAC address Mask has default value on Help Page, (It shall be removed.). ASF4628BBS5-FLF-EC-00218; CLI: Parameter ? information is not correct on Console(config-if)#port security action trap?.. ASF4628BBS5-FLF-EC-00219; CLI: Network-access link-detection action description is incorrect on link-up and link-up-down part.. ASF4628BBS5-FLF-EC-00220; Web: Clear ACL hardware counter button shall be removed from the Web page.. ASF4628BBS5-FLF-EC-00221; CPU Utilization: Run some configuration for a long time, CPU will keep being 100% and not decreased any more.. |  |
| --- | --- |

###### V0.9.7.3

| ASF4628BBS5-FLF-EC-00052 | PortTunk : Fiber can't support 10F 10H 100F 100H |
| --- | --- |
| ASF4628BBS5-FLF-EC-00059 | SFP ports 1-26 -> non-combo ports should use default media type "sfp-forced" |
| ASF4628BBS5-FLF-EC-00060 | LLDP Local Device Information should not display Router capability |
| ASF4628BBS5-FLF-EC-00061 | correction to formatting - show s? |
| ASF4628BBS5-FLF-EC-00073 | MAC access-list permit untagged-802.3 any any - fails |
| ASF4628BBS5-FLF-EC-00096 | Help:Help page description about "igmp last member query interval" range is not correct. |
| ASF4628BBS5-FLF-EC-00099 | Static Route: When DUT set managment network as next hop via Web UI. Web UI will display error message and accept it setting |
| ASF4628BBS5-FLF-EC-00100 | Help:DUT allows to create up to 255 static arp entries, but help description is 128. |
| ASF4628BBS5-FLF-EC-00101 | MLD: DUT can not displayed the entry of the source address when operator specified the VLAN interface |
| ASF4628BBS5-FLF-EC-00123 | IP-PIM:It should display who is DR in show ip pim neighbor |
| ASF4628BBS5-FLF-EC-00128 | The Tx Frames and Rx Frams was not same. the packet was lost. |
| ASF4628BBS5-FLF-EC-00131 | RIP:It display "RIP is not enabled on this interface" on show rip interface when disabling secondary address |
| ASF4628BBS5-FLF-EC-00132 | RIP:It should display all ip address on show ip rip interface |
| ASF4628BBS5-FLF-EC-00134 | RIP:DUT should display error or warining message when it attempt to establish a subnet network with host ip address. |
| ASF4628BBS5-FLF-EC-00136 | RIP:The response message from secondary address should include primary subnet when enabling split horizon |
| ASF4628BBS5-FLF-EC-00146 | PIMSM:DUT will display exception when running igmp proxy and pimsm |
| ASF4628BBS5-FLF-EC-00167 | Web UI:VLAN range of help page for PIM should be 1~4094. |
| ASF4628BBS5-FLF-EC-00168 | PIMSM:It should npt allow to change PIMSM parameter(dr-priority and join-prune-interval) when running PIM DM mode |
| ASF4628BBS5-FLF-EC-00171 | PIM MIB: PIMRPSetComponent and PIMNeighborMode didn't get the value by mib browser. |
| ASF4628BBS5-FLF-EC-00172 | build envir: can't rebuild processs automatically when dep libs had been updated |
| ASF4628BBS5-FLF-EC-00173 | Help Page: Port status is incorrect. |
| ASF4628BBS5-FLF-EC-00174 | DHCP: There is two more "Netbios name server:" field, but no "Next Server:" field in the "show ip dhcp pool" command |
| ASF4628BBS5-FLF-EC-00176 | sFlow: Socket Port of sFlow interface description is incorrect. |
| ASF4628BBS5-FLF-EC-00178 | Link-Detection:Enable network access link-detection on port, then disconnect the port will display error msg on CLI. |
| ASF4628BBS5-FLF-EC-00179 | NetworkNotice:Configure valid logging host, then show logging trap , it displays invalid logging host. |
| ASF4628BBS5-FLF-EC-00180 | Route table:CLI cannot type keyboard "A" to display All table list. |
| ASF4628BBS5-FLF-EC-00181 | DHCP: There is one more "Subnet mask:" field when operator used "show ip dhcp pool" |
| ASF4628BBS5-FLF-EC-00183 | Web UI: When join max OSPF entry. OSPF information format is incorrect. |
| ASF4628BBS5-FLF-EC-00184 | L2switch: The auto negotiation can't be disabled on DUT port. |
| ASF4628BBS5-FLF-EC-00185 | Voice vlan:The defaule value of port mode between CLI and WEB are different. |
| ASF4628BBS5-FLF-EC-00187 | TACACS: The interface names of "Accounting Type" and "Authorization Type" are inconsystent between CLI and Web |
| ASF4628BBS5-FLF-EC-00188 | L2switch: Flow control can't be enabled on DUT port, and DUT can't work flow control. |
| ASF4628BBS5-FLF-EC-00189 | PortSecurity: The DUT did not delete secured MAC address correctly when the action of Port Security was "Shutdown" state. |
| ASF4628BBS5-FLF-EC-00191 | DHCP: There is no Web help page of IP Service -> DHCP -> Client |
| ASF4628BBS5-FLF-EC-00192 | EFM OAM: There is no Web help page of Administration -> OAM -> All Pages |
| ASF4628BBS5-FLF-EC-00193 | Voice vlan:when multiple neighbors send lldp packets at the same time,aging time will incorrect. |
| ASF4628BBS5-FLF-EC-00195 | L2 switch: If client sends too many unicast packets to another client with line speed, DUT will drop a little packets. |
| ASF4628BBS5-FLF-EC-00196 | ATC: ATC can not be configured by web. |
| ASF4628BBS5-FLF-EC-00198 | MAC-BasedVlan: The help page describes that MAC format must be xx-xx-xx-xx-xx-xx,but the format xxxxxxxxxxxx can also be set. |
| ASF4628BBS5-FLF-EC-00199 | MAC Auth: The type of authenticated mac should be "delete on timeout" rahter than "delete on reset" in mac learning table. |
| ASF4628BBS5-FLF-EC-00200 | MAC-Based vlan: The range of vlan id which the MAC address associates with is incorrect in mib. |
| ASF4628BBS5-FLF-EC-00201 | Storm: The help message of unknown-unicast is incorrect. |
| ASF4628BBS5-FLF-EC-00202 | Storm: Only broadcast is descripted in the The Traffic -> Storm Control help page |
| ASF4628BBS5-FLF-EC-00203 | Storm: "Unknown Unicast Threshold" do not work properly |
| ASF4628BBS5-FLF-EC-00204 | ATC: DUT will delay 2~3 seconds to send ATC trap packet after input rate limit is larger than fire threshold. |
| ASF4628BBS5-FLF-EC-00206 | LLDP-MED: There is no command or option to enable LLDP-MED Function via CLI or Web |

###### V0.9.7,2

ASF4628BBS5-FLF-EC-00038; show ipv6 interface - corrections to spelling/grammar. ASF4628BBS5-FLF-EC-00039; show ipv6 traffic - corrections to spelling/grammar. ASF4628BBS5-FLF-EC-00041; Administration > LLDP (Configure Interface) - correction to spelling/grammar. ASF4628BBS5-FLF-EC-00054; Administration > LLDP (Configure Interface) - correction to spelling/grammar. ASF4628BBS5-FLF-EC-00059; SFP ports 1-26 -> non-combo ports should use default media type "sfp-forced". ASF4628BBS5-FLF-EC-00072; Administration > SNMP (Configure Global) - move statistics to Show Statistics page. ASF4628BBS5-FLF-EC-00080; ARP:It doesn't provide item to create static arp entry via web and mib browser.. ASF4628BBS5-FLF-EC-00081; ARP:Configure static arp entry , show running will display "ipv6 neighbor 192.168.1.1 vlan 1 00-00-e8-00-00-01". ASF4628BBS5-FLF-EC-00084; ARP: Create vlan index to enable/disable proxy arp via mib browser fail.. ASF4628BBS5-FLF-EC-00085; OSPF: DUT can set invalid area ID.. ASF4628BBS5-FLF-EC-00086; MLD: The valid range of max-resp-interval in CLI is different from CLI help. ASF4628BBS5-FLF-EC-00087; MLD: "last-memb-query-resp-interval" of MLD shall be changed to "last-memb-query-interval". ASF4628BBS5-FLF-EC-00088; IGMPProxy:Enable igmp proxy on vlan1 on web&mib, it will enable igmp on vlan 1 on web&mib automatically.. ASF4628BBS5-FLF-EC-00089; MLD: Static group address can be configured with link-local address and global unicast address. ASF4628BBS5-FLF-EC-00090; MLD: Static group address can be configured with ::. ASF4628BBS5-FLF-EC-00091; Ping: "IP -->General --> Ping" page is empty.. ASF4628BBS5-FLF-EC-00092; ARPProxy:No "mac-address xxxx.xxxx.xxxx" command on vlan interface.. ASF4628BBS5-FLF-EC-00094; Reboot:When DUT reboot. Reboot process will display fail message.. ASF4628BBS5-FLF-EC-00095; MLD: Specified source address of static multicast group can not be displayed correctly. ASF4628BBS5-FLF-EC-00098; IGMP:Send igmp join report or only send multicast data somtimes will cause DUT display error message.. ASF4628BBS5-FLF-EC-00101; MLD: DUT can not displayed the entry of the source address when operator specified the VLAN interface. ASF4628BBS5-FLF-EC-00102; ARP:Can't remove static arp entries via CLI/web/mib browser.. ASF4628BBS5-FLF-EC-00103; ARP:Learning dynamic arp entries and create static arp entries will cause DUT crash.. ASF4628BBS5-FLF-EC-00104; MLD: Static MLD group membership will be removed after "clear ipv6 mld group". ASF4628BBS5-FLF-EC-00105; MLD: FWD status of source address is wrong when the mode is changed from include to exclude. ASF4628BBS5-FLF-EC-00106; MLD: DUT hang up when it processes MLDv2 control packets. ASF4628BBS5-FLF-EC-00107; ARP:Dynamic ARP entry can't follow "arp cache time out" parameter which user configured to work.. ASF4628BBS5-FLF-EC-00109; Light: LED will not light up. when the port is link up.. ASF4628BBS5-FLF-EC-00110; OSPF: If area set as Stub area. Routing table have not default route entry .. ASF4628BBS5-FLF-EC-00111; OSPF: NSSA of OSPF information format is incorrect.. ASF4628BBS5-FLF-EC-00112; MLD: Maximum response code is incorrect in MLDv2 query. ASF4628BBS5-FLF-EC-00113; VRRP:Can't configure vrrp priority to 254 via CLI.. ASF4628BBS5-FLF-EC-00117; IP-PIM:When "no router pim" then "router pim" again, multicast packet can't forward correctly unless reboot the DUT.. ASF4628BBS5-FLF-EC-00118; MLD: DUT hang up when process the MLDv2 report with different groups take multi-source addresses. ASF4628BBS5-FLF-EC-00119; IP-PIM6:When "no router pim6" then "router pim6" again, multicast packet can't forward correctly unless reboot the DUT.. ASF4628BBS5-FLF-EC-00120; IP-PIM:Sometimes It will display error message when no ip multicast-routing. ASF4628BBS5-FLF-EC-00121; IP-PIM:The another directly-connected route should be up when send multicast traffic and shutdown root port of spanning-tree. ASF4628BBS5-FLF-EC-00122; MLD:DUT didn't learn all group quickly.. ASF4628BBS5-FLF-EC-00123; IP-PIM:It should display who is DR in show ip pim neighbor. ASF4628BBS5-FLF-EC-00124; IGMPv3:Maximum igmp group per vlan only can be 512, it should be 1024.. ASF4628BBS5-FLF-EC-00125; UDP Helper: UDP helper address can be configured with subnet address, boradcast address, and multicast address. ASF4628BBS5-FLF-EC-00127; OSPFV3: OSPF v3 format is incorrect.. ASF4628BBS5-FLF-EC-00129; OSPFv3:When change OSPF Priority. DR and BDR elect sometime is incorrect.. ASF4628BBS5-FLF-EC-00133; OSPF:Area range to summary route is not work.. ASF4628BBS5-FLF-EC-00138; OSPFv3: OSPFv3 can set invalid Router-ID.. ASF4628BBS5-FLF-EC-00139; IPPIM:It should display a specific group with the source when showing ip mroute x.x.x.x x.x.x.x (summary).. ASF4628BBS5-FLF-EC-00140; PIMSM:It didn't remove the priority when using the cmd "no ip pim rp-candidate interface vlan 3". ASF4628BBS5-FLF-EC-00141; PIMSM:It will display default group-prefix after setting pim rp-candidate with a specific group and priority,then reboot.. ASF4628BBS5-FLF-EC-00142; PIMSM:It should not display previous group-prefix when setting new group-pfrefix. ASF4628BBS5-FLF-EC-00143; DHCP Server: There is no command can detail the configuration of DHCP pool. ASF4628BBS5-FLF-EC-00144; DHCP Server: The active pool of DHCP pool entry is incorrect. ASF4628BBS5-FLF-EC-00145; DHCP Server: DHCP Server can not be enabled properly, but the DHCP pool still be active. ASF4628BBS5-FLF-EC-00146; PIMSM:DUT will display exception when running igmp proxy and pimsm. ASF4628BBS5-FLF-EC-00148; VRRP:Can't set new vrrp virtual ip directly via CLI.. ASF4628BBS5-FLF-EC-00149; OSPFv3: DUT can not redistribute static route.. ASF4628BBS5-FLF-EC-00150; PIMSM:DUT will not forward multicast traffic when enabling igmp snp. ASF4628BBS5-FLF-EC-00151; IPPIM:DUT will not send trap with correct information when PIM neighbor loss.. ASF4628BBS5-FLF-EC-00152; DHCP Server: DUT do not send DHCPNAK when received invalid DHCPREQUEST (INIT-Reboot state). ASF4628BBS5-FLF-EC-00154; DHCP Server: DUT do not update DHCP database for Bootp protocol. ASF4628BBS5-FLF-EC-00155; DHCP Server: There are only 5 DHCP excluded pools can be configured on DUT. ASF4628BBS5-FLF-EC-00156; StaticRoute:Configure static route entry via CLI, then show ip route can't see this entry.. ASF4628BBS5-FLF-EC-00157; OSPF: MIB have IPv6 address AddressType in OSPFv2.. ASF4628BBS5-FLF-EC-00159; OSPFv3: When Route entry arrive Max age time(3600). DUT does not flooding multicast LSA with age set to 3600 seconds.. ASF4628BBS5-FLF-EC-00161; DHCP Server: DUT can not refer the client identifier to respond DHCP Offer. ASF4628BBS5-FLF-EC-00166; PIMSM:The uptime will display correctly when enable pimsm bsr-router(or rp-candidate) and run over 1 day. ASF4628BBS5-FLF-EC-00169; PIMSM:It show the same entry continuously on running-config when setting rp-address or rp-candidate with group mask 0.0.0.0.

###### V 0.9.7.1..

ASF4628BBS5-FLF-EC-00083; ARP:ARP page's vlan list only has vlan1 to let user enable/disable proxy arp. It can't display all exist vlans in vlan list.. ASF4628BBS5-FLF-EC-00077; IGMP:It doesn't provide IGMP function on web.. ASF4628BBS5-FLF-EC-00080; ARP:It doesn't provide item to create static arp entry via web and mib browser.. ASF4628BBS5-FLF-EC-00082; MIB: Use Eliteviewer to compile private mib file "ECS4660-28F.mib" will display "parse error".. ASF4628BBS5-FLF-EC-00076; VRRP:It doesn't provide VRRP function on web.. ASF4628BBS5-FLF-EC-00037; show lldp config - correction to spelling/grammar. ASF4628BBS5-FLF-EC-00040; show spanning-tree -> missing Fast Forwarding. ASF4628BBS5-FLF-EC-00060; LLDP Local Device Information should not display Router capability. ASF4628BBS5-FLF-EC-00067; corrections to grammar/formatting - show ip arp inspection interface. ASF4628BBS5-FLF-EC-00071; corrections to grammar - show policy map.

###### V 0.9.7.0..

1. Initial version

2. Support OSPF,RIP v1/v2,static route

3. support OSPFv3

4. support IGMP v1/v2/v3/IGMP proxy

5. support MLD

6. support PIM-SM, PIM-DM,IPv6 PIM-DM

7. support VRRP

8. support ARP/Proxy ARP

9. support UDP helper

10. support DHCP server

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Installation and Configuration Notes:

In general, the ECS46600-28F Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS46600-28F Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

###### V 0.9.3.0

1. Support IEEE 1588

###### V 0.9.1.0

1. Initial version

Known Restrictions and Limitations:

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2006 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ac | 1000Base-Tx |
| IEEE 802.3ad | Link Aggregation |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1213 | MIB-II | - system group - interfaces group - ip group - icmp group - tcp group - udp group - snmp group |
| 1493 | Bridge MIB | - dot1dBase group - dot1dStp group - dot1dTp group - dot1dStatic group |
| 2863 | Interfaces Evolution MIB | - ifXTable group - ifStackTable group |
| 2819 | RMON MIB | - statistics group - history group - alarm group - event group |
| 2618 | RADIUS MIB | - radiusAuthClientMIB |
| 2665 | Etherlike MIB | - dot3StatsTable group |
| 2737 | Entity MIB | - entityPhysical group |
| 2674 | P-bridge | - dot1dExtBase group - dot1dPriority group - dot1dGarp group |
| 2674 | Q-bridge | - dot1qBase group - dot1qTp group - dot1qStatic group - dot1qVlan |
| IEEE 802.1X | IEEE8021-PAE-MIB | - dot1xPaeSystem group - dot1xPaeAuthenticator group - dot1xPaeSupplicant group |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | coldStart warmStart linkDown linkUp authenticationFailure |
| RFC 1493 | newRoot topologyChange |
| RFC 2819 | risingAlarm fallingAlarm |