Aggregation Switch ECS4660-28F

Phase II Firmware Version 1.2.2.2

INTRODUCTION:

The ECS4660-28F is a cost effective managed carrier grade Layer 3 aggregation Switch. Each ECS4660-28F unit contains 24 GE SFP ports, two XFP ports with two  SFP+ expansion modules.

The base hardware is a 1.5U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common Http browsers such as Netscape Navigator (shall be Version 6.2 or above) or Microsoft’s Internet Explorer (should be Version 5.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the ECS4660-28F from a RJ45 type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out of band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release note prior to the installation or upgrade of this product.

Firmware Specification:

Runtime

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Formal release | 1.2.2.2 | Customer | 10/29/2013 |

Daig

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Formal release | 1.0.1.1 | Customer | 7/10/2012 |

Loader

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Formal release | 1.3.2.4 | Customer | 7/10/2011 |

LOADER CODE modifications:

###### V1.3.2.4

1. support power cycle burn in mode

DIAGNOSTIC CODE modifications:

###### V1.0.1.1

1. support power cycle burn in mode

Runtime  CODE modifications:

###### V1.2.2.2

1. ASF4628BBS5-FLF-EC-00545 ACL will failed to bind on port after repeat enable/disable

2. ASF4628BBS5-FLF-EC-00546 ECS4660-28F can't relay dhcp packets via trunking port .

###### V1.2.2.1

1. Support clear efm oam event-log (no web)

2. Support OAM event log to show link status & dying gasp clear and send critical event

3. Support DAI CLI allows IP all-zero validation

4. Support support IPSG MAC mode & IPSG record for blocking traffic

5. Support support IGMPSNP FORWARD with PRIORITY

6. Support support spanning topology change propagation stop

7. Support 10G SFP+ module

8. Support non-spanning tree loopback detection (no web)

9. Support UDLD (no web)

10. Support IP MAC binding (no web).

11. Support Egress ACL

12. Support ACL counter

13. Support DDM threshold auto/manual configuration

14. Support RTC

15. Support HW watchdog/SW watchdog

16.Support IGMP RADIUS authentication (no web)

17. Support PVST/CDP filter (no web)

18. Support ERPS domain from 2 to 13

19. Support MAC notification

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Installation and Configuration Notes:

In general, the ECS4660-28F Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS4660-28F Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

Known Restrictions and Limitations:

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2013 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ac | 1000Base-Tx |
| IEEE 802.3ad | Link Aggregation |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1213 | MIB-II | - system group - interfaces group - ip group - icmp group - tcp group - udp group - snmp group |
| 1493 | Bridge MIB | - dot1dBase group - dot1dStp group - dot1dTp group - dot1dStatic group |
| 2863 | Interfaces Evolution MIB | - ifXTable group - ifStackTable group |
| 2819 | RMON MIB | - statistics group - history group - alarm group - event group |
| 2618 | RADIUS MIB | - radiusAuthClientMIB |
| 2665 | Etherlike MIB | - dot3StatsTable group |
| 2737 | Entity MIB | - entityPhysical group |
| 2674 | P-bridge | - dot1dExtBase group - dot1dPriority group - dot1dGarp group |
| 2674 | Q-bridge | - dot1qBase group - dot1qTp group - dot1qStatic group - dot1qVlan |
| IEEE 802.1X | IEEE8021-PAE-MIB | - dot1xPaeSystem group - dot1xPaeAuthenticator group - dot1xPaeSupplicant group |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | coldStart warmStart linkDown linkUp authenticationFailure |
| RFC 1493 | newRoot topologyChange |
| RFC 2819 | risingAlarm fallingAlarm |