Aggregation Switch ECS4660-28F

Phase II Firmware Version 1.2.4.3

INTRODUCTION:

The ECS4660-28F is a cost effective managed carrier grade Layer 3 aggregation Switch. Each ECS4660-28F unit contains 24 GE SFP ports, two XFP ports with two  SFP+ expansion modules.

The base hardware is a 1.5U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common Http browsers such as Netscape Navigator (shall be Version 6.2 or above) or Microsoft’s Internet Explorer (should be Version 5.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the ECS4660-28F from a RJ45 type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out of band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release note prior to the installation or upgrade of this product.

Firmware Specification:

Runtime

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Patch release | 1.2.4.3 | Customer | 5/13/2016 |
| Patch release | 1.2.4.2 | Customer | 9/1/2015 |
| Patch release | 1.2.4.1 | Customer | 3/9/2015 |
| Patch release | 1.2.4.0 | Customer | 5/21/2014 |

Daig

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Formal release | 1.0.1.1 | Customer | 7/10/2012 |

Loader

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Formal release | 1.3.2.4 | Customer | 7/10/2011 |

LOADER CODE modifications:

###### V1.3.2.4

1. support power cycle burn in mode

DIAGNOSTIC CODE modifications:

###### V1.0.1.1

1. support power cycle burn in mode

Runtime CODE modifications:

###### V1.2.4.3

| 1 | ASF4628BBS5-FLF-EC-00626 | 100m sfp can't link up properly after switch rebooting |
| --- | --- | --- |
| 2 | ASF4628BBS5-FLF-EC-00627 | [L_MM] platform use hardcode to allocate 8M for tx/rx/mref buffer |
| 3 | ASF4628BBS5-FLF-EC-00628 | SNMP_node_Bug-OSPF |
| 4 | ASF4628BBS5-FLF-EC-00629 | SNMP_node_Bug-BGP |
| 5 | ASF4628BBS5-FLF-EC-00630 | Enhancement of the software watchdog log content. |
| 6 | ASF4628BBS5-FLF-EC-00631 | MIB object definition in typo or bad syntax (ES3528MV2-FLF-EC-00655~668 and more w/o tickets) |
| 7 | ASF4628BBS5-FLF-EC-00632 | The system would probably halt at "Entering transition mode" while iterative reloading by scripts. This issue is particularly reported in test code v16.04.25.18 (v1.2.4.2 + bugfixes) by EC TS. However, not heard in the formal patch code v1.2.4.2 (2015/09/01 released) and other test codes. |

###### V1.2.4.2

| 1 | ASF4628BBS5-FLF-EC-00572 ES4627MB-FLF-EC-00728 | Stacking: The DUT will dump exception stackframe and crash when user execute the "copy start to running" command. |
| --- | --- | --- |
| 2 | ASF4628BBS5-FLF-EC-00573 ES5652BT1-FLF-EC-00329 | RMON: Input snmpset command by net-snmp, the system will shows exception log. |
| 3 | ASF4628BBS5-FLF-EC-00574 ECS3510-28T-00543 | remove spanning tree MSTP mode then switch will hang in boot process |
| 4 | ASF4628BBS5-FLF-EC-00580 ECS4620-28T-00311 | [enhance] display total flash size in CLI 'dir' command for EC products. |
| 5 | ASF4628BBS5-FLF-EC-00586 | destination unreachable packets cause CPU rising |
| 6 | ASF4628BBS5-FLF-EC-00588 | switch didn't response membership query from uplink when IGMP proxy enabled |
| - | ASF4628BBS5-FLF-EC-00589 | switch didn't response general query and membership query from uplink with different ip subnet when IGMP proxy enabled (Nothing to Modify) |
| 7 | ASF4628BBS5-FLF-EC-00590 ECS4210-52P-00806 | IPv4-AddrConfig: During configuration interface vlan process, the DUT shows exception message then crash. |
| 8 | ECS4210-52P-00790 ECS4210-52P-00791 | DHCPServer: The DUT can not allocate the ip address from the configured ip address pool, need to re-enabled the DHCPservice. DHCPServer: The DUT can not reuse the dynamic ip address when dynamic ip addresses have been released manually. |
| 9 | AOS5700-54X-00354 | IPv6SG: If Port set IPv6 SG as S-IP, DUT will hang up and watchdog will activate. |

###### V1.2.4.1

1. ASF4628BBS5-FLF-EC-00566 We can't bind the egress service-policy to the port

2. ASF4628BBS5-FLF-EC-00567 apply IP ACL on port then ARP packet will be dropped

3. ASF4628BBS5-FLF-EC-00568 spanning tree re-election when disconnect one of link

4. ASF4628BBS5-FLF-EC-00569 can't link up with 100M SFP randomly

###### V1.2.4.0

1. ASF4628BBS5-FLF-EC-00551 Show log ram caused exception

2. ASF4628BBS5-FLF-EC-00552 Show log flash caused exception

3. ASF4628BBS5-FLF-EC-00558 support CLI command "show spanning-tree brief"

4. ASF4628BBS5-FLF-EC-00559 support CLI command show running-config interface 1/x

5. ASF4628BBS5-FLF-EC-00560 [enhancement] change ARP entries from 8K to 16K with chip consistency

6. ASF4628BBS5-FLF-EC-00561 [enhancement] support vrrp ping command

###### V1.2.2.3

1. ASF4628BBS5-FLF-EC-00556 support SFP 100FX automatically by default no need to use media-type sfp-forced 100fx

2. ASF4628BBS5-FLF-CN-00071 RMON: After we configure Even with maximum-length and show running configuration; DUT show exception

3. ASF4628BBS5-FLF-EC-00553 inject 4K ARP packets with 20 pkts/sec but the ARP can not learn up to 4K

###### V1.2.2.1

1. Support clear efm oam event-log (no web)

2. Support OAM event log to show link status & dying gasp clear and send critical event

3. Support DAI CLI allows IP all-zero validation

4. Support support IPSG MAC mode & IPSG record for blocking traffic

5. Support support IGMPSNP FORWARD with PRIORITY

6. Support support spanning topology change propagation stop

7. Support 10G SFP+ module

8. Support non-spanning tree loopback detection (no web)

9. Support UDLD (no web)

10. Support IP MAC binding (no web).

11. Support Egress ACL

12. Support ACL counter

13. Support DDM threshold auto/manual configuration

14. Support RTC

15. Support HW watchdog/SW watchdog

16.Support IGMP RADIUS authentication (no web)

17. Support PVST/CDP filter (no web)

18. Support ERPS domain from 2 to 13

19. Support MAC notification

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Installation and Configuration Notes:

In general, the ECS4660-28F Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS4660-28F Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

Known Restrictions and Limitations:

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2013 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ac | 1000Base-Tx |
| IEEE 802.3ad | Link Aggregation |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1213 | MIB-II | - system group - interfaces group - ip group - icmp group - tcp group - udp group - snmp group |
| 1493 | Bridge MIB | - dot1dBase group - dot1dStp group - dot1dTp group - dot1dStatic group |
| 2863 | Interfaces Evolution MIB | - ifXTable group - ifStackTable group |
| 2819 | RMON MIB | - statistics group - history group - alarm group - event group |
| 2618 | RADIUS MIB | - radiusAuthClientMIB |
| 2665 | Etherlike MIB | - dot3StatsTable group |
| 2737 | Entity MIB | - entityPhysical group |
| 2674 | P-bridge | - dot1dExtBase group - dot1dPriority group - dot1dGarp group |
| 2674 | Q-bridge | - dot1qBase group - dot1qTp group - dot1qStatic group - dot1qVlan |
| IEEE 802.1X | IEEE8021-PAE-MIB | - dot1xPaeSystem group - dot1xPaeAuthenticator group - dot1xPaeSupplicant group |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | coldStart warmStart linkDown linkUp authenticationFailure |
| RFC 1493 | newRoot topologyChange |
| RFC 2819 | risingAlarm fallingAlarm |