ES4624H-SFP-FLF-38

Phase 3Firmware Version 1.3.1.15

INTRODUCTION:

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Testing Version | 0.0.1.3 | Init, EIT Testing | 2010/2/11 |
| Testing Version | 1.1.0.1 | Test | 2010/3/8 |
| Testing Version | 1.1.1.0 | Test | 2010/3/30 |
|  | 1.1.1.1 | For MG | 2010/4/2 |
| Test Version | 1.1.1.2 | NTC Test | 2010/4/16 |
| Test Version | 1.1.1.3 | NTC Test | 2010/4/27 |
| PilotRun Version | 1.1.2.0 | =1.1.1.3. NTC Test | 2010/5/4 |
| Patch Version | 1.1.2.1 | Update | 2010/5/14 |
| Test Version | 1.2.1.0 | Functional test | 2010/6/4 |
| Regrssion Version | 1.2.1.1 | Regression | 2010/7/2 |
| Test Version | 1.2.1.2 | NTC system test | 2010/7/23 |
| Test Version | 1.2.1.3 | NTC bug regression | 2010/8/16 |
| Test Version | 1.2.1.4 | RD internal test CC Code is 9/17 AM10:00 | 2010/9/17 |
| Test Version | 1.2.1.5 | NTC bug regression CC Code is 12/14 1:30PM | 2010/12/14 |
| Test Version | 1.2.1.6 | NTC bug regression CC(ES4626F-SW-FLF-38_V1.2.1.8_Ph2_EC_branch) Code is 12/17 1:30PM | 2010/12/17 |
| ECN Version | 1.2.2.0 | The same as 1.2.1.6 except version number | 2010/12/22 |
| RD Version | 1.3.1.0 | RD internal test CC Code(EC_VOB) is 2/15 AM10:00 | 2011/2/15 |
| RD Version | 1.3.1.1 | RD internal test CC Code(EC_VOB) is 2/16 1:30PM | 2012/2/16 |
| RD Version | 1.3.1.3 | RD internal test CC Code(EC_VOB) is 5/29 10:00AM | 2012/5/29 |
| Phase3 TS test version | 1.3.1.5 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) is 8/17 10:00AM | 2012/8/17 |
| Phase3 Patch | 1.3.1.6 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) is 9/5 10:00AM | 2012/9/5 |
| Phase3 Patch | 1.3.1.7 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 9/12 10:00AM | 2012/9/12 |
| Phase3 Patch | 1.3.1.8 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 9/19 10:00AM | 2012/9/19 |
| Phase3 Patch | 1.3.1.9 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 3/6 10:00AM | 2013/3/6 |
| Phase3 Patch | 1.3.1.10 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 7/23 10:00AM | 2013/7/23 |
| Phase3 Patch | 1.3.1.11 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 9/11 10:00AM | 2013/9/11 |
| Phase3 Patch | 1.3.1.12 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 11/29 1:30P | 2013/11/29 |
| Phase3 Patch | 1.3.1.13 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 11/17 10:00AM | 2013/11/18 |
| Phase3 Patch | 1.3.1.14 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 4/2 10:00AM | 2014/4/2 |
| Phase3 Patch | 1.3.1.15 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 7/30 11:00AM | 2014/7/30 |

3LOADER:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 0.0.1.1 | Init | 2010/2/11 |
| Patch Version | 0.3.0.0 | CC Code(EC_VOB ES4626F-SW-FLF-38_V1.4.1.2_Ph4_branch) on 7/30 11:00AM | 2014/8/12 |

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Loader V0.0.1.1

| 1 | None | None |
| --- | --- | --- |

Loader V0.3.0.0

| 1 | ES4626F-SW-FLF-38-01771 | When we reload the device, we will always see the "colde restart" status on the log |
| --- | --- | --- |

Runtime V0.0.1.3

| 1 | None | None |
| --- | --- | --- |

Runtime V1.1.0.1

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00004 | need have a newly diag for this project |
| 2 | ES4624H-SFP-FLF-38-00018 | Please verify DiffServ on this project |
| 3 | ES4624H-SFP-FLF-38-00019 | Please verify ACL on this project |
| 4 | ES4624H-SFP-FLF-38-00025 | Please verify WEB on this project |
| 5 | ES4624H-SFP-FLF-38-00033 | Combo port sfp-preferred-auto doesn't work |
| 6 | ES4624H-SFP-FLF-38-00036 | PVLAN: Private-vlan VLAN id range is incorrect(1-4094) on CLI |
| 7 | ES4624H-SFP-FLF-38-00043 | qos map trust-mode dscp enable automatically on configured static trunk members. |
| 8 | ES4624H-SFP-FLF-38-00046 | For TFTP download can set invalid ip address for TFTP server |
| 9 | ES4624H-SFP-FLF-38-00047 | While cold reboot DUT display error message |
| 10 | ES4624H-SFP-FLF-38-00049 | SFP ports connected to smartbit does not link up after reboot DUT |
| 11 | ES4624H-SFP-FLF-38-00051 | system description display incorrect on CLI/WEB |
| 12 | ES4624H-SFP-FLF-38-00052 | description of vlan display incorrect while create mst instance |
| 13 | ES4624H-SFP-FLF-38-00053 | web user display online when set management ip filter for http-client |
| 14 | ES4624H-SFP-FLF-38-00054 | DUT prompt exception error, pc's doesn't ping to dut when shutdown trunk, DUT hang after execute any CLI command |
| 15 | ES4624H-SFP-FLF-38-00055 | Root bridge prompt exception error after reboot non-root bridge & while booting non-root bridge hang for 10 minutes |
| 16 | ES4624H-SFP-FLF-38-00056 | Root & Non-root bridge prompt exception error with "[bcmRX]RX: Failed to allocate mem" error after reboot |
| 17 | ES4624H-SFP-FLF-38-00057 | DUT continuously prompt error message and ping sessions failed in overnight test |
| 18 | ES4624H-SFP-FLF-38-00058 | Root(DUT1) and Non-root(DUT2) bridge prompt exception error after disconnect all inter links |
| 19 | ES4624H-SFP-FLF-38-00059 | Root bridge(DUT1) prompt error message after enable igmp querier |
| 20 | ES4624H-SFP-FLF-38-00060 | DUT prompt "[bcmRX]RX: Failed to allocate mem" error while TFTP download. |
| 21 | ES4624H-SFP-FLF-38-00061 | DUT prompt critical error message when check "sh ip igmp snooping" on Root bridge(DUT1) |
| 22 | ES4624H-SFP-FLF-38-00062 | DUT prompt "[bcmRX]RX: Failed to allocate mem" error when send 1000 packets/seconds bi-direction traffic |
| 23 | ES4624H-SFP-FLF-38-00063 | DUT prompt "[bcmRX]RX: Failed to allocate mem" error when configure igmp configuration |
| 24 | ES4624H-SFP-FLF-38-00064 | CLI respond after 4 minutes when execute "show gvrp configuration" or "show gvrp timer" commands on Root bridge(DUT1) |
| 25 | ES4624H-SFP-FLF-38-00065 | DUT's prompt exception error, after 15 min. continuously display error msg. & pc ping session lost when disable LACP |
| 26 | ES4624H-SFP-FLF-38-00066 | Root bridge(DUT1) prompt exception error, after 15 min. continuously display error msg. & pc ping session lost when disable s |
| 27 | ES4624H-SFP-FLF-38-00067 | DUT's prompt exception error, after 15 min. continuously display error msg. & pc ping session lost when move trunk members |
| 28 | ES4624H-SFP-FLF-38-00068 | VRRP: vrrp router doesn't change state from initial to backup when its master priority is not 255 |
| 29 | ES4624H-SFP-FLF-38-00069 | Support DHCP Snooping, DHCP Snooping option 82, IP Source Guard, DAI |
| 30 | ES4624H-SFP-FLF-38-00070 | Support IPv6 ACL |
| 31 | ES4624H-SFP-FLF-38-00071 | Support web page of DHCP Snooping, DHCP Snooping option 82, IP Source Guard, DAI, IPv6 ACL |
| 32 | ES4624H-SFP-FLF-38-00072 | Support SNMP of DHCP Snooping, DHCP Snooping option 82, IP Source Guard, DAI, IPv6 ACL |
| 33 | ES4624H-SFP-FLF-38-00073 | VRRP: vrrp shouldn't change initial state when there's no interface ip subnet covered virtaul ip subnet |
| 34 | ES4624H-SFP-FLF-38-00074 | 10BASE-T -- Transmit Output Voltage and Jitter - Resistive Load over standard |
| 35 | ES4624H-SFP-FLF-38-00075 | 10BASE-T -- Harmonic Content over standard |
| 36 | ES4624H-SFP-FLF-38-00076 | CLI will hang after static route then ip config |

Runtime V1.1.1.0

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00077 | (Enhance) Dump stack frame and log messages on task exception |
| 2 | ES4624H-SFP-FLF-38-00079 | TACACS+: DUT have not sent Accounting stop package to TACACS server. |
| 3 | ES4624H-SFP-FLF-38-00080 | RadiusAcct :DUT have not sent Accounting stop package to radius server. |
| 4 | ES4624H-SFP-FLF-38-00081 | CLI : The CLI does not show on the accounting. |
| 5 | ES4624H-SFP-FLF-38-00082 | 802.1X:Aegis can not authentication with radius server via DUT. |
| 6 | ES4624H-SFP-FLF-38-00083 | IGMP-SNP-Filtering:It can't display all profile-IDs for user choose to bind on interface on web. |
| 7 | ES4624H-SFP-FLF-38-00084 | IGMP Snooping:In "IGMP Member" page, max group:16 , but it can be up to 1024 groups. |
| 8 | ES4624H-SFP-FLF-38-00085 | Cos : By test plan.Set different priority can not to smooth run. |
| 9 | ES4624H-SFP-FLF-38-00086 | dhcp : Host address can accept incorrect ip address net mask |
| 10 | ES4624H-SFP-FLF-38-00087 | CLI : In policy-map configuration.DUT can not specify set the DSCP value |
| 11 | ES4624H-SFP-FLF-38-00088 | Trunk/LACP: One port can be configured to static and dynamic trunk member at the same time . |
| 12 | ES4624H-SFP-FLF-38-00091 | MIB : DynamicArpInspection set a port as a trusted port have incorrect value. |
| 13 | ES4624H-SFP-FLF-38-00093 | IGMP:CLI and web of DUT will not access when join 1024 group |
| 14 | ES4624H-SFP-FLF-38-00094 | RIP:DUT will crash when setting network vlan and show ip rip interface VLAN |
| 15 | ES4624H-SFP-FLF-38-00095 | staticRoute:From web, it should allow to set default route |
| 16 | ES4624H-SFP-FLF-38-00096 | Web: Mail address should be edge-core, not Accton |
| 17 | ES4624H-SFP-FLF-38-00097 | Web:the value of the column about BDR ospf router ID and ip address should change |
| 18 | ES4624H-SFP-FLF-38-00098 | PIMDM:It display error message when unplug transceiver, no router pim, and no ip address. |
| 19 | ES4624H-SFP-FLF-38-00099 | OSPF:when Router-ID allow to change. it should work and display information.And when it didn't allow, it should display error |
| 20 | ES4624H-SFP-FLF-38-00100 | LACP:it didn't work when frist enable lacp , then remove static trunk |
| 21 | ES4624H-SFP-FLF-38-00101 | RIP:DUT didn't learn static route when redistribute static |
| 22 | ES4624H-SFP-FLF-38-00102 | RIP:It display error message, but succeed to set when setting static route |
| 23 | ES4624H-SFP-FLF-38-00103 | IGMP:DUT will crash and display CLITask when no ip igmp static-group |
| 24 | ES4624H-SFP-FLF-38-00104 | PIMSM: DUT didn't forward data to the vlan of the static-group. |
| 25 | ES4624H-SFP-FLF-38-00105 | DynamicArpInspection:When set arp inspection validate. Package have incorrect. |

Runtime V1.1.1.1

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00078 | Enhancement: RIP CLI enhancement part 1: "maximum_prefix" and "passive-interface IFNAME" |
| 2 | ES4624H-SFP-FLF-38-00106 | Change the logo file on web page |

Runtime V1.1.1.2

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00035 | LED does not link up when link conect to copper ports. |
| 2 | ES4624H-SFP-FLF-38-00036 | PVLAN: Private-vlan VLAN id range is incorrect(1-4094) on CLI |
| 3 | ES4624H-SFP-FLF-38-00037 | MgtIPFilter: DUT can be access through telnet when remove management filter ip address configured for telnet. |
| 4 | ES4624H-SFP-FLF-38-00038 | DUT prompt error message while configure trunk members |
| 5 | ES4624H-SFP-FLF-38-00039 | DHCP snooping doesn't work |
| 6 | ES4624H-SFP-FLF-38-00040 | Can not create more than 32 MSTP Instance while DUT support maximum 64 MSTP Instance |
| 7 | ES4624H-SFP-FLF-38-00042 | Ports Lost Packet error message while booting when save any config and reboot DUT |
| 8 | ES4624H-SFP-FLF-38-00043 | qos map trust-mode dscp enable automatically on configured static trunk members. |
| 9 | ES4624H-SFP-FLF-38-00044 | Vlan id which are not mapped with instance that goes out of VLAN Member column. (This IS ONLY alignment issue). |
| 10 | ES4624H-SFP-FLF-38-00045 | switch all renumber cli command is exist while DUT does not support stacking |
| 11 | ES4624H-SFP-FLF-38-00046 | For TFTP download can set invalid ip address for TFTP server |
| 12 | ES4624H-SFP-FLF-38-00047 | While cold reboot DUT display error message |
| 13 | ES4624H-SFP-FLF-38-00048 | DUT display error message While performing Power-On Self Tests(POST) |
| 14 | ES4624H-SFP-FLF-38-00049 | SFP ports connected to smartbit does not link up after reboot DUT |
| 15 | ES4624H-SFP-FLF-38-00050 | Flash size doesn't change when add/delete opcode or config file into flash |
| 16 | ES4624H-SFP-FLF-38-00052 | description of vlan display incorrect while create mst instance |
| 17 | ES4624H-SFP-FLF-38-00053 | web user display online when set management ip filter for http-client |
| 18 | ES4624H-SFP-FLF-38-00054 | DUT prompt exception error, pc's doesn't ping to dut when shutdown trunk, DUT hang after execute any CLI command |
| 19 | ES4624H-SFP-FLF-38-00055 | Root bridge prompt exception error after reboot non-root bridge & while booting non-root bridge hang for 10 minutes |
| 20 | ES4624H-SFP-FLF-38-00056 | Root & Non-root bridge prompt exception error with "[bcmRX]RX: Failed to allocate mem" error after reboot |
| 21 | ES4624H-SFP-FLF-38-00057 | DUT continuously prompt error message and ping sessions failed in overnight test |
| 22 | ES4624H-SFP-FLF-38-00058 | Root(DUT1) and Non-root(DUT2) bridge prompt exception error after disconnect all inter links |
| 23 | ES4624H-SFP-FLF-38-00059 | Root bridge(DUT1) prompt error message after enable igmp querier |
| 24 | ES4624H-SFP-FLF-38-00060 | DUT prompt "[bcmRX]RX: Failed to allocate mem" error while TFTP download. |
| 25 | ES4624H-SFP-FLF-38-00061 | DUT prompt critical error message when check "sh ip igmp snooping" on Root bridge(DUT1) |
| 26 | ES4624H-SFP-FLF-38-00062 | DUT prompt "[bcmRX]RX: Failed to allocate mem" error when send 1000 packets/seconds bi-direction traffic |
| 27 | ES4624H-SFP-FLF-38-00063 | DUT prompt "[bcmRX]RX: Failed to allocate mem" error when configure igmp configuration |
| 28 | ES4624H-SFP-FLF-38-00064 | CLI respond after 4 minutes when execute "show gvrp configuration" or "show gvrp timer" commands on Root bridge(DUT1) |
| 29 | ES4624H-SFP-FLF-38-00065 | DUT's prompt exception error, after 15 min. continuously display error msg. & pc ping session lost when disable LACP |
| 30 | ES4624H-SFP-FLF-38-00066 | Root bridge(DUT1) prompt exception error, after 15 min. continuously display error msg. & pc ping session lost when disable s |
| 31 | ES4624H-SFP-FLF-38-00067 | DUT's prompt exception error, after 15 min. continuously display error msg. & pc ping session lost when move trunk members |
| 32 | ES4624H-SFP-FLF-38-00078 | Enhancement: RIP CLI enhancement part 1: "maximum_prefix" and "passive-interface IFNAME" |
| 33 | ES4624H-SFP-FLF-38-00089 | Dynamic Arp Inspection : When IXIA 1 sent arp to IXIA 2. CLI display debug mode. |
| 34 | ES4624H-SFP-FLF-38-00090 | Dynamic Arp Inspection : CLI display debug mode. |
| 35 | ES4624H-SFP-FLF-38-00092 | ACL: When create maximum ACLs, the WebUI hang up. |
| 36 | ES4624H-SFP-FLF-38-00106 | Change the logo file on web page |
| 37 | ES4624H-SFP-FLF-38-00107 | PIMDM:CLI and web of DUT will not access when show ip mrouter from web |
| 38 | ES4624H-SFP-FLF-38-00108 | OSPF:The value of OSPF process always is -1 when show ip ospf route. |
| 39 | ES4624H-SFP-FLF-38-00109 | 802.1X:when interface port-control set force-authorized Aegis can not connect with radius server via DUT |
| 40 | ES4624H-SFP-FLF-38-00110 | GVRP:DUT can't join dynamic vlan automatically. |
| 41 | ES4624H-SFP-FLF-38-00112 | Routing:DUT didn't learn route entry by RIP/OSPF when configure first time after booting by default |
| 42 | ES4624H-SFP-FLF-38-00116 | 802.1X:When Set auth and radius service port were VLAN2.DUT have not sent any package to radius service |
| 43 | ES4624H-SFP-FLF-38-00117 | ipv6_acl : DUT runnung configuration did not display ipv6_acl. |
| 44 | ES4624H-SFP-FLF-38-00120 | 802.1x : windows 802.1x client can not connect with radius server via DUT |
| 45 | ES4624H-SFP-FLF-38-00121 | DAI : additional Validation set IP mode. ARP target Protocol address set 192.168.255.255(broadcast) can be pass. |

Runtime V1.1.1.3

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00077 | (Enhance) Dump stack frame and log messages on task exception |
| 2 | ES4624H-SFP-FLF-38-00109 | 802.1X:when interface port-control set force-authorized Aegis can not connect with radius server via DUT |
| 3 | ES4624H-SFP-FLF-38-00111 | PIMDM:It didn't receive all multicast data when join max groups |
| 4 | ES4624H-SFP-FLF-38-00114 | GVRP:When dynamic vlan changes, DUT will not be able to correct receive packet. |
| 5 | ES4624H-SFP-FLF-38-00115 | DHCP Snooping:The maximum dynamic binding entries per port are 5, but total max entries on web page is 2400. |
| 6 | ES4624H-SFP-FLF-38-00116 | 802.1X:When Set auth and radius service port were VLAN2.DUT have not sent any package to radius service |
| 7 | ES4624H-SFP-FLF-38-00118 | DHCP : DUT did not assign IP address (192.168.1.18), which is configured in the host DHCP pool. |
| 8 | ES4624H-SFP-FLF-38-00119 | ipv6: DUT cannot bind ACL to port when the ACL create Maximum ACE(default 128.) |
| 9 | ES4624H-SFP-FLF-38-00120 | 802.1x : windows 802.1x client can not connect with radius server via DUT |
| 10 | ES4624H-SFP-FLF-38-00123 | diffserv : When set PHB value in policy mode.The real traffic rate I received doesn't the same as what expected according to |
| 11 | ES4624H-SFP-FLF-38-00124 | GVRP:The VLAN status of DUT is not correct after Topology Change. |
| 12 | ES4624H-SFP-FLF-38-00125 | CLI: Can not save running-config to startup-conig via CLI |
| 13 | ES4624H-SFP-FLF-38-00126 | [DHCP server] "show ip dhcp" displays abnormal message |

Runtime V1.1.2.0

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | None | All the same like V1.1.1.3 except change version number. |

Runtime V1.1.2.1

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00124 | GVRP:The VLAN status of DUT is not correct after Topology Change. |
| 2 | ES4624H-SFP-FLF-38-00127 | DAI : when DUT sent unicast package.Package will flooding. |
| 3 | ES4624H-SFP-FLF-38-00131 | show router dvmrp should be removed |
| 4 | ES4624H-SFP-FLF-38-00133 | Telnet : DUT can not handle over 4 of accounting session by telnet. |
| 5 | ES4624H-SFP-FLF-38-00134 | Cos : Modify the interface Cos Priority will affact the Global Cos mapping results on all the 24 ports. |
| 6 | ES4624H-SFP-FLF-38-00135 | GVRP:When DUT occur Topology Change, DUT can't create dynamic vlan. |
| 7 | ES4624H-SFP-FLF-38-00136 | Web : Set PHB to Queue function does not work. |
| 8 | ES4624H-SFP-FLF-38-00137 | Web:It didn't display multicast routing ingormation detail by web. |
| 9 | ES4624H-SFP-FLF-38-00138 | OSPF:it didn't modify ospf priority and cost's value from CLI |
| 10 | ES4624H-SFP-FLF-38-00139 | MVR:Maximum mvr group range in CLI is 1-1024, but it only can configure up to 255. |

Runtime V1.2.1.0

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00128 | corrections for grammar |
| 2 | ES4624H-SFP-FLF-38-00129 | corrections for grammar |
| 3 | ES4624H-SFP-FLF-38-00141 | Enhance to phase2 features |
| 4 | ES4624H-SFP-FLF-38-00143 | Enhance to phase2 feature (web part) |

Runtime V1.2.1.1

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00122 | diffserv : DUT can not set DSCP value . |
| 2 | ES4624H-SFP-FLF-38-00130 | web: update web help page V1.1.1.3 |
| 3 | ES4624H-SFP-FLF-38-00140 | [Diag] ledtest need 29sec to test. Too long. And test procedure has some problem. |
| 4 | ES4624H-SFP-FLF-38-00142 | Radius:Timeout of Radius setting have different range in Web UI and Console |
| 5 | ES4624H-SFP-FLF-38-00144 | [LED] Diag LED is always flashed with orange color. & It always light green in "Diag" word on front panel |
| 6 | ES4624H-SFP-FLF-38-00147 | (Duplicate ES4626F-SW-FLF-38-00918) [802.1X] Client could ping to DUT successfully from Force-unauthorized port. |
| 7 | ES4624H-SFP-FLF-38-00151 | [Cable-diag] debug message doesn't take off. |
| 8 | ES4624H-SFP-FLF-38-00153 | [Rate Limit] Ingress rate limit should use pause metering, not rule. |
| 9 | ES4624H-SFP-FLF-38-00154 | [Cluster] Set command "cluster commander" will cause DUT exception. |
| 10 | ES4624H-SFP-FLF-38-00155 | [QinQ] Selective QinQ function don't work. |
| 11 | ES4624H-SFP-FLF-38-00161 | [Auto Traffic Control] auto-control-release don't work. |
| 12 | ES4624H-SFP-FLF-38-00163 | [RSPAN] CLI command "vlan 2 media ethernet state active rspan" is missing. |
| 13 | ES4624H-SFP-FLF-38-00169 | [Enhance] FP rule modification. IPv6 acl/qos design change. (CLI, SNMP) |
| 14 | ES4624H-SFP-FLF-38-00178 | [QinQ] After 'copy Factory_Default_Config.cfg to Running-config', global Dot1q-tunnel system doesn't back to disable. |
| 15 | ES4624H-SFP-FLF-38-00179 | IGMP: Display group source will loss when other group has the same source |
| 16 | ES4624H-SFP-FLF-38-00180 | MLD: last-member-max-resp-query-interval command is wrong in runcfg and max-resp-interval shall compare admin query inteval |

Runtime V1.2.1.2

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00132 | System Fan should be removed from "show system" and "Post Result". |
| 2 | ES4624H-SFP-FLF-38-00149 | [Enhance p2] Support multicast protection (IP multicast stream protect). (WEB) |
| 3 | ES4624H-SFP-FLF-38-00150 | [802.1x] Max Request setting doesn't work. |
| 4 | ES4624H-SFP-FLF-38-00151 | [Cable-diag] debug message doesn't take off. |
| 5 | ES4624H-SFP-FLF-38-00152 | [CopyConfig] DUT fails to save running-config into start-up config. |
| 6 | ES4624H-SFP-FLF-38-00156 | [link detection] detection action of shutdown doesn't work. |
| 7 | ES4624H-SFP-FLF-38-00158 | [Pingv6] ping6 local-link address doesn't work. |
| 8 | ES4624H-SFP-FLF-38-00160 | [IPv6 DSCP remapping CoS] function don't work. |
| 9 | ES4624H-SFP-FLF-38-00164 | (Duplicate EPR ES3510MA-FLF-38-00251) [Auto Upgrade] This function works abnormally. |
| 10 | ES4624H-SFP-FLF-38-00165 | [Delay Reboot] Set reload time at Max. limitation of date range, but DUT would shows "reboot time was passed". |
| 11 | ES4624H-SFP-FLF-38-00166 | [Delay Reboot] Use command "reload regularity <hour> <minute>" to reload, the reload time is slower than expect time one date |
| 12 | ES4624H-SFP-FLF-38-00168 | [Delay Reboot] If we change calendar to exceed "reload setting time", the Remaining reload Time present negative value. |
| 13 | ES4624H-SFP-FLF-38-00169 | [Enhance] FP rule modification. IPv6 acl/qos design change. (CLI, SNMP) |
| 14 | ES4624H-SFP-FLF-38-00170 | [Enhance] FP rule modification. IPv6 acl/qos design change. (WEB) |
| 15 | ES4624H-SFP-FLF-38-00171 | [Web] Administration > Log > Remote IPv6 address can't display correctly. |
| 16 | ES4624H-SFP-FLF-38-00173 | [DHCP Dynamic Provision] After getting config from TFTP and finish second startup provision, DUT can't get Console ready. |
| 17 | ES4624H-SFP-FLF-38-00181 | One of PilotRun dut have Port2 LED issue. Need verify it. |
| 18 | ES4624H-SFP-FLF-38-00182 | Enhance igmp snooping display per vlan configuration |
| 19 | ES4624H-SFP-FLF-38-00183 | MLDSNPv2: when one port is in exclude mode and one port is in include mode, the port in exclude mode can receive data |
| 20 | ES4624H-SFP-FLF-38-00184 | [MVR] fail to provision MVR config |
| 21 | ES4624H-SFP-FLF-38-00185 | [Cluster] DUT lacks CLI command "rcommand" |
| 22 | ES4624H-SFP-FLF-38-00186 | HTTPS: Cannot display web page after setting https port number. |
| 23 | ES4624H-SFP-FLF-38-00187 | Suggestion: The description/text of ssh timeout should be the same. |
| 24 | ES4624H-SFP-FLF-38-00188 | Time-Range: When remove time-range.CLI have display incorrect. |
| 25 | ES4624H-SFP-FLF-38-00189 | [enhance] add command "show running interface xxx" |

Runtime V1.2.1.3

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00122 | diffserv : DUT can not set DSCP value . |
| 2 | ES4624H-SFP-FLF-38-00128 | corrections for grammar |
| 3 | ES4624H-SFP-FLF-38-00129 | corrections for grammar |
| 4 | ES4624H-SFP-FLF-38-00145 | [Enhance] mac address learning support hardware learn |
| 5 | ES4624H-SFP-FLF-38-00146 | (Duplicate ES4626F-SW-FLF-38-00958) [Mac Learn] DUT coudn't Learn Mac addresses to Max. value. |
| 6 | ES4624H-SFP-FLF-38-00148 | [Enhance p2] Support multicast protection (IP multicast stream protect). (CLI, SNMP) |
| 7 | ES4624H-SFP-FLF-38-00157 | [802.1x] Mac-based-auth of operation-mode cause Authorized state of "show Dot1x" to be incorrect. |
| 8 | ES4624H-SFP-FLF-38-00172 | [Port Security] This Function work abnormally. |
| 9 | ES4624H-SFP-FLF-38-00176 | [Guest Vlan] When a port applied Guest Vlan State and received a EAPOL Start, It couldn't switch to back Authenticating State |
| 10 | ES4624H-SFP-FLF-38-00177 | [Guest Vlan] After a port applied Guest Vlan State, change operation-mode to 'mac-based-auth' would cause to work abnormally. |
| 11 | ES4624H-SFP-FLF-38-00186 | HTTPS: Cannot display web page after setting https port number. |
| 12 | ES4624H-SFP-FLF-38-00188 | Time-Range: When remove time-range.CLI have display incorrect. |
| 13 | ES4624H-SFP-FLF-38-00190 | Web : The console icon on the front panel of the Web main page can't light when plug the console into the console port. |
| 14 | ES4624H-SFP-FLF-38-00191 | Web:It should display OSPF protocol on routing table by web when learning from OSPF |
| 15 | ES4624H-SFP-FLF-38-00193 | CLI:RIP auth string length should be 16, not 20 in CLI |
| 16 | ES4624H-SFP-FLF-38-00195 | L3Sys:It didn't forward data when using static route |

Runtime V1.2.1.4

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00177 | [Guest Vlan] After a port applied Guest Vlan State, change operation-mode to 'mac-based-auth' would cause to work abnormally. |
| 2 | ES4624H-SFP-FLF-38-00192 | L3Sys:DUT displayed the exception and didn't access DUT when using snmpwalk |
| 3 | ES4624H-SFP-FLF-38-00194 | OSPF: The max entry of OSPF should be 512,but the total entry is more than max entry. |
| 4 | ES4624H-SFP-FLF-38-00197 | [enhance] the entry of MVR group support 1024. |
| 5 | ES4624H-SFP-FLF-38-00198 | IGMP-SNP-Filtering:Maximum profile write 56 on web, but it can't create 57 profile. |
| 6 | ES4624H-SFP-FLF-38-00199 | IGMP-SNP-Filtering:Crate maximum 57 profiles, |
| 7 | ES4624H-SFP-FLF-38-00200 | IGMP-SNP-Filtering: Max trunk interface shows 32 on "Multicast --> IGMP Snooping --> Filter" page |
| 8 | ES4624H-SFP-FLF-38-00201 | (124) [CLI] - use question can display help of parameter, but Console(config-ether-cfm-ma)#mep mpid 1 cc? seem to be unu |
| 9 | ES4624H-SFP-FLF-38-00203 | Tacacs_server: tacacs-server timeout and retransmit can not be set on CLI. |
| 10 | ES4624H-SFP-FLF-38-00206 | VLAN Trunking : We suggest the port of the DUT shall support access mode. |
| 11 | ES4624H-SFP-FLF-38-00207 | STORM: When we set broadcast storm, the received rate is incorrect. |
| 12 | ES4624H-SFP-FLF-38-00209 | VRRP:Two DUTs' vrrp status are "Master". |
| 13 | ES4624H-SFP-FLF-38-00211 | In CLI,it's not support "(con)#ip dhcp snooping data flash" |
| 14 | ES4624H-SFP-FLF-38-00212 | corrections to grammar for "show ip traffic" |
| 15 | ES4624H-SFP-FLF-38-00213 | This switch is not stackable. Please remove "Renumbering" from the menu list in the web and web help and CLI |
| 16 | ES4624H-SFP-FLF-38-00214 | Please remove these items from the menu list for the web and web help pages |
| 17 | ES4624H-SFP-FLF-38-00215 | update web help page to V1.2.1.3 |

Runtime V1.2.1.5

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00162 | [Auto Traffic Control] alarm-fire-threshold is not accurate. |
| 2 | ES4624H-SFP-FLF-38-00196 | IPCFG: after adding 32 ipv4 addresses, add ipv6 address should fail and display error message. |
| 3 | ES4624H-SFP-FLF-38-00202 | Web_UI:DUT keep to add trunk member via Web UI. It will display error message. |
| 4 | ES4624H-SFP-FLF-38-00204 | Web_UI:Max-reauth-req can not be set via Web UI. |
| 5 | ES4624H-SFP-FLF-38-00205 | rate_limit:If the packet-rate of unknown unicast storm is set .the result is incorrect. |
| 6 | ES4624H-SFP-FLF-38-00208 | ACL: Receiver can't receive packets whose source ip and destination ip is permitted. |
| 7 | ES4624H-SFP-FLF-38-00210 | ACL:When port bind IP/IPV6 standard ACL. DUT forwarding packet have incorrect. |
| 8 | ES4624H-SFP-FLF-38-00216 | Web_UI : DUT did not set password type (encrypted password or not ) via Web UI. |
| 9 | ES4624H-SFP-FLF-38-00217 | [IPv6 Tunnel]change tunnel ttl will not take effect |
| 10 | ES4624H-SFP-FLF-38-00218 | [IPv6 Tunnel] connect endpoint directly to DUT will cause 6to4 tunnel using software route |
| 11 | ES4624H-SFP-FLF-38-00219 | Enhancement: VLAN behavior consistency |
| 12 | ES4624H-SFP-FLF-38-00220 | [enhance] STP: add block & shutdown functions in STP |
| 13 | ES4624H-SFP-FLF-38-00221 | MVR: add mvr group range configuration |
| 14 | ES4624H-SFP-FLF-38-00222 | MVR: add mvr group range configuration (web part) |

Runtime V1.2.1.6

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00208 | ACL: Receiver can't receive packets whose source ip and destination ip is permitted. |
| 2 | ES4624H-SFP-FLF-38-00210 | ACL:When port bind IP/IPV6 standard ACL. DUT forwarding packet have incorrect. |
| 3 | ES4624H-SFP-FLF-38-00228 | ACL: Receiver can't receive packets whose source ip and destination ip is permitted. (Need check in to correct branch) |

Runtime V1.3.1.0

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00205 | rate_limit:If the packet-rate of unknown unicast storm is set .the result is incorrect. |
| 2 | ES4624H-SFP-FLF-38-00230 | [enhance] support G.8032 (ERPS) (CLI + SNMP) |
| 3 | ES4624H-SFP-FLF-38-00234 | [enhance] support CFM & need combine with ERPS. |
| 4 | ES4624H-SFP-FLF-38-00235 | [DDM] can not show anything while "show interfaces transceiver ethernet <port>" |
| 5 | ES4624H-SFP-FLF-38-00236 | ERPS: Add non-ERPS device protection. |
| 6 | ES4624H-SFP-FLF-38-00237 | ERPS: support max number of domain to nature limit (max number of ports / 2) |
| 7 | ES4624H-SFP-FLF-38-00238 | get multicas-data-drop running status requs 0 size |

Runtime V1.3.1.1

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00239 | Enhancement: CLI show more than one page, need to provide guideline |
| 2 | ES4624H-SFP-FLF-38-00240 | Enhancement: add "show spanning-tree brief" function |
| 3 | ES4624H-SFP-FLF-38-00241 | Enhancement: Change some LLDP default value from false to true which is same as MO |
| 4 | ES4624H-SFP-FLF-38-00242 | [Diagnose]Enhanced line speed loopback test |
| 5 | ES4624H-SFP-FLF-38-00245 | DHCP Dynamic Provision:Boot filename length over 32 characters will be error. |
| 6 | ES4624H-SFP-FLF-38-00246 | ECS4610-24F version 1.2.2.3 IP Source Guard Issue |
| 7 | ES4624H-SFP-FLF-38-00247 | ECS4610-24F Version 1.2.2.3 無法設定traffic-segmentaion uplink port |
| 8 | ES4624H-SFP-FLF-38-00248 | configration file could not be save in version 1.2.2.4 |
| 9 | ES4624H-SFP-FLF-38-00249 | show running-config will makes switch hang |
| 10 | ES4624H-SFP-FLF-38-00250 | [ECS4610-24F_V1.2.2.5] Multicast stream with TTL=1 is increasing CPU usage |
| 11 | ES4624H-SFP-FLF-38-00251 | ECS4610-24F: Load balancing mechanism |
| 12 | ES4624H-SFP-FLF-38-00253 | Console(config-if)#lldp basic-tlv ? not correct |

Runtime V1.3.1.3

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00254 | [ERPS] These is no non-erps-dev-protect |
| 2 | ES4624H-SFP-FLF-38-00255 | Port 1and Port 2 SFP Link-Up LED light Error |
| 3 | ES4624H-SFP-FLF-38-00256 | SFP Port Shutdown need to TX disable |
| 4 | ES4624H-SFP-FLF-38-00257 | [Enhance] Enlarge the max of file size |

Runtime V1.3.1.5

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00259 | ES4624H-SFP boot exception |

Runtime V1.3.1.5

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00261 | Port 1and Port 2 SFP Link-Up LED light Error |
| 2 | ES4624H-SFP-FLF-38-00262 | SFP Port Shutdown need to TX disable |

Runtime V1.3.1.7

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00258 | Snmpwalk display error then stop |

Runtime V1.3.1.8

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00258 | Snmpwalk display error then stop |
| 2 | ES4624H-SFP-FLF-38-00264 | DUT will hang up when update the runtime version without cold start |

Runtime V1.3.1.9

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00265 | cannot use snmp set portCapabilities |
| 2 | ES4624H-SFP-FLF-38-00266 | Disabled Flow Control is still active] |
| 3 | ES4624H-SFP-FLF-38-00268 | The multicast reserve address will trap to CPU by default. |

Runtime V1.3.1.10

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00267 | The VRRP packet should not be trapped to CPU when VRRP disable. |
| 2 | ES4624H-SFP-FLF-38-00269 | Did not receive any VRRP packet and all VRRP router become as Master |

Runtime V1.3.1.11

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00270 | Use SNMP Setting Runtime Opcode cause system exception |
| 2 | ES4624H-SFP-FLF-38-00271 | CLI command "Show interface transceiver" fails after the SFP has been inserted for more than 248 days |

Runtime V1.3.1.12

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00273 | Speed up the booting time while the VLAN-trunking is configured. |
| 2 | ES4624H-SFP-FLF-38-00274 | [Enhancement] Warming message when the arp table is full. |

Runtime V1.3.1.13

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00276 | Host ping DUT will be OK while arp entries has been full. (Include “Host connected on wrong interface still pingable”) |
| 2 | ES4624H-SFP-FLF-38-00277 | CPU increase too high while learning arp entry. |

Runtime V1.3.1.14

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00273 | Speed up the booting time while the VLAN-trunking is configured. |
| 2 | ES4624H-SFP-FLF-38-00274 | [Enhancement] Warming message when the arp table is full. |
| 3 | ES4624H-SFP-FLF-38-00275 | Missing virtual ip information when show vrrp brief |
| 4 | ES4624H-SFP-FLF-38-00276 | Host ping DUT will be OK while arp entries has been full. |
| 5 | ES4624H-SFP-FLF-38-00277 | CPU increase too high while learning arp entry. |
| 6 | ES4624H-SFP-FLF-38-00278 | [VIA]We can't set save ssh key and generate key at the same time on the web. |
| 7 | ES4624H-SFP-FLF-38-00279 | receive many arp packets in short time will cause ping lost |

Runtime V1.3.1.15

|  | EPR ID | Headline |
| --- | --- | --- |
| 1 | ES4624H-SFP-FLF-38-00280 | When the mac-address-table is full, I can’t login the vlan interface which has be create on switch. |
| 2 | ES4624H-SFP-FLF-38-00281 | support DDM via SNMP |
| 3 | ES4624H-SFP-FLF-38-00282 | GVRP can't learn VLAN from other switches |
| 4 | ES4626F-SW-FLF-38-01771 | When we reload the device, we will always see the "colde restart" status on the log |

modifications:

Loader V0.0.1.1

| 1. Initial version |  |
| --- | --- |

Runtime V0.0.1.3

| 1. Initial version |  |
| --- | --- |

Runtime V1.2.1.0

| 1. Get ES4626F-SW-FLF-38 phase2 V1.2.1.1 and ES3510MA phase2 V1.1.4.3 features. |  |
| --- | --- |

Runtime V1.2.1.5

| 1 | [Enhance] MVR: add mvr group range configuration |
| --- | --- |

Runtime V1.3.1.0

| 1 | [enhance] support G.8032 (ERPS) |
| --- | --- |
| 2 | [enhance] support CFM & need combine with ERPS. |
| 3 | ERPS: Add non-ERPS device protection. |

Runtime V1.3.1.3

| 1 | [Enhance] Enlarge the max of file size. (ES4624H-SFP-FLF-38-00257) It need use a jump code to update to this version V1.3.1.3. |
| --- | --- |

Work as Designed (wAD) eprs:

Runtime V1.2.1.3

| Id | EPR ID | Headline |
| --- | --- | --- |
|  | Solution Log |  |
| 1 | ES4624H-SFP-FLF-38-00146 | (Duplicate ES4626F-SW-FLF-38-00958) [Mac Learn] DUT coudn't Learn Mac addresses to Max. value. |
|  | The test procedure does not consider the possibility of hash collisions. The chip can learn up to its capacity after injecting addintial random SA IXIA stream. |  |
| 2 | ES4624H-SFP-FLF-38-00172 | [Port Security] This Function work abnormally. |
|  | According to current spec (V0.1), it is expected. |  |
| 3 | ES4624H-SFP-FLF-38-00190 | Web : The console icon on the front panel of the Web main page can't light when plug the console into the console port. |
|  | Our product does not support this function. WAD. |  |

Runtime V1.2.1.4

| Id | EPR ID | Headline |
| --- | --- | --- |
|  | Solution Log |  |
| 1 | ES4624H-SFP-FLF-38-00206 | VLAN Trunking : We suggest the port of the DUT shall support access mode. |
|  | In current plan, L3 product does not support access mode. |  |
| 2 | ES4624H-SFP-FLF-38-00209 | VRRP:Two DUTs' vrrp status are "Master". |
|  | Testing procedure doesn't set vrrp vlan as native vlan. This will cause vrrp packet received different as configured vrrp vlan. That's why 2 DUT run as master. |  |
| 3 | ES4624H-SFP-FLF-38-00211 | In CLI,it's not support "(con)#ip dhcp snooping data flash" |
|  | "ip dhcp snooping database flash" command is under previledge mode, not global configuration mode. |  |

Known Bugs and Issues:

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

| 1 | For port rate limit feature. By Broadcom AE Nathan's suggestion, PAUSE metering cannot work well without PAUSE frame mechanism, so use FP metering instead. |
| --- | --- |