EdgeCore ES4524C

Firmware Version 1.2.0.4

INTRODUCTION:

EdgeCore ES4524C is a switch containing defines 24-port Gigabit Ethernet Layer 2/4 Switch. The switch provides 24 10/100/1000GbE CU ports.  Four of those ports are combo ports with associated SFP transceiver slots.

The base hardware is a 1U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common Http browsers such as Netscape Navigator or Microsoft’s Internet Explorer (both browsers should be Version 6.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the EdgeCore ES4524C from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release note prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.2.0.4 | Customer | 2004/08/16 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Category | Feature | Support | Priority |
| --- | --- | --- | --- |
| Monitor Environment | Power Status | Yes | RTS |
| #### Switching Features |  |  |  |
| VLAN | 802.1Q – Tag-based | Yes | RTS |
|  | 802.1Q – Port-based | Yes | RTS |
|  | 802.1Q – Remove Tag | Yes | RTS |
|  | 255 VLANs per switch | Yes | RTS |
|  | 802.1v Protocol VLAN | Yes | RTS |
|  | GVRP | Yes | RTS |
| Priority/ CoS | L2: 802.1p – Tag-based | Yes | RTS |
|  | L2: 802.1p – Port-based | Yes | RTS |
|  | L2: 802.1p - Queuing | 8 queues per port | RTS |
|  | L3 Aware: IP TOS | Yes | RTS |
| Link Aggregation | 802.3ad | Yes | RTS |
|  | Cisco EtherChannel | Yes | RTS |
| Port Mirroring | Port Mirroring | Yes | RTS |
| Multicast Support | IGMP Snooping v1/v2 | Yes | RTS |
|  | Broadcast Storm Control | Yes | RTS |
| Spanning Tree | STP per Switch | Yes | RTS |
|  | Fast Link Option | Yes | RTS |
|  | Rapid Spanning Tree convergence | Yes | RTS |
| #### System Software |  |  |  |
| IP Address Management | Static | Yes | RTS |
|  | BootP | Yes | RTS |
|  | DHCP client | Yes | RTS |
| Configuration Management | Configuration File | Yes | RTS |
|  | Firmware upgrade | Yes | RTS |
|  | Dual firmware images | Yes | RTS |
| Access Control | Various privileges | Yes | RTS |
|  | Remote authentication & authorization -- RADIUS | Yes | RTS |
|  | Remote authentication & authorization – TACACS | Yes | RTS |
|  | ACL (128 rules per ports) | Yes | RTS |
| Web Management | Switch Front Panel View | Yes | RTS |
|  | Look & Feel | Yes | RTS |
|  | Support Secure socket layer | Yes | RTS |
|  | WEB enhanced counters and utilization counters | Yes | RTS |
| Character-Based Management | Cisco-like CLI via Telnet and Serial Port | Yes | RTS |
| Logging | Event logging | Yes | RTS |
|  | Command line activity | Yes | RTS |
|  | Remote syslog (RFC-3164) | Yes | RTS |
| SNMP Management | SNMP v2c | Yes | RTS |
|  | MIB support | Yes | RTS |
|  | Private MIB | Yes | RTS |
|  | Alerts | Yes | RTS |
|  | SNMP access IP address filtering | Yes | RTS |
| Port Security | Mac Based Port Security | Yes | RTS |
|  | Intrusion action shutdown and Trap | Yes | RTS |
|  | 802.1x port based | Yes | RTS |
|  | SSHv1/v2 | Yes | RTS |
| MAU MIB | MAU MIB | Yes | RTS |
| SMTP | Simple Mail Transfer Protocol | Yes | RTS |
| SNTP | Simple Network Timer Protocol | Yes | RTS |
| DNS | Domain Name Server | Yes | RTS |
| 3rd party management applications | HP OpenView for Windows | Yes | RTS |

Installation and Configuration Notes:

In general, the EdgeCore ES4524C will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing EdgeCore ES4524C, please follow the TFTP download instructions that are included with your firmware image upgrade kit. TFTP download instructions are also available on the Support web site at:

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

modifications:

Known Restrictions and Limitations:

- Flow control does not work crossing the chip.

- Trunk/LACP port members needs to be at the same port configuration

- VLAN name max length is 32(same as RFC2674 MIB definition). When user try to set a vlan name from ifalias(rfc2863MIB), user still can set 64 characters maximum but will always be truncated to 32 characters at most, since both are referred to same database. All the other name(port/trunk) max length are still kept at 64 characters at most.

- When I inject priority and tag traffic by SMB, the transmission value has not follow rule for system.

- When running street test over 2 days, the LACP and Dynamic Vlan would crash on system.

- 802.1X:There is only 1 EAP request for MD5 received after send EAP response packet for ID

- Cos: Strict Queuing priority failed.

- when use Secure Shell Client generate the DSA private key, and download to DUT ,the console will halt

Any other problems than those listed above should be reported to our Technical Support Staff.

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2003 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1v | Protocol VLAN |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1213 | MIB-II | - system group - interfaces group - ip group - icmp group - tcp group - udp group - snmp group |
| 1493 | Bridge MIB | - dot1dBase group - dot1dStp group - dot1dTp group - dot1dStatic group |
| 2863 | Interfaces Evolution MIB | - ifXTable group - ifStackTable group |
| 2819 | RMON MIB | - statistics group - history group - alarm group - event group |
| 2618 | RADIUS MIB | - radiusAuthClientMIB |
| 2665 | Etherlike MIB | - dot3StatsTable group |
| 2737 | Entity MIB | - entityPhysical group |
| 2674 | P-bridge | - dot1dExtBase group - dot1dPriority group - dot1dGarp group |
| 2674 | Q-bridge | - dot1qBase group - dot1qTp group - dot1qStatic group - dot1qVlan |
| 2668 | MAU MIB | - dot3IfMauBasic group - dot3IfAutoNeg group |

rivate Enterprise MIB Support:

| Title | #### Version |
| --- | --- |
| ES4524C_38.mib |  |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | coldStart warmStart linkDown linkUp authenticationFailure |
| RFC 1493 | newRoot topologyChange |
| RFC 2819 | risingAlarm fallingAlarm |