Fast Ethernet Switch ES4524D/ES4548

Phase 2 Firmware Version 1.2.0.5

INTRODUCTION:

The ES4524D/ES4548D is a 24/48-port 10/100/1000 L2 standalone management Gigabit Ethernet Switch. The switch configuration is 20/44 10/100/1000 Base-T ports and 4 combo ports, which are comprised of an RJ45 and a SFP.

The base hardware is a 1U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RJ-45 RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common HTTP browsers such as Netscape Navigator (Version 6.2 or above) or Microsoft’s Internet Explorer (Version 5.0 or above)

Local Console Management (LCM) allows the user to monitor and configure the ES4524D/ES4548D from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| System I Version | 1.0.0.3 | Test | 2007/06/03 |
| System II Version | 1.0.0.4 | Test | 2007/07/02 |
| Disposed Version | 1.1.0.1 | Release | 2007/07/29 |
| ACT Version | 1.1.0.2 | Release | 2007/08/26 |
| Patch Version | 1.1.0.3 | Release | 2007/10/03 |
| Patch Version | 1.1.0.4 | Patch | 2007/10/22 |
| Patch Version | 1.1.0.5 | Patch | 2008/01/04 |
| Current Version | 1.1.0.6 | Patch | 2008/06/01 |
| Patch Version | 1.1.0.7 | Patch Release | 2008/11/17 |
| Patch Version | 1.1.0.8 | Patch Release | 2008/12/12 |
| Patch Version | 1.1.0.9 | Patch Release | 2009/01/06 |
| Patch Version | 1.1.0.10 | Patch Release | 2009/01/22 |
| Patch Version | 1.1.0.11 | Patch Release | 2009/01/23 |
| Patch Version | 1.1.0.12 | Patch Release | 2009/2/19 |
| Test Version (Cancel) | 1.2.0.0 | NTC test(The same as 1.1.0.12) | 2009/2/24 |
| Patch Version | 1.2.0.1 | Patch Release | 2009/3/9 |
| Patch Version | 1.2.0.2 | Patch Release | 2009/3/19 |
| Patch Version | 1.2.0.3 | Patch Release | 2009/3/26 |
| Patch Version | 1.2.0.4 | Patch Release | 2009/4/10 |
| Patch Version | 1.2.0.5 | Patch Release | 2009/6/12 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Layer 2 Features |  |  | Supported | Checking Items | Rang/Value | Default setting |
| --- | --- | --- | --- | --- | --- | --- |
| L2 MAC Address Entries |  |  | YES |  | 8K |  |
| Static MAC Address Entries |  |  | YES |  | 1024(Sharded with L2 Address Table) |  |
| MAC address aging time |  |  | YES |  | 0, 10 ~ 65535 seconds | 300 |
| 802.3x |  |  | YES |  |  | Disable |
| Spanning Tree |  | IEEE 802.1D | YES |  |  | Disable |
|  |  | IEEE 802.1s | YES | Max. instances | 32 | Disable |
|  |  | IEEE 802.1w | YES |  |  | Enable |
|  |  | Spanning Tree Fast Forwarding | YES |  |  | Disable |
|  |  | BPDU Filtering | YES |  |  | Disable |
|  |  | Root Guard | YES |  |  | Disable |
| Link Aggregation |  | Static Ethernet Channel | YES | Max. Port Channel Groups | Up to 24 Groups per system | No Port Channel has Configured |
|  |  |  |  | Max. Port Member Per Group | Up to 8 Ports per system | No Port Member has Configured |
|  |  |  |  | Load Balance Methods | 1) XOR of DstMAC/SrcMAC 2) XOR of DstIP/SrcIP | XOR of DstMAC/SrcMAC |
|  |  |  |  | Pattern for Load Balance | Six Trunk Balance mode on Unicast 1. SA, VLAN, Ethertype, source module ID and port ID 2. DA, VLAN, Ethertype, source module ID and port ID 3. SA xor DA, VLAN, Ethertype, source module ID and port ID 4. SIP and source TCP/UDP port 5. DIP and destinat | SIP and source TCP/UDP port |
|  |  | IEEE 802.3AD | YES | Max. Port Channel Groups | Up to 32 Groups per system | No Port Channel has Configured |
|  |  |  |  | Max. Port Member Per Group | Up to 8 Ports per system | No Port Member has Configured |
|  |  |  |  | Load Balance Methods | 1) XOR of DstMAC/SrcMAC 2) XOR of DstIP/SrcIP | XOR of DstMAC/SrcMAC |
|  |  |  |  | Pattern for Load Balance | Six Trunk Balance mode on Unicast 1. SA, VLAN, Ethertype, source module ID and port ID 2. DA, VLAN, Ethertype, source module ID and port ID 3. SA xor DA, VLAN, Ethertype, source module ID and port ID 4. SIP and source TCP/UDP port 5. DIP and destinat | SIP and source TCP/UDP port |
| VLAN |  | IEEE 802.1Q | YES | Max. VLAN Supported | 256 |  |
|  |  |  |  | Available VLAN ID Range | 1 ~ 4092 |  |
|  |  |  |  | VLAN Learning Mode | IVL |  |
|  |  |  |  | Supported multiple untagged per Port | YES |  |
|  |  |  |  | Ingress Filer Setting | YES | Disable |
|  |  | IEEE 802.1V | YES |  | 1) 16 entries per port 2) VLAN group by (Frame type, Protocol) - 3 Frame type : Ethernet type II, 802.3 LLC, 802.3 SNAP - Protocols : IPv4, IPv6, IPX-RAW, IPX-LLC,…other |  |
|  |  | Private VLAN Edge | YES | Private VLAN Type | Traffic Segmentation | Disable |
|  |  | Multicast VLAN Registration | NO |  |  |  |
|  |  | VLAN Switching | NO |  |  |  |
| GVRP |  |  | YES | Max. VLAN Supported | 128 |  |
| Port Mirroring |  |  | YES | Port Mirror Method | Tx Only, Rx Only, Both Tx/Rx | Both Tx/Rx |
|  |  |  |  | Monitor multiple source ports | Yes |  |
|  |  |  |  | Max. Port Mirror sessions | 1 |  |
| RSPAN |  |  | YES |  |  |  |
|  | VLAN Filter |  | NO |  |  |  |
| DHCP |  | Client | YES |  | YES | enabled |
|  |  | Relay | NO |  | YES(5 Entries/VLAN) |  |
|  |  | Snooping | YES |  |  |  |
|  |  | Option 82 | YES |  |  |  |
|  |  | Server Filtering | NO |  |  | N/A |
| ARP inspection |  |  | NO |  |  |  |
| Strom Control |  | Broadcast | YES | Min., Max., threshold & Step | 500 ~ 3FFFF, 1 packet | Disable |
|  |  | Multicast | NO | Min., Max., threshold & Step |  |  |
|  |  | Unknown unicast | NO | Min., Max., threshold & Step |  |  |
| Port Blocking |  | Broadcast | NO |  |  |  |
|  |  | Multicast | NO |  |  |  |
|  |  | Unknown unicast | NO |  |  |  |
| Q-in-Q |  |  | YES |  |  | Disabled |
| L2 Protocol Tunneling |  |  |  |  |  |  |
| IGMP Snooping |  | v1/v2 | YES | Max. Groups supported | 255 Group 32 Router port | Disable |
|  |  | v3 | NO |  |  |  |
| IGMP |  | IGMP Immediate Leave | YES |  |  |  |
|  |  | IGMP filtering | YES |  |  |  |
|  |  | IGMP throttling | YES |  |  |  |
|  |  | Leave proxy | YES |  |  |  |
| QoS Features |  |  |  |  |  |  |
| Number of pripority queue |  |  | 8 | Weight of Queues | 1:2:4:6:8:10:12:14 | 1:2:4:6:8:10:12:14 |
| Scheduling for pripority queue |  | WRR Pripority scheduling | YES |  | Per port configure weight value range 1-15 | Enable |
|  |  | Strict Pripority scheduling | YES |  |  | Disable |
|  |  | Hybrid Pripority scheduling | YES |  |  | Disable |
| COS |  | 802.1p based COS | YES |  | Per System Configure |  |
|  |  | IP TOS/Precedence based COS | YES |  | Per System Configure |  |
|  |  | DSCP based COS | YES |  | Per System Configure |  |
|  |  | TCP/UDP Port based COS | YES |  | Per System Configure |  |
|  |  | VLAN based priority | NO | Max. VLAN Supported |  |  |
| Rate Limiting |  | Ingress Rate Limit | YES | Min., Max., threshold & Step | 1) Per Port-based 2) Giga: - Range 1M bps - 1G bps - Resolution 1M bps | Disable |
|  |  | Egress Rate Limit | YES | Min., Max., threshold & Step | 1) Per Port-based 2) Giga: - Range 1M bps - 1G bps - Resolution 1M bps | Disable |
|  |  | Per Port per COS | NO | Min., Max., threshold & Step |  |  |
|  |  |  |  | Max. Queues Supported |  |  |
|  |  | Per Port per VLAN ID | NO | Min., Max., threshold & Step |  |  |
|  |  |  |  | Max. VLAN Supported |  |  |
| DiffServ |  |  | YES |  |  | Disable |
| Security Features |  |  |  |  |  |  |
| Port Security |  | Static (Sticky) | YES | Maximum entries per port | 1) Per system/stack : Total 1024 MAC 2) Per port : upto 1024 MAC 3) auto-learned support | Disable |
|  |  | Dynamic | YES | Maximum entries per system | Phase II | Disable |
|  |  | Configuable | YES | User can decide static or dynamic via UI |  |  |
|  |  | L3 port security (IP source guard) | YES |  | Phase II |  |
| 802.1x |  | Port-Based | YES |  | 1) support both Single-host/ Multi-host mode 2) Single-host mode: - Per port 1 MAC 3) Multi-host mode: - Per port : upto 1024 MAC - Per system/stack : Total 1024 MAC) | Disable |
|  |  | Mac-Based | NO | Maximum entries per port |  | Disable |
|  |  | Encryption supported | YES | Types supported | MD5, TLS,TTLS, PEAPI) |  |
| ACL |  | Number of rules: | YES | Maximum masks supported | NO | N/A |
|  |  |  |  | Maximum rules per ACL | 96 MAC ACE 96 IPv4 ACE | N/A |
|  |  |  |  | Maximum ACLs per system | 32 | N/A |
|  |  | L2/L3/L4 | YES | Maximum ACLs per port | 1. L2 SA/DA MAC, VLAN based 2. L3 SIP/DIP, subnet based 3. L4 TCP/UDP port | N/A |
| Gratuitous ARP filtering |  |  | NO |  |  |  |
| DOS Attack protection (CPU) |  |  | YES |  |  |  |
| RADIUS (AAA) Client |  |  | YES |  | 1) encryption : MD5 2) Multiple Radius Server : 5 servers |  |
| TACACS+ Client |  |  | YES |  | 1) encryption : MD5 2) Multiple TACACS Server : 1 servers |  |
| ACS |  | V3.0 | YES |  |  |  |
| User name password authentication |  | Local Authentication | YES | Maximum User Entries | 5 |  |
|  |  | Remote Authentication via RADIUS/TACACS+ | YES | Maximum Server Entries | Radius * 5, TACACS+ * 1 |  |
| Management Interface Access Filtering |  | SNMP | YES | Maximum Filtering Entries | 5 |  |
|  |  | WEB | YES | Maximum Filtering Entries | 5 |  |
|  |  | TELNET | YES | Maximum Filtering Entries | 5 |  |
| Management Features |  |  |  |  |  |  |
| Cisco Like CLI |  |  | YES |  |  |  |
| Web Based Management |  |  | YES |  |  |  |
| Telnet |  |  | YES | Maximum Telnet sessioons supported | 4 |  |
| LED support |  |  | YES |  |  |  |
| Environmental alarm relay support |  |  | NO |  |  |  |
| RJ-45 management port support |  |  | YES |  |  |  |
| SNMP |  | v1 | YES | Max. Community entries supported | 5 |  |
|  |  | v2c | YES | Max. Community entries supported | 5 |  |
|  |  | v3 | YES |  |  |  |
| RMON |  | RMON I (1,2,3,9 group) | YES |  | 1) Statistic : Auto create, per port 1 entry 2) Histroy : Auto create, per port 3 entries 3) Alarm : Auto create, per port 1 entries 4) Event : per port 1 entry in average Note: RMON configure will not be saved to Flash) | 1) Statistic : Auto create, per port 1 entry 2) Histroy : Auto create, per port 2 entries 3) Alarm : Auto create, per port 1 entries 4) Event : per port 1 entry in average Note: RMON configure will not be saved to Flash) |
|  |  | RMON II | YES |  | RMON II Probe configure supported Other items are not supported. |  |
| SSH 2.0 |  |  | YES |  | supported cipher : 3DES Digest: MD5, SHA 1, Public key: RSA |  |
| HTTPS and SSL (Secured Web) |  |  | YES |  | supported cipher : 3DES, RC4 Digest: MD5, SHA 1, Public key: RSA |  |
| Dual Image |  |  | YES |  |  |  |
| Configuration File |  | Multiple config file | YES |  |  |  |
| Backup / Restore |  | TFTP | YES |  |  |  |
|  |  | HTTP | YES |  |  |  |
| SNTP |  |  | YES |  | 3 Server, user configurable |  |
| Event/Error Log |  | Local Flash | YES |  | 4080 Entries |  |
|  |  | Remote server via System Log (RFC3164) | YES |  | 5 Servers |  |
|  |  | SMTP | YES |  | 3 Servers |  |
| DNS |  | Client | YES |  |  |  |
|  |  | Proxy | YES |  |  |  |
| Remote Ping |  |  | YES |  |  |  |
| IP Clustering |  |  | YES |  |  |  |
| System Status |  |  | YES |  |  |  |
| Performance Monitoring |  |  | NO |  |  |  |
| Software Download/ Upgrade |  | TFTP | YES |  |  |  |
|  |  | HTTP | YES |  |  |  |
|  |  | Xmodem | YES |  |  |  |
| VDSL OAM |  |  | NO |  |  |  |
| 802.3ah EFM- Ethernet in the First Mile |  |  | NO |  |  |  |
| TDR/VCT |  |  | NO |  |  |  |
| IPv6 Features |  |  |  |  |  |  |
| IPv4/IPv6 Dual Protocol Stack |  |  | YES |  |  |  |
| IPv6 Address Types |  | Unicast | YES |  |  |  |
|  |  | Multicast | YES |  |  |  |
| ICMPv6 |  |  | YES |  |  |  |
| ICMPv6 Redirect |  |  | YES |  |  |  |
| IPv6 Path MTU Discovery |  |  | YES |  |  |  |
| IPv6 Neighbor Discovery |  | Router Discovery | YES |  |  |  |
|  |  | Duplicate Address Detection | YES |  |  |  |
|  |  | Static Cache Entry | YES |  |  |  |
|  |  | Parameter Discovery | YES |  |  |  |
|  |  | Prefix Discovery | YES |  |  |  |
|  |  | Address Resolution | YES |  |  |  |
|  |  | Unreachable Neighbor Detection | YES |  |  |  |
|  |  | Redirect (Host) | YES |  |  |  |
| Stateless Autoconfiguration |  |  | YES |  |  |  |
| Manual Configuration |  |  | YES |  |  |  |
| Generic Prefix |  |  | YES |  |  |  |
| SNMP over IPv6 |  |  | YES |  |  |  |
| HTTP over IPv6 |  |  | YES |  |  |  |
| SSH over IPv6 |  |  | YES |  |  |  |
| IPv6 Telnet Support |  |  | YES |  |  |  |
| IPv6 DNS Resolver |  |  | YES |  |  |  |
| IPv6 RADIUS+ Support |  |  | YES |  |  |  |
| IPv6 TACACS+ Support |  |  | YES |  |  |  |
| IPv6 Syslog Support |  |  | YES |  |  |  |
| IPv6 SNTP Support |  |  | YES |  |  |  |
| IPv6 SMTP Support |  |  | YES |  |  |  |
| IPv6 TFTP Support |  |  | YES |  |  |  |
| Remote IPv6 Ping |  |  | YES |  |  |  |
| IPv6 QoS |  |  |  |  |  |  |
| IPv6 ACL |  | Number of rules: | YES |  |  |  |
|  |  | L2/L3/L4 | YES |  | 1. L2 SA/DA MAC, VLAN based 2. L3 SIP/DIP, subnet based 3. L4 NextHeader, TCP/UDP port, Flow Label |  |
| IPv6 DSCP remapping CoS |  |  | NO |  |  |  |
| Layer 3 Features IPv4 |  |  |  |  |  |  |
| Host Table |  |  | NO |  |  |  |
| Static Route |  |  | NO |  |  |  |
| Net Table |  |  | NO |  |  |  |
| Number of IP interface |  |  | NO |  |  |  |
| Multi-netting |  |  | NO |  |  |  |
| CIDR |  |  | NO |  |  |  |
| Unicast Routing |  | RIP v1/v2 | NO |  |  |  |
|  |  | OSPF | NO |  |  |  |
| ARP |  |  | NO |  |  |  |
| Proxy ARP |  |  | NO |  |  |  |
| Multicast Routing |  | Multicast groups | NO |  |  |  |
|  |  | IGMP v1/v2 | NO |  |  |  |
|  |  | DVMRP | NO |  |  |  |
|  |  | PIM-DM | NO |  |  |  |
|  |  | PIM-SM | NO |  |  |  |
| IP Redundancy |  | VRRP | NO |  |  |  |

Installation and Configuration Notes:

In general, the ES4524D/ES4548D Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ES4524D/ES4548D Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

V0.0.0.3

1. First version for NTC testing

V0.0.0.4

1. ES4524D-FLF-38-00012 --- Clustering: EIT test fail in item 61.10

2. ES4524D-FLF-38-00015 --- [QinQ] EIT test fail:66.6

3. ES4524D-FLF-38-00017 --- Cluster: Cluster request packets interval is incorrectly

4. ES4524D-FLF-38-00018 --- NetworkNotice: Syslog server display is incorrect.

5. ES4524D-FLF-38-00019 --- NetworkNotice: DUT can't set SMTP email-address without "@"

6. ES4524D-FLF-38-00020 --- Cluster: DUT can't create cluster member over 16 .

7. ES4524D-FLF-38-00021 --- PortSecurity:If some port join to LACP then all ports can't be enabled security on the web.

8. ES4524D-FLF-38-00024 --- IPv6-ACL: The status of port binding always shows "disable" after binding an IPv6 ACL to any port.

9. ES4524D-FLF-38-00026 --- Radius: After save and reload the DUT, radius authentication failed.

10. ES4524D-FLF-38-00027 --- ICMPv6: ICMPv6 can’t work when configure IPv6 Global address only.

11. ES4524D-FLF-38-00028 --- ICMPv6: Use console ping to PC1"ping ipv6 2000::1" is successfully, but must retry 3 times.

12. ES4524D-FLF-38-00029 --- ICMPv6: After save and reload the DUT, the ICMPv6 can’t work.

13. ES4524D-FLF-38-00032 --- ACL: Web can't remove the mac ACE including ether type.

14. ES4524D-FLF-38-00033 --- ACL: Web can't remove the mac ACE which CLI created with VID.

15. ES4524D-FLF-38-00034 --- ACL: Max VID bitmask is 4093 at CLI but 4095 at Web.

16. ES4524D-FLF-38-00035 --- CLI: DUT only supports standalone, so stacking command should be removed.

17. ES4524D-FLF-38-00036 --- DUT can not be loggeg in through TACACS from privilege 0 changing to privilege 15 mode.

18. ES4524D-FLF-38-00039 --- MgtIPFilter: DUT can't align the ip address when show management ip filter .

19. ES4524D-FLF-38-00040 --- ACL: DUT does not support Egress ACL, so Egress items should be remove at Web.

20. ES4524D-FLF-38-00041 --- ACL: (Suggestion) CLI looks irregular to show ACL with long rules.

21. ES4524D-FLF-38-00042 --- NetworkNotice: The typeface of logging sendmail hint is incorrect.

22. ES4524D-FLF-38-00043 --- SNTP: DUT can configure 3 SNTP Server only,but DUT SNTP: DUT can configure 3 SNTP Server only,but DUT support 5 SNTP Server .

23. ES4524D-FLF-38-00046 --- IGMPSnooping: When DUT received leave message ,DUT would send GS query to the member port which received V1 query.

24. ES4524D-FLF-38-00047 --- ACL: IP ACL can accept a ~ f for input box(decimal format) at Web.

25. ES4524D-FLF-38-00048 --- ACL: Precedence rule does not work when IP ACL makes ACE with Precedence + ToS.

26. ES4524D-FLF-38-00049 --- PortMirror : Listen port loses packets near 5% at TX mode.

27. ES4524D-FLF-38-00050 --- QinQ: The Up-link port doesn’t copy priority value of the incoming packets to SPVLAN tag.

28. ES4524D-FLF-38-00051 --- QinQ: All ports auto change to Access mode when enable Access mode of port 1.

29. ES4524D-FLF-38-00052 --- ICMPv6: PC can’t ping to DUT through IPv6 when DUT enable IPv6 address.

30. ES4524D-FLF-38-00053 --- L2Switch:The SMB(X+1) with 10M/F capability couldn't not received any packets sent from the SMB(X) when preamble length set a

31. ES4524D-FLF-38-00056 --- CoS: CLI shows error mesg "bad reserved_rule_id[261]" after setting muliple entries with IP port mapping.

32. ES4524D-FLF-38-00057 --- CoS: CLI shows error mesg "bcm_field_entry_remove[-16]" after setting a IP port mapping entry & disabling it at global.

33. ES4524D-FLF-38-00060 --- L2switch:the Description display not correctly via SNMP MIB browser.

34. ES4524D-FLF-38-00061 --- PortTrunk: CLI can create 32 trunk groups, but Product Spec. claims 8.

35. ES4524D-FLF-38-00063 --- PortMirror: Listen port at RX mode receives more broadcast packets than other ports.

36. ES4524D-FLF-38-00065 --- SNMP: The DUT do not send out TRAP when use other UDP-port of Trap manager.

37. ES4524D-FLF-38-00067 --- Diffserv: Diffserv received rate is incorrectly when use VLAN traffic class.

38. ES4524D-FLF-38-00070 --- L2switch: The Mac address within "01-80-c2-00-00-00~01-80-c2-00-00-0f" can't be filter.

39. ES4524D-FLF-38-00072 --- SSH: Download the public-key DSA for admin user, DUT has display tExcTask message.

40. ES4524D-FLF-38-00073 --- SSHMIB: Object "sshUserKeyDelAction" can’t delete DSA/RSA/Both public-key.

41. ES4524D-FLF-38-00074 --- SNMPv3: Configuration local EngineID 8 chars is failed from CLI command/WEB browser.

42. ES4524D-FLF-38-00075 --- IF MIB: Get ifInUnknownProtos and verity it doesn't increment value when send unknown ether type.

43. ES4524D-FLF-38-00078 --- Q-Bridge MIB: Set the value is “00 00 01” of dot1qStaticUnicastAllowedToGoTo.1.0.0.238.0.0.3.0, it can't set.

44. ES4524D-FLF-38-00080 --- L2 switch :Throughput:DUT lost 90% packets with 100% loading uni-direction under Throughput with Spanning Tree test.

45. ES4524D-FLF-38-00085 --- SNMPv3: Community MIB cannot get, getnext, set.

46. ES4524D-FLF-38-00086 --- PortMirror: Product Spec. claims that max sessions is 2, but DUT can create more than 2 sessions.

47. ES4524D-FLF-38-00087 --- RateLimit : Ingress RL of trunk doesn't work.

48. ES4524D-FLF-38-00023 --- IPv6-ACL: CLI sets DA of extended ACL to "3FFF::/128", but its behavior is like "3FFF::/64".

49. ES4524D-FLF-38-00030 --- IPv6-ACL: Web can't remove IPv6 standard ACL including host SIP.

50. ES4524D-FLF-38-00031 --- ACL: Ether Type is decimal at Web but Hex at CLI.

51. ES4524D-FLF-38-00044 --- HTTPS&SSL: DUT can't download cetificattion file from WEB UI.

52. ES4524D-FLF-38-00066 --- Diffserv: The meter rate shall modify to "1-1000000" of the policy rules.

V0.0.0.5

1.ES4524D-FLF-38-00014 --- [QinQ]:EIT test Fail:66.4

2.ES4524D-FLF-38-00019 --- NetworkNotice: DUT can't set SMTP email-address without "@"

3.ES4524D-FLF-38-00022 --- IPv6-ACL: The ACEs including link-local IP disappear after DUT saves running-config & reboots.

4.ES4524D-FLF-38-00025 --- ImageDownload:the display not properly on console after download incorrect image.

5.ES4524D-FLF-38-00026 --- Radius: After save and reload the DUT, radius authentication failed.

6.ES4524D-FLF-38-00043 --- SNTP: DUT can configure 3 SNTP Server only,but DUT SNTP: DUT can configure 3 SNTP Server only,but DUT support 5 SNTP Server .

7.ES4524D-FLF-38-00045 --- ACL: CLI loses control after assigning ACL with illegal IP.

8.ES4524D-FLF-38-00058 --- CoS: Minimum value of TCP port is 1 at CoS but 0 at ACL.

9.ES4524D-FLF-38-00062 --- ImageDownload: The display not properly on console( it show as "Write to FLASH Programming.") when download the same version

10.ES4524D-FLF-38-00064 --- LACP: Status of port1 become "Error" after CLI disables port-channel1 continuously.

11.ES4524D-FLF-38-00068 --- Diffserv: The class-map/policy-map name does not display when configure description on WEB.

12.ES4524D-FLF-38-00069 --- MSTP:DUT lost packets under the testing of attach file.

13.ES4524D-FLF-38-00071 --- IPv6-AddressConfig: When Enable IPV6 address autoconfig,DUT would not send router solicication.

14.ES4524D-FLF-38-00076 --- IP MIB: Default TTL is 255 when form PC ping to DUT.

15.ES4524D-FLF-38-00079 --- RMON:Object historyControlOwner should have value "Monitor" but it is empty.

16.ES4524D-FLF-38-00081 --- L2 switch:The RPU LED doesn't color green when the connect the AC power back to the DUT.

17.ES4524D-FLF-38-00082 --- SNMP: Object dot3StatsAlignmentErrors doesn't increase any value when transmit bad Alignment packets.

18.ES4524D-FLF-38-00088 --- GVRP:DUT doesn't send empty event when receiving LeaveEmpty in VO state.

19.ES4524D-FLF-38-00089 --- GVRP:DUT only send one JoinEmpty event when receiving JoinEmpty,Empty,LeaveIn or LeaveEmpty event in QA state.

20.ES4524D-FLF-38-00090 --- UserAuth:Some objects value of RADIUS MIB doesn't increase.

21.ES4524D-FLF-38-00091 --- PortTrunk: After Web chooses a Load balance mode, CLI shows a different one.

22.ES4524D-FLF-38-00092 --- IPV6-AddressConfig: The typeface of ipv6 neighbor seeting is incorrectly.

23.ES4524D-FLF-38-00094 --- Cluster: The MAX ip-pool value is incorrectly

24.ES4524D-FLF-38-00095 --- Cluster: DUT has no CLI to clear cluster candidate

25.ES4524D-FLF-38-00096 --- Cluster: DUT can add cluster member over 32

26.ES4524D-FLF-38-00098 --- SNTP: DUT cannot configure SNTP of IPv4/IPv6 when user configuration first SNTP server only.

27.ES4524D-FLF-38-00099 --- Network Monitor: Oversize and Jabbers counters cannot work.

28.ES4524D-FLF-38-00101 --- SNTP: The product does not prodive calendar command.

29.ES4524D-FLF-38-00103 --- DUT doesn't have tftp to public-key choice on the web.

30.ES4524D-FLF-38-00104 --- DUT doesn't show public-key which users upload information on the web.

31.ES4524D-FLF-38-00105 --- SSH client will be down if use wrong passphrase on RSA.SSH client can continually try passphrase on DSA.

32.ES4524D-FLF-38-00106 --- Cluster: Cluster member would hang up when create MAX cluster member on commander .

33.ES4524D-FLF-38-00107 --- IP Filter: CLI has not IPV6 specific command.

34.ES4524D-FLF-38-00129 --- 802.1x: The DUT allows 2 client authenticated when dot1x operation-mode set as single mode.

35.ES4524D-FLF-38-00130 --- configuration file: The DUT doesn't reject error configuration file.

36.ES4524D-FLF-38-00132 --- Diffserv: The Police of Policy element does not exist after user configure it well.

37.ES4524D-FLF-38-00133 --- Diffserv: The class-map/policy-map name does not display when configure description on WEB.

38.ES4524D-FLF-38-00134 --- 802.1X:Object dot1xAuthSessionTerminateCause value of PAE MIB is incorrect.

39.ES4524D-FLF-38-00135 --- SNMP:Object dot1qVlanCreationTime of vlan 4093 will disappear after creating new

40.ES4524D-FLF-38-00136 --- CoS: CLI shows error mesg "bcm_field_action_add[-15] & [-16]" after setting IP port mapping entry & removing it.

41.ES4524D-FLF-38-00137 --- RateLimit: Ingress RL is inaccuracy to enable over 40% limit for ES4548D-FLF-38.

42.ES4524D-FLF-38-00138 --- ACL: Web can't create IP Extended ACL well.

43.ES4524D-FLF-38-00139 --- ACL: CLI should remove information about Egress ACL.

44.ES4524D-FLF-38-00141 --- ACL: Web accepts any characters to Ether Type.

45.ES4524D-FLF-38-00142 --- ACL: It doesn't work for Ethernet Type with 0.

46.ES4524D-FLF-38-00143 --- ACL: It can't make any rules for ACL named by special characters or space key.

47.ES4524D-FLF-38-00144 --- IPv6-ACL: CLI should avoid showing Standard type with Extended ACL name.

48.ES4524D-FLF-38-00145 --- DiffservMIB: DiffservMIB: Some object(Type = INTEGER32) could set "aa", "kk", "@@" on MIB Browser.

49.ES4524D-FLF-38-00146 --- DiffservMIB: Get object diffServIpAceControlCodeBitmask, the default value is "65535".

50.ES4524D-FLF-38-00147 --- IPv6 Radius: The prompt messages of IPv6-addr shall modify to XX:XX:XX:XX::XX.

51.ES4524D-FLF-38-00156 --- IPv6 SNMP: DUT doesn't auto carry %100 when I configure trap manager IPv6 addr "fe80::56".

52.ES4524D-FLF-38-00157 --- SNMPv3 : Configure invalid trap manager IP addr "192.168.1.1.1.1" on WEB browser, DUT be able to set.

53.ES4524D-FLF-38-00158 --- SNMPv3: Configre UDP port is "@@/ -1/0.1", DUT shall filter this setting.

54.ES4524D-FLF-38-00159 --- IPv6 SMTP: When user configure none specified character, it does not be filtered.

55.ES4524D-FLF-38-00161 --- Diffserv: Configure special character of Class rule(VLAN), it does not be filtered.

56.ES4524D-FLF-38-00163 --- Vlan: DUT would forward tagged packets to the port which did not the member of the vlan.

57.ES4524D-FLF-38-00164 --- Cluster: Cluster member would hang up when create MAX cluster member on commander .

58.ES4524D-FLF-38-00170 --- The MAC-address agint-time can' t be setting on CLI.

59.ES4524D-FLF-38-00171 --- IPv6 SNMP: SNMP-INFORM does not work well through IPv6 when ports Link-up/Link-down/AuthenticationFailed.

60.ES4524D-FLF-38-00172 --- Diffserv:CLI command: burst range <64-1522> should be modified to new naming constants in arg.c

61.ES4524D-FLF-38-00174 --- System can't do BPDU flooing when STP disabled.

62.ES4524D-FLF-38-00176 --- QinQ (web) : modify TPID's setting method from per port setting to per system setting.

V0.0.0.6

56.ES4524D-FLF-38-00163 --- Vlan: DUT would forward tagged packets to the port which did not the member of the vlan.

V1.0.0.0

1. ES4524D-FLF-38-00037 --- IPv6-ACL: "% interface" for Link-local address can't write to configure.

2. ES4524D-FLF-38-00038 --- IPv6-ACL: CLI should preven %(interface) from setting to none Link-Local address.

3. ES4524D-FLF-38-00059 --- IPV6-AddressConfig: DUT can't delete general-Prefix

4. ES4524D-FLF-38-00083 --- SNMP: The sysContact/sysName/sysLocation characters was disorder.

5. ES4524D-FLF-38-00086 --- PortMirror: Product Spec. claims that max sessions is 2, but DUT can create more than 2 sessions.

6. ES4524D-FLF-38-00102 --- RateLimit: Ingress RL becomes workless after DUT saves running-config & reboots.

7. ES4524D-FLF-38-00131 --- IPV6-TAHI-BaseInterOp: DUT would hang up when ping a host which unreachable .

8. ES4524D-FLF-38-00140 --- ACL: The MAC ACE can't be removed at Web after only assigning VID Bitmask or Ether Type Bitmask.

9. ES4524D-FLF-38-00148 --- RateLimit: Ingress RL becomes workless after DUT saves running-config & reboots.

10. ES4524D-FLF-38-00151 --- ManytoOne:The performance test of many to one had packet loss in part of frame size the rate about 0.3%-100%.

11. ES4524D-FLF-38-00152 --- PartiallyMeshed:The DUT occurs packet lost 100% with the all port's speed/deplex and frame size 8192/9216.

12. ES4524D-FLF-38-00153 --- FullyMeshed:The DUT occurs packet lost 5%-35% with the port's speed/duplex 10F/100F/1G and frame size 1024-9216.

13. ES4524D-FLF-38-00160 --- IPv6 SMTP: DUT has not auto add "%" when user apply IPv6 Link-local addr "fe80::123" of Email-src-addr.

14. ES4524D-FLF-38-00165 --- IPv6 address: When user configure invalid IPv6 addr of each protocal on DUT(CLI/WEB/MIB), it shall be filtered.

15. ES4524D-FLF-38-00166 --- Enable 12G bandwidth of BCM 5630x series

16. ES4524D-FLF-38-00171 --- IPv6 SNMP: SNMP-INFORM does not work well through IPv6 when ports Link-up/Link-down/AuthenticationFailed.

17. ES4524D-FLF-38-00177 --- AdvCustomer: multicast forward incorrectly in the system test

18. ES4524D-FLF-38-00179 --- CoreCustomer: LACP Admin-key can't set to 1

19. ES4524D-FLF-38-00180 --- Cluster: fix code in cluster_mgr

20. ES4524D-FLF-38-00181 --- Update trunk group support to 24 groups.

21. ES4524D-FLF-38-00182 --- Cluster: Cluster member can't set sysName by SNMPV3

22. ES4524D-FLF-38-00184 --- SMTP: when I configure "@" only for sendmail , the DUT has be set.

23. ES4524D-FLF-38-00185 --- DiffservMIB: The diffServMeterRate cannot set value "100001 to 1000000".

24. ES4524D-FLF-38-00186 --- DiffservMIB: The named of the Class-map and Policy-map could be set when I configure named include space, example "P P".

25. ES4524D-FLF-38-00187 --- CoS: CoS can't set TCP port value with 0 by CLI or Web.

26. ES4524D-FLF-38-00188 --- CoS: Trunk has CLI command to set queue bandwidth, but the command is useless.

27. ES4524D-FLF-38-00189 --- MgtIPFilter: The IP range does not display when I configure IP range from "192.168.255.160" to "192.168.255.210" by CLI.

28. ES4524D-FLF-38-00191 --- Radius: when I use "user4" of Normal-user by Radius-server to login the DUT, which is not convert to Enable mode.

29. ES4524D-FLF-38-00192 --- Logging: DUT should accept name@IPv6-address for SMTP src/dest email address

30. ES4524D-FLF-38-00193 --- ACL: Web & SNMP can create ACL name with space key, but CLI can't.

31. ES4524D-FLF-38-00194 --- SNMP:Some object value of Q-Bridge MIB is incorrect.

32. ES4524D-FLF-38-00195 --- RMON:Object etherStatsOversizePkts and etherStatsJabbers value is always 0.

33. ES4524D-FLF-38-00202 --- When the CLI provision processing "port security" command, illegal access cause the exception in MIPS.

34. ES4524D-FLF-38-00204 --- the protocol vlan system configuration setting will disappear after save and reboot DUT.

35. ES4524D-FLF-38-00206 --- [ES4548D-FLF-38]:Enable ingress rate limit on ethernet port will effect on ports of both chip

V1.0.0.1

1.ES4524D-FLF-38-00054 --- L2switch :There are 3 frames flooding when do the MAC table size test.

2.ES4524D-FLF-38-00083 --- SNMP: The sysContact/sysName/sysLocation characters was disorder.

3.ES4524D-FLF-38-00196 --- GVRP: DUT can't enable GVRP on the lacp port-channel when the port-channel has no member .

4.ES4524D-FLF-38-00201 --- LACP: LACP trunk doesn't work well after the port member is mirrored.

5.ES4524D-FLF-38-00207 --- Cluster: There is no error message when trying to start a Telnet session with an inactive member.

6.ES4524D-FLF-38-00208 --- IPConfiguration: The DUT will hang up when restart IP DHCP.

7.ES4524D-FLF-38-00211 --- ACL: The ACL port binding setting can't be apply when name of ACL rule set as " " (space key).

8.ES4524D-FLF-38-00213 --- CoS: TCP Port Mapping doesn't work well through Web.

9.ES4524D-FLF-38-00215 --- IPConfigure: DUT will generate IP default-gateway automatic when user delete IP address then save and reload it.

10.ES4524D-FLF-38-00219 --- Failed to set dot1q-tunnel mode while enable dot1q-tunnel.

11.ES4524D-FLF-38-00221 --- DUT would transmit GS query contiunous and group would not time out when receive IGMP Leave message

12.ES4524D-FLF-38-00229 --- IPv6-ACL: CLI, SNMP, and Web can make any standard IPv6 ACE at V0.0.0.6 but can't at V1.0.0.0.

13.ES4524D-FLF-38-00237 --- ACL: The ACE entry can't be remove when Ethernet Type Bit Mask set as 12343.

V1.0.0.3

1. ES4524D-FLF-38-00093 IPv6-addressConfig: DUT would error message when ping host with verbose.

2. ES4524D-FLF-38-00097 IPv6-AddressConfig: The traffic of IPV6 ICMP Output display incorrect.

3. ES4524D-FLF-38-00108 IPv6-ANVL-NDP: ANVL test failed

4. ES4524D-FLF-38-00109 IPv6-ANVL-NDP: ANVL test failed

5. ES4524D-FLF-38-00110 IPv6-ANVL-NDP: ANVL test failed

6. ES4524D-FLF-38-00111 IPv6-ANVL-NDP: ANVL test failed

7. ES4524D-FLF-38-00112 IPv6-ANVL-NDP: ANVL test failed

8. ES4524D-FLF-38-00113 IPv6-ANVL-NDP: ANVL test failed

9. ES4524D-FLF-38-00114 IPv6-ANVL-NDP: ANVL test failed

10. ES4524D-FLF-38-00115 IPv6-ANVL-NDP: ANVL test failed

11. ES4524D-FLF-38-00116 IPv6-ANVL-NDP: ANVL test failed

12. ES4524D-FLF-38-00117 IPv6-ANVL-NDP: ANVL test failed

13. ES4524D-FLF-38-00118 IPv6-ANVL-NDP: ANVL test failed

14. ES4524D-FLF-38-00119 IPv6-ANVL-NDP: ANVL test failed

15. ES4524D-FLF-38-00120 IPv6-ANVL-NDP: ANVL test failed

16. ES4524D-FLF-38-00121 IPv6-ANVL-NDP: ANVL test failed

17. ES4524D-FLF-38-00122 IPv6-ANVL-NDP: ANVL test failed

18. ES4524D-FLF-38-00123 IPv6-ANVL-NDP: ANVL test failed

19. ES4524D-FLF-38-00124 IPv6-ANVL-NDP: ANVL test failed

20. ES4524D-FLF-38-00125 IPv6-ANVL-NDP: ANVL test failed

21. ES4524D-FLF-38-00126 IPv6-ANVL-NDP: ANVL test failed

22. ES4524D-FLF-38-00127 IPv6-ANVL-NDP: ANVL test failed

23. ES4524D-FLF-38-00150 IPv6 Stress: Open a session to telnet to DUT, the DUT will crash when “show ip inter” and “show ipv6 inter” in CLI.

24. ES4524D-FLF-38-00154 CoreCustomer: Packets would flood in the system test

25. ES4524D-FLF-38-00214 IPv6 SNMP: DUT does not send out TRAP/INFORM messages through IPv6 when warm-start/cold-start it.

26. ES4524D-FLF-38-00215 IPConfigure: DUT will generate IP default-gateway automatic when user delete IP address then save and reload it.

27. ES4524D-FLF-38-00220 LACP: Ingress RL with LCAP trunk becomes workless after DUT saves running-config & reboots.

28. ES4524D-FLF-38-00230 Diffserv: MG-Soft can set control code without indicating protocol with TCP.

29. ES4524D-FLF-38-00232 Diffserv: The value of MIB object "diffServIpAceType" alway gets "standard" after setting to "extended".

30. ES4524D-FLF-38-00240 In cgi_submit_tunnelstatus, the argument is_static of SWCTRL_IS_TRUNK_MEMBER function is wrong.

31. ES4524D-FLF-38-00247 IPv6 stress: PC connect to DUT through telnet session overnight, the DUT cannot pint to PC.

32. ES4524D-FLF-38-00249 Projx000746, When 4079 events (max loggable) logged on flash, console/ping/telnet become very slowly.

33. ES4524D-FLF-38-00252 (ES4649-ZZ)Cluster:support Console/WEB access from command to member

34. ES4524D-FLF-38-00253 Support MVR

35. ES4524D-FLF-38-00254 XSTP Root Guard feature

36. ES4524D-FLF-38-00256 Unknown multicast data behaviors are differ from setting/removeing router port.

37. ES4524D-FLF-38-00257 Support IGMP filtering, throttling and IGMPSNP leave-proxy, immediately-leave, v3

38. ES4524D-FLF-38-00258 Clustering Web speed is too slow

39. ES4524D-FLF-38-00263 SSH2:will abruptly end the ssh session while showing large data(ex:show tech)

V1.0.0.4

1. ES4524D-FLF-38-00267 RQT#091:Register "ip source-guard" to a port maximum in a port.However, can register to other port.Why can register to other

2. ES4524D-FLF-38-00268 RQT#129: The IP Source Guard binding entry can't be added to maximum number per port sometimes

3. ES4524D-FLF-38-00260 Enhancement: Support DHCP Snooping + Option82, IP Source Guard

4. ES4524D-FLF-38-00227 SNMPv3: The Security Level of ccc did not display on WEB when create a user "ccc" from MIB.

5. ES4524D-FLF-38-00242 IQT#008: In the interface configuration mode,spanning-tree p? , is unrecognized.

6. ES4524D-FLF-38-00274 SSHv2:After connect to DUT with RSA/DSA public key then user need to input username and password.

7. ES4524D-FLF-38-00272 IGMP Filter: After we save and rebooting, DUT loss the IGMP Filter and Throttling configuration in Trunk.

8. ES4524D-FLF-38-00262 Cluster: Telnet relay to a real member first, then to an nonexistent member, we can get the previous relayed session.

9. ES4524D-FLF-38-00278 dhcprelay : the DUT can't process dhcp request message correctly when set the option 82 jpolicy to drop.

10. ES4524D-FLF-38-00279 dhcprelay : when set the option 82 policy to "keep" the DUT can't process dhcp request message correctly

11. ES4524D-FLF-38-00283 RootGuard:RootGuard doesn't work and no associated pages on the web and no information on the CLI.

12. ES4524D-FLF-38-00261 Porting TACAS+ (AAA)

13. ES4524D-FLF-38-00286 TACACSAcct:DUT can't login to DUT as admin privilege with TACACS+ Authorization.

14. ES4524D-FLF-38-00285 TACACSAcct:DUT will reload automatically when show Accounting Summary on the CLI and web.

15. ES4524D-FLF-38-00282 SNMP:CLI will show some messages after continuously snmp Get-Next for some time.

V1.1.0.1

1. ES4524D-FLF-38-00055 -- CoS: CLI has no Hybrid mode command, but Feature List claims to support.

2. ES4524D-FLF-38-00212 -- CoS: The failed mesg needs be modified when CLI changes bandwidths of the trunk without any port membmers.

3. ES4524D-FLF-38-00248 -- change the setting range of TPID from 0x800~0xFFFF to 0x8000~0xFFFF.

4. ES4524D-FLF-38-00250 -- ST2#013:The port monitor command can be configured for two ports once

5. ES4524D-FLF-38-00255 -- Check in un-compaitable files of ES4524D, after ES4626H updated UoW features.

6. ES4524D-FLF-38-00259 -- 802.1X pass-through EAP-PEAP, EAP-TLS, and EAP-TTLS

7. ES4524D-FLF-38-00261 -- Porting TACAS+ (AAA)

8. ES4524D-FLF-38-00266 -- Enhancement: Show correct error message when selector is unavailable for IPSG or QoS.

9. ES4524D-FLF-38-00269 -- PortMirror:If use TX port mirror method then mirroring port will receive vlan1 taggged packets.

10. ES4524D-FLF-38-00271 -- IGMPSnoop: The IGMP-version configuration is incorrect in WEB.

11. ES4524D-FLF-38-00273 -- SSHv2:Web show success when user download the RSA private key to DUT.

12. ES4524D-FLF-38-00275 -- QinQ: We cannot set the ether-type of outer tag from WEB.

13. ES4524D-FLF-38-00276 -- QinQ: After we enable dot1q-tunnel mode to access on port 1 from WEB, the page show enabling on all ports.

14. ES4524D-FLF-38-00277 -- QinQ: After we enable dot1q-tunnel mode to access on trunk 1 from WEB, the page show enabling on all trunks.

15. ES4524D-FLF-38-00280 -- QinQ: We cannot set the ether-type of outer tag on trunk from WEB.

16. ES4524D-FLF-38-00281 -- QinQ: After we set the ether-type of outer tag=9100 on some trunk from CLI, the ether-type of all non-trunk ports be changed.

17. ES4524D-FLF-38-00284 -- TACACSAcct:Web TACACS server settings doesn't work.

18. ES4524D-FLF-38-00287 -- rfc_2021.c include non-necessary cli_mgr.h which only needed when SYS_CPNT_3COM_CLI is TRUE

19. ES4524D-FLF-38-00288 -- 802.1X:DUT can't send out EAP Request MD5-Challenge packet.

20. ES4524D-FLF-38-00289 -- RootGuard:RootGuard doesn't have associated pages on the web.

21. ES4524D-FLF-38-00290 -- SSHv2:DUT can't login to DUT as admin privilege with RSA/DSA private key.

V1.1.0.2

1. ES4524D-FLF-38-00055 CoS: CLI has no Hybrid mode command, but Feature List claims to support.

2. ES4524D-FLF-38-00190 ACL: (Suggestion) CLI had better note that <cr> only apply to <600-ffff> for Ether Type.

3. ES4524D-FLF-38-00212 CoS: The failed mesg needs be modified when CLI changes bandwidths of the trunk without any port membmers.

4. ES4524D-FLF-38-00289 RootGuard:RootGuard doesn't have associated pages on the web.

5. ES4524D-FLF-38-00291 802.1x: the 802.1x with TTLS authentication function can't work.

6. ES4524D-FLF-38-00292 802.1X: the 802.1x with PEAP authentication can't work.

7. ES4524D-FLF-38-00293 802.1X : the 802.1X with TLS authentication can't work.

8. ES4524D-FLF-38-00296 TacacsAcct: the exception happen on DUT with tacacs + accounting.

9. ES4524D-FLF-38-00297 IP Source Guard: After we save running configuration and rebooting, DUT loses the static entry.

10. ES4524D-FLF-38-00298 IP Source Guard: When we create static entries over maximum value, DUT does not reject the setting.

11. ES4524D-FLF-38-00299 IP Source Guard: After we inject illegal packets, DUT learn these addresses.

12. ES4524D-FLF-38-00300 MR2228: no timestamp on the web page log.

V1.1.0.3

1. ES4548D-FLF-38-00001 Setting lost BUG? SS2GR48i - PVLAN - Web

V1.1.0.4

1. ES4548D-FLF-38-00001 Setting lost BUG? SS2GR48i - PVLAN - Web

V1.1.0.5

1. ES4524D-FLF-38-00305 Customer Technical Escalation form from Edge-Core website

V1.1.0.6

1. ES4524D-FLF-38-00306 "switchport mode private-vlan" command should be removed

V1.1.0.7

1. ES4524D-FLF-38-00304 projx000755 ,Accton Doc:60-2007-09-03,ESC537820 must set the security value to "3" on the edge port for 1 valid end user

2. ES4524D-FLF-38-00309 [RSPAN] Add RSPAN feature.

V1.1.0.8

| 1 | ES4524D-FLF-38-00308 | ES4524D Current Time and Time Zone show web page can't found. |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00310 | [network-access] Porting network-access aging. |

V1.1.0.9

| 1 | ES4524D-FLF-38-00311 | [Mirror] Can not monitor tagged packets when source port and destination port are at different chips(cross chip) |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00312 | [RSPAN] It does not work correctly if config 2 sessions. |
| 3 | ES4524D-FLF-38-00315 | SNMP v3 will not response after clock timezone configuration |

V1.1.0.10

| 1 | ES4524D-FLF-38-00318 | ES4524D-Configuration of trunk ports via WEB interface is failing |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00319 | Flowcontrol doesn't work correctly on ES4524D |

V1.1.0.11

| 1 | ES4524D-FLF-38-00314 | [Enhancement] Increase username and password length to 16 |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00316 | Management ip filter does not support SSH and HTTPS |
| 3 | ES4524D-FLF-38-00317 | Too few characters in a username and a password on ES4524D |
| 4 | ES4524D-FLF-38-00320 | There is moemory leakage when do SNMP walk |

V1.1.0.12, V1.2.0.0

| 1 | ES4524D-FLF-38-00313 | [Web, SNMP, Counter] statistics doesn't be cleared on Web GUI/SNMP for ES4524D |
| --- | --- | --- |

V1.2.0.1

| 1 | ES4524D-FLF-38-00330 | Port-channel statistics could not be recorded on ES4524D |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00322 | MIB file got compile error when using MG-SOFT |
| 3 | ES4524D-FLF-38-00331 | syslog bug(System time is updated by sntp) |
| 4 | ES4524D-FLF-38-00324 | ES4524D can not use port channel as downlink when configuring pvlan |

V1.2.0.2

| 1 | ES4524D-FLF-38-00329 | slow response of CLI when telnet |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00336 | Problem of counter statistics on ES4524D |

V1.2.0.3

| 1 | ES4524D-FLF-38-00327 | Port secruity issue in Web page |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00328 | "Static IGMP Member Port" could be set valid through the Web but can't be displayed |
| 3 | ES4524D-FLF-38-00339 | slow response of CLI issue got to improve |

V1.2.0.4

| 1 | ES4524D-FLF-38-00342 | Management freeze caused by MVR (only WEB interface) |
| --- | --- | --- |

V1.2.0.5

| 1 | ES4524D-FLF-38-00344 | 802.1x:The display of "Encryption Key" page on Web UI is not correct. |
| --- | --- | --- |
| 2 | ES4524D-FLF-38-00345 | 802.1x:The "Secret Text String" just can fit 27 characters on Web UI. |
| 3 | ES4524D-FLF-38-00346 | TACACSAcct:When set tacacs server key is equal or over 37 characters,save and reboot the DUT,the configuration lost. |
| 4 | ES4524D-FLF-38-00347 | TACACSAcct:Set an invalid ip to tacacs server,it displays error message but it can set successfully. |
| 5 | ES4524D-FLF-38-00348 | 802.1x:There is no any command for setting "Supplicant timeout" either on CLI or Web |
| 6 | ES4524D-FLF-38-00350 | IPSrcGuard: binding cannot be configured in Firefox/Google Chrome |
| 7 | ES4524D-FLF-38-00351 | IPSrcGuard: The total count of static bindings is incorrect when using Firefox/Chrome |
| 8 | ES4524D-FLF-38-00353 | IPSrcGuard: VLAN tagged frames are not forwarded even though the static configuration is set for the correct VLAN |
| 9 | ES4524D-FLF-38-00355 | 802.1x:The DUT cannot handle the port control when the 802.1x client is used TLS |
| 10 | ES4524D-FLF-38-00360 | IPSrcGrd: DHCP packets are not forwarded when IPSrcGd is enabled |
| 11 | ES4524D-FLF-38-00361 | PortSecurity: ports can be configured so that both LACP and portsec are enabled |
| 12 | ES4524D-FLF-38-00364 | DHCPSnoopingOption82:In DHCP Discover and request packets, it has wrong vendor class identifier "ES3528". |
| 13 | ES4524D-FLF-38-00368 | TACACSAcct: Can not remove method on AAA with service request is 'command' in CLI. |
| 14 | ES4524D-FLF-38-00371 | MVR:It can't access DUT from PC after creating mvr group and launching group member configuration page |
| 15 | ES4524D-FLF-38-00373 | MVR: MVR command description is incorrect |
| 16 | ES4524D-FLF-38-00376 | MVR:It should display which port fail to set immediate leave |
| 17 | ES4524D-FLF-38-00378 | RSPAN:It shouldn't allow to create port monitor when 2 RSPAN session are created in web |
| 18 | ES4524D-FLF-38-00381 | MVR:MVR must be not able to enable when igmp snooping did not enable |
| 19 | ES4524D-FLF-38-00382 | MVR:It always display IGMPSNP,not MVR when acting MVR function |
| 20 | ES4524D-FLF-38-00383 | DHCPSnooping:DHCP snooping binding entry can't be displayed on web. |
| 21 | ES4524D-FLF-38-00384 | recommended changes to grammar/formatting |
| 22 | ES4524D-FLF-38-00387 | RSPAN:It will forward packets to destination port when enable Port security |
| 23 | ES4524D-FLF-38-00388 | IGMPSNP:DUT can't join multicast group after enable MVR, and then disable MVR |
| 24 | ES4524D-FLF-38-00390 | LACP:Click "Remove" button from "LACP Configuration" page on web, all ports will disappear. |
| 25 | ES4524D-FLF-38-00391 | Web:It should exclude RSPAN when selecting VLAN static table by web |
| 26 | ES4524D-FLF-38-00394 | SNTP:It can set an invalid SNTP server ip. |
| 27 | ES4524D-FLF-38-00395 | RSTP:The DUT cannot display the correct STA state. |
| 28 | ES4524D-FLF-38-00396 | please remove renumbering from the web, it's already been removed from cli |
| 29 | ES4524D-FLF-38-00398 | Leave Proxy should be added to the IGMP Configuration page. |
| 30 | ES4524D-FLF-38-00399 | Time configuration options supported in web are not yet implemented in cli. |
| 31 | ES4524D-FLF-38-00400 | Missing new attributes,"Retransmit Times & Request Times", for show tacacs-server. |
| 32 | ES4524D-FLF-38-00401 | SNTP:"SNTP Configuration" page fail. |
| 33 | ES4524D-FLF-38-00413 | IGMPv3Snooping:It can't filter multicast packets with specific source ip. |
| 34 | ES4524D-FLF-38-00414 | RSTP:The DUT cannot configure the Maximum value of Hello Time & Maximum Age & Forward Delay. |
| 35 | ES4524D-FLF-38-00418 | Cluster: the webUI will return 404 error when cluster member includes 3rd party machines |
| 36 | ES4524D-FLF-38-00434 | Remove the UI of IGMPv3 Snooping |

modifications:

V0.0.0.4

1.Change SNTP supported server to 3 servers.

2.Change Trunk Group support to 32 groups.

3.Change Spanning Tree Root Guard support to project phase II.

V0.0.0.5

1.Change Clustering Members support to 36 members.

2.Change Trunk Group support to 24 groups.

3.Change Port Mirror session to 1 and support one port monitor multiports.

V0.0.0.6

N/A

V1.0.0.0

N/A

V1.0.0.3

1. Add Spanning Tree Root Guard

2. Add DHCP Snooping

3. Add DHCP Option 82

4. Add 802.1x TLS,TTLS, PEAP Encryption supported

5. Add TACACS+ (AAA)

V1.1.0.1

1. IGMP Immediate Leave

2. IGMPV3 Snooping

3. QinQ

4. LLDP

5. Stacking Hot Swap

V1.1.0.2

1. 4K vlan is now supported.

2. LLDP is now corrected.

V1.1.0.7

1. RSPAN is now supported. Except VLAN filter.

V1.1.0.8

1. Add “network-access aging”

V1.1.0.11

1. Increase username and password length to 16

V1.2.0.5

1. This product does not support IGMPv3 snooping.

Known Bugs and Issues:

V1.1.0.2

1. ES4524D-FLF-38-00013 Modulization is not effiecency for clustering cause compile error for projects who have no clustering supported.

2. ES4524D-FLF-38-00016 [QinQ] EIT test fail: 66.7

3. ES4524D-FLF-38-00077 ICMP MIB: Ping to unknown IP from DUT, then get object and verify it , the value doesn't increment.

4. ES4524D-FLF-38-00084 SNMP: The TCP MIB object tcpEstabResets/tcpRetransSegs have not increase value.

5. ES4524D-FLF-38-00100 L2 switch: the status of flow control type show as "none" when enable flow control both on DUT and IXIA at half mode.

6. ES4524D-FLF-38-00117 IPv6-ANVL-NDP: ANVL test failed

7. ES4524D-FLF-38-00128 IPv6-ANVL-NDP: ANVL test failed

8. ES4524D-FLF-38-00149 IPv6 SSH: DUT doesn't response “neighbor advertisement” when SSH session run few hours .

9. ES4524D-FLF-38-00151 ManytoOne:The performance test of many to one had packet loss in part of frame size the rate about 0.3%-100%.

10. ES4524D-FLF-38-00152 PartiallyMeshed:The DUT occurs packet lost 100% with the all port's speed/deplex and frame size 8192/9216.

11. ES4524D-FLF-38-00153 FullyMeshed:The DUT occurs packet lost 5%-35% with the port's speed/duplex 10F/100F/1G and frame size 1024-9216.

12. ES4524D-FLF-38-00155 CoreCustomer: packets would flood when change the spanning bridge priority on system test

13. ES4524D-FLF-38-00167 CoreCustomer: Packets would flood in the system test.

14. ES4524D-FLF-38-00168 AdvCustomer: Packets would flood when reboot DUT on the system test

15. ES4524D-FLF-38-00169 AdvCustomer: Sometimes packets would flood when topology change on the system enviroment .

16. ES4524D-FLF-38-00173 AdvCustomer: SFP port would shudown automatically in the system test

17. ES4524D-FLF-38-00178 AdvCustomer:Unicast packets would flood in the system test

18. ES4524D-FLF-38-00184 SMTP: when I configure "@" only for sendmail , the DUT has be set.

19. ES4524D-FLF-38-00186 DiffservMIB: The named of the Class-map and Policy-map could be set when I configure named include space, example "P P".

20. ES4524D-FLF-38-00197 AdvCustomer: Packets would flood when GVRP and MSTP work together .

21. ES4524D-FLF-38-00198 IPv6: Find the root cause of ES4524D-FLF-38-00131(temporary patch)

22. ES4524D-FLF-38-00199 LACP: Sometimes DUT can't join LACP port-channel to the vlan tagged member when the port mode set to Q-trunk .

23. ES4524D-FLF-38-00203 WEB: Help page shall modify description from WEB browser.

24. ES4524D-FLF-38-00209 Cluster: Cluster member would hang up when create MAX cluster member on commander .

25. ES4524D-FLF-38-00214 IPv6 SNMP: DUT does not send out TRAP/INFORM messages through IPv6 when warm-start/cold-start it.

26. ES4524D-FLF-38-00216 IPv6 address: DUT shall modify description of help from CLI.

27. ES4524D-FLF-38-00217 IPv6 address: IPv6 Prefix-length "0" does not work.

28. ES4524D-FLF-38-00231 Diffserv: MIB Object "ClassMapAttachCtlAction" & "AclAttachCtlAction" are changed values when setting them with wrong way.

29. ES4524D-FLF-38-00236 IPv6: When user configure invalid IPv6 addr by CLI/WEB/MIB, it shall be filtered.

30. ES4524D-FLF-38-00239 PortTrunk: Received broadcast pacekts are changed when Trunk enables both Storm & Ingress Rate Limit for DUT.

31. ES4524D-FLF-38-00243 AdvCustomer: GVRP can't learn Vlan Correctly .

32. ES4524D-FLF-38-00244 AdvCustomer: DUT would SNMP error message

33. ES4524D-FLF-38-00273 SSHv2:Web show success when user download the RSA private key to DUT.

34. ES4524D-FLF-38-00294 QinQ: We cannot use CLI to set the ether-type in global mode.

35. ES4524D-FLF-38-00295 QinQ: The ether-type command should be removed from interface.

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

1. When configuring IPv6 EXT QoS and RATE-LIMIT at same time, the RATE-LIMIT will be not work.

2. QinQ will not support with IGMP snooping at same time.

3. Due to this ASIC do not support “Per port Enable/Disable QinQ”, when issue command “No switchport mode” , the port will actually still remain as “Access mode”.

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2006 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |
| IEEE 802.1X | Port-based Network Access Control |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1907 | SNMPv2-MIB (MIB II) | - system group ( 1-9) - snmp group (1-6, 8-22, 24-32) |
| 2011 | IP-MIB (MIB-II) | - ip group (1-20, 22-23) - icmp group (1-26) |
| 2012 | TCP-MIB (MIB-II) | - tcp group (1-15)(tcpConnTable only support read.) |
| 2013 | UDP-MIB (MIB-II) | - udp group (1-5) |
| IEEE 802.1X MIB | IEEE8021-PAE-MIB | - dot1xPaeSystem (1-2) - dot1xPaeAuthenticator (1-3)(not support per-port timer setting) |
| 1493 | Bridge MIB | - dot1dBase group (1-4) - dot1dStp group (1-15) - dot1dTp group (1-4) - dot1dStatic group (1) |
| 2863 | Interfaces Evolution MIB (IF-MIB) | - interface group (1-2) (ifadminStatus not support set to 3(testing)) - ifXTable group (1) - ifStackTable group (1)(support read-only) |
| 2819 | RMON MIB | - statistics group (1) - history group (1-2) - alarm group (1) - event group (1-2) |
| 2618 | RADIUS MIB | - radiusAuthClientMIB (1) |
| 2665 | Etherlike MIB | - dot3StatsTable group (1-19) |
| 2737 | Entity MIB | - entityPhysical group (1-16) (support read-only) |
| 2674 | P-bridge | - dot1dExtBase group (1-4) - dot1dPriority group (1,3) - dot1dGarp group (1) |
| 2674 | Q-bridge | - dot1qBase group (1-5) - dot1qTp group (1-2) - dot1qStatic group(1) - dot1qVlan (1-5) (9-10) |
| V-Bridge MIB | V-Bridge MIB | - dot1vProtocol group |
| 3636 | MAU MIB | - dot3IfMauBasicGroup - dot3IfMauAutoNegGroup |

| 1612 | DNS Reslover MIB | - dnsServConfig - dnsServCounter - dnsServCounterAuthAns - dnsServCounterAuthNoNames - dnsServCounterAuthNoDataResps - dnsServCounterNonAuthDatas - dnsServCounterNonAuthNoDatas - dnsServCounterReferrals - dnsServCounterErrors - dnsServCounterRelNames - dnsServCounterReqRefusals - dnsServCounterReqUnparses - dnsServCounterOtherErrors - dnsServOptCounter - dnsResMIBObjects - dnsResConfig - dnsResCounter - dnsResCache - dnsResCacheStatus - dnsResCacheMaxTTL - dnsResCacheGoodCaches - dnsResCacheBadCaches - dnsResOptCounter |
| --- | --- | --- |
| 3411 | SNMP FrameWork | - snmpFrameworkMIBObjects |
| 3412 | SNMP MPD MIB | - snmpMPDMIBObjects |
| 3413 | SNMP Target MIB | - snmpTargetObjects |
| 3413 | SNMP Notify MIB | - SnmpNotifyObjects - snmpNotifyTable - usmMIBObjects |
| 3415 | SNMP View-Based ACM MIB | - vacmMIBObjects - Private MIB - switchMgt - portMgt - trunkMgt - lacpMgt - staMgt - tftpMgt - restartMgt - mirrorMgt - igmpSnoopMgt - ipMgt - bcastStormMgt - vlanMgt - priorityMgt - trapDestMgt - qosMgt - securityMgt - sysLogMgt - lineMgt |

ES4524D/ES4548D Private Enterprise MIB Support:

| Title | #### Version |
| --- | --- |
| ES4524D.mib | 20061206001 |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | ColdStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| RFC 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| RFC 2863 | linkDown (v2c) linkUp (v2c) |
| RFC 1493 | newRoot (v1) topologyChange (v1) |
| RFC 1757 | risingAlarm (v1) fallingAlarm (v1) |
| RFC 2819 | risingAlarm (v2c) fallingAlarm (v2c) |

Customer Support:

By Phone:

By Email:

By Web:

By Fax: