ES4612-38

Firmware Version 1.0.6.5

INTRODUCTION:

ES4612-38 is a switch containing defines 12-port Gigabit Ethernet Layer 2/3/4 Switch. The switch provides 8 slot-in minigbic GE ports and 4 combo ports (minigbic and 10/100/1000 TX combo).

The base hardware is a 1U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common Http browsers such as Netscape Navigator or Microsoft’s Internet Explorer (both browsers should be Version 6.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the ES4612-38 from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release note prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.0.6.5 | Customer | 2009/10/23 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Category | Feature | Support |
| --- | --- | --- |
| Monitor Environment | Power Status | Yes |
| Switching Features |  |  |
| VLAN | 802.1Q – Tag-based | Yes |
|  | 802.1Q – Port-based | Yes |
|  | 802.1Q – Remove Tag | Yes |
|  | 802.1V – Protocol VLAN | Yes |
|  | 255 VLANs per switch | Yes |
|  | GVRP(127+ VLANs per switch) | Yes |
| Priority/ CoS | L2: 802.1p – Tag-based | Yes |
|  | L2: 802.1p – Port-based | Yes |
|  | L2: 802.1p – Queuing | 8 queues per port |
|  | L3 Aware: IP TOS | Yes |
|  | IP Precedence |  |
|  | IP DSCP | Yes |
|  | WRR (Weighted Round Robin Queueing) | Yes |
| Link Aggregation | 802.3ad (LACP) | Yes |
|  | Cisco EtherChannel (Static) | Yes |
| Port Mirroring | Port Mirroring | Yes |
| Rate Limiting | Ingress/Egress (Configuration per port) | Yes |
| Multicast Support | IGMP Snooping | Yes |
|  | Broadcast Storm Control | Yes |
| Spanning Tree | Spanning Tree Protocol (STP, IEEE 802.1D) | Yes |
|  | Rapid Spanning Tree Protocol (RSTP, IEEE 802.1w) | Yes |
|  | Multiple Spanning Tree Protocol (MSTP, IEEE 802.1s) | Yes |
| Routing Features |  |  |
| Multicast Routing | DVMRP (Distance-Vector Multicast Routing Protocol) | Yes |
|  | PIM DM (Protocol-Independent Multicasting Dense Mode) | Yes |
|  | IGMP (Internet Group Management Protocol) | Yes |
| IP Routing | ARP, Proxy ARP | Yes |
|  | Static Routes | Yes |
|  | RIP V1/V2 (Routing Information Protocol V1/V2) | Yes |
|  | OSPF v2 (Open Shortest Path First Version 2) | Yes |
|  | VRRP (Virtual Router Redundancy Protocol) | Yes |
| Additional Feature | CIDR (Classless Inter-Domain Routing) | Yes |
| System Software |  |  |
| IP Address Management | Static | Yes |
|  | BootP | Yes |
|  | DHCP client / relay / server | Yes |
| Configuration Management | Configuration File | Yes |
|  | Firmware upgrade | Yes |
|  | Dual firmware images | Yes |
| Access Control | Various privileges | Yes |
|  | Remote authentication & authorization – RADIUS | Yes |
|  | Remote authentication & authorization – TACACS | Yes |
|  | 802.1x Port based | Yes |
|  | HTTPS | Yes |
|  | SSH 2.0 | Yes |
| Web Management | Switch Front Panel View | Yes |
|  | Look & Feel | Yes |
|  | Support Secure socket layer | Yes |
|  | WEB enhanced counters and utilization counters | Yes |
| Character-Based Management | Cisco-like CLI via Telnet and Serial Port | Yes |
| Logging | Event logging | Yes |
|  | Command line activity | Yes |
|  | Remote syslog(RFC-3164) | Yes |
| SNMP Management | SNMP v3 | Yes |
|  | MIB support | Yes |
|  | Private MIB | Yes |
|  | Alerts | Yes |
|  | SNMP access IP address filtering | Yes |
| Port Security | Mac Based Port Security | Yes |
|  | Intrusion action shutdown and Trap | Yes |
| MAU MIB | MAU MIB | Yes |
| DNS Server | DNS Server | Yes |
| SNTP | Simple Network Time Protocol | Yes |
| SMTP | Mail Alerts | Yes |
| 3rd party management applications | HP OpenView for Windows | Yes |

Installation and Configuration Notes:

In general, the ES4612-38 will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ES4612-38, please follow the TFTP download instructions that are included with your firmware image upgrade kit. TFTP download instructions are also available on the Ericsson Support web site at: www.edge-core.com

Firmware Changes and Enhancements:

The following issues are the enhancement in this release of firmware.

Loader

V2.3.0.0 - Change CAS Latency from 2 to 3. - Change the Refresh Interval to 920.

V2.3.0.1 - Support auto baud rate.

Diagnostic Code

V2.2.0.0 - Add a Resume Post Command in CLI.

V2.2.0.1 - Support auto baud rate.

Runtime Code

V1.0.2.26 - A SSH issue. When upload RSA 768 or 1024 key size to DUT from TFTP server, the DUT cannot write to flash programming.

V1.0.2.31 - A MSTP issue. After show spanning-tree mst configuration, the instances information is error. - A PIM-DM issue. IGMP Join Report Processing is error.

V1.0.2.32 1. Change the value of the max number of primary subnet from 32 to 256.

V1.0.2.33 1. ES4612-38-00064 --- RIP:save and reboot the DUT will loss rip entry.

V1.0.2.35 1. ES4612-38-00065 --- ES4612 IGMP Snooping unknown Mcast-packet flooding issue

V1.0.2.36 1. ES4612-FM-00007 --- MR3312 Mask ACL problem with duplicate rules

V1.0.2.37 1. ES4612-FM-00008 --- 10.0.0.0 route problem

V1.0.2.38 - ES4612-FM-00011 --- MR3312 lost snmp view - ES4612-38-00066 --- RMON problem

V1.0.2.39 - ES4612-FM-00009 --- ospf ospf dead interval disappear prpblem

V1.0.2.40 - ES4612-FM-00010 --- PIM-DM problem when route table changes to source IP in mroute table

V1.0.2.41 - Enlarge multicast group entry from 64 upto 255

V1.0.2.42 - ES4612-38-00068 --- 4 OSPF SNMP problem

V1.0.2.43 - ES4612-38-00069 --- switch can’t 4ecognize leave packet with Source ip =0.0.0.0

V1.0.2.44 - ES4612-38-00070 --- Web UI should check password length

V1.0.2.45 - ES4612-20-00335 --- 20-2007-06-04-Esn410 loss IP management interface - ES4612-20-00336 --- Snmp host and snmp set will cause the ping fail

V1.0.2.46 - ES4612-38-00071 --- ES4612 port channel configure queue bandwidth

V1.0.2.47 (Fail) - Support Diffsrv

V1.0.2.48 - Support Diffsrv

V1.0.2.49 - ES4612-38-00072 --- the message showing in the console and telnet is different - ES4612-38-00073 --- ES4612 ACL configuration question - ES4612-38-00074 --- ES4612C: IGMP Joins are not handled correctly (Tagged VLAN, PIM DM)

V1.0.2.50 - ES4612-38-00075 --- ES4612: ACL with Priority and DSCP cannot be set on port.

V1.0.2.51 - ES4612-38-00078 --- obtaining IP-MAC pairs from not all VLANs by SNMP 0n ES4612 priority low - ES4612-38-00080 --- ES4612 : GVRP is not working properly when trunk port is set as acceptable-frame-type tagged - ES4612-FM-00012 --- MR3312 SNMP ospfAreaLsaChksumSum is sometimes not correct

V1.0.2.52 - ES4612-38-00082 --- Switch will not send trap with F/W v1.0.2.50

V1.0.2.53 - ES4612-38-00086 --- Fail to add/delete an arp table entry via snmp - ES4612-38-00087 --- Increase quantity of multicast forwarding entry to 255

V1.0.2.54 - ES4612-FM-00013 --- ES4612-FM-00013: MR3312 SNMPv3 trap problem

V1.0.2.55 - ES4612-38-00088 --- When setting ACL out whether MAC ACL or IP standard ACL, It’s doesn’t work in v1.0.2.54 - ES4612-38-00090 --- If creating 255 VLANs and assign IP to every VLAN, the 170th VLAN IP will fail when assign - ES4612-38-00091 --- Policy-maps don’t work normally

V1.0.2.56 - ES4612-38-00096 --- Problem of LACP

V1.0.2.57 - ES4612-38-00095 --- PC & switch can’t ping each other after nessus scanning a period of time. - ES4612-38-00098 --- MR3312(ES4612-38) jumbo frames fail on management problem!

V1.0.5.1 - ES4612-17-00078 --- New feature: support Multicast VLAN Registration (MVR) protocol. - ES4612-38-00097 --- New feature: support OSPF – redistribute connected.

V1.0.5.2 (Formal Release: tested by NTC, conditional passed by Brand) - ES4612-38-00109 --- MVR: The ports which send IGMP report packets did not join MVR VLAN when enable MVR function, and also did not receive multicast data. - ES4612-38-00110 --- MVR: DUT shows the wrong type when the join packet group was defined by MVR. - ES4612-38-00111 --- MVR: DUT can not show the group status by Web UI. - ES4612-38-00112 --- MVR: DUT can not join the port to group when this port is a MVR source port. - ES4612-38-00113 --- MVR: Multicast table and MVR member table can not clear when client pc has send leave packet. - ES4612-38-00114 --- MVR: The join client will leave and stop to receive the multicast packets automatically.

V1.0.6.0 (Formal Release: tested by NTC, conditional passed by Brand) - Only version number is changed, this version doesn’t have any modification and resolved EPR.

V1.0.6.1 - ES4612-38-00115 --- MST VLAN map problem. - ES4612-38-00116 --- Fails to set PVLAN in aggregation port. - ES4612-38-00118 --- Enhancement: support 4K VLAN. - ES4612-38-00119 --- Enhancement: extend the configurable MVR group to 1024.

V1.0.6.2 - ES4612-38-00120 --- MAC address table cannot display the learned addresses of last trunk port - ES4612-17-00119 --- Enhancement: new feature support – VLAN Trunking - ES4612-17-00120 --- Enhancement: new feature support – EAPS - ES4612-17-00121 --- Enhancement: extend the login password length to 32 bytes - ES4612-17-00122 --- Enhancement: support multiple RADIUS servers (maximum 5 server)

V1.0.6.3 - ES4612-38-00121 --- SNMP is slowing down while reading a lot of MACs - ES4612-38-00122 --- Can’t set aclAclGroupEgressIpAcl

V1.0.6.4 - ES4612-38-00127 --- MSTP priority problem - ES4612-38-00128 --- Bad log message with 2K VLAN

V1.0.6.5 - ES4612-38-00123 --- SNMP walk problem, can't SNMP walk 6.10.57.1.44 (MVR MIB)

modifications:

Known Restrictions and Limitations:

Those issues are under debugging as follows:

Any other problems than those listed above should be reported to our Technical Support Staff.

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2005 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1s | Multiple Spanning Tree |
| IEEE 802.1v | Protocol VLAN |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.1x | Port Authenication |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1213 | MIB-II | - system group - interfaces group - ip group - icmp group - tcp group - udp group - snmp group |
| 1493 | Bridge MIB | - dot1dBase group - dot1dStp group - dot1dTp group - dot1dStatic group |
| 2863 | Interfaces Evolution MIB | - ifXTable group - ifStackTable group |
| 2819 | RMON MIB | - statistics group - history group - alarm group - event group |
| 2618 | RADIUS MIB | - radiusAuthClientMIB |
| 2665 | Etherlike MIB | - dot3StatsTable group |
| 2737 | Entity MIB | - entityPhysical group |
| 2674 | P-bridge | - dot1dExtBase group - dot1dPriority group - dot1dGarp group |
| 2674 | Q-bridge | - dot1qBase group - dot1qTp group - dot1qStatic group - dot1qVlan |
| 2668 | MAU MIB | - dot3IfMauBasic group - dot3IfAutoNeg group |
| 2096 | IP Forwarding MIB |  |
| 1724 | RIPv2 MIB | - rip2Global group - rip2IfStat group - rip2IfConf group |
| 2932 | IPv4 Multicast Routing MIB | - ipMRouteTable group - ipMRouteNextHopTable group - ipMRouteInterfaceTable group |
| 2933 | Internet Group Management Protocol MIB | - igmpInterfaceTable group - igmpCacheTable group |
| 2934 | Protocol Independent Multicast MIB for IPv4 | - pimInterfaceTable group - pimNeighborTable group - pimIpMRouteTable group - pimIpMRouteNextHopTable group |
| 1850 | Open Shortest Path First | - ospfGeneralGroup - ospfAreaTable - ospfStubAreaTable - ospfLsdbTable - ospfIfTable - ospfIfMetricTable - ospfVirtIfTable - ospfNbrTable - ospfVirtNbrTable - ospfExtLsdbTable - ospfAreaAggregateTable |

rivate Enterprise MIB Support:

| Title | #### Version |
| --- | --- |
| ES4612-38.mib |  |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | coldStart warmStart linkDown linkUp authenticationFailure |
| RFC 1493 | newRoot topologyChange |
| RFC 2819 | risingAlarm fallingAlarm |