Edge-Core ES4700_ES4600 Series

6.1.N.0 Release note

Edge-Core ES4700_ES4600 series 6.1.N.0 release note

Abstract：This software release note includes all the detail information about version, version history, version change, bug fixes list, limitation and workaround.

- Index

Index	3

1.	Version Release Introduction	4

1.1	6.1 Version release introduction	4

1.1.1	6.1.40.0 Version release introduction	4

1.1.2	6.1.39.0 Version Release Introduction	5

1.1.3	Release date	5

1.2	6.0.207.0 Version release introduction	7

1.2.1	Release date	7

1.2.2	Version table	8

1.3	5.4.128.0 Version Release Introduction	11

1.3.1	Release date	11

1.3.2	Version table	11

1.4	5.4.123.0 Version Release Introduction	15

1.4.1	Release date	15

1.4.2	Version table	15

2.	Bug fixed table	19

2.1	6.1.N.0 Version bug fixed table	19

2.2	6.0.207.0 Version bug fixed table	20

2.3	5.4.128.0 Version bug fix list	20

2.4	5.4.123.0Version bug fix list	21

3.	Existing problem and workaround	23

4.	Version CHANGE NOTICE	23

4.1	hardware  and IMG functionCHANGE NOTICE	23

4.2	BootRom function CHANGE NOTICE	26

4.2.1	ES4700 seriesGCommon and  GZoma board card group  BootRom function CHANGE NOTICE	26

4.2.2	ES4624(26)-SFP series BootRom function CHANGE NOTICE	26

4.2.3	ES4626(50) series BootRom function CHANGE NOTICE	27

4.3	IMG Command line CHANGE NOTICE	27

4.4	BootRom Command line CHANGE NOTICE	30

4.4.1	ES4700 seriesGCommon and  GZoma board card group BootRom Command line CHANGE NOTICE	30

4.4.2	ES4624(26)-SFP seriesBootRom Command line CHANGE NOTICE	30

4.4.3	ES4626(50) seriesBootRom Command line CHANGE NOTICE	31

5.	Version function list	31

5.1	hardware function list	31

5.2	IMG function	32

5.3	BootRom function list	32

5.3.1	ES4700 seriesGCommon and  GZoma board card group  BootRom function Table	32

5.3.2	ES4624(26)-SFP series BootRom function list	33

5.3.3	ES4626(50) Serials BootRom Function List	33

6.	ES4700 series agent card module mapping list	34

# Version Release Introduction

## 6.1 Version release introduction

### 6.1.40.0 Version release introduction

#### Date Released

Date Released：19 March 2010

#### Version Mapping Table

##### ES4700 Serials Firmware Mapping Table

| Product Series | ES4700series |  |
| --- | --- | --- |
| Model Name | ES4704BD、ES4710BD |  |
| Files Released | File Name | Size（Bytes） |
|  | IMGfile |  |
|  | ES4704(10)BD_6.1.40.128_LITE_nos.img | 12,336,811 |
|  | BootRomfile |  |
|  | ES4704(10)BD_3.0.11_boot.rom | 1,537,328 |
|  | Vendor.cfgfile |  |
|  | ES4704(10)BD-3.0.1-vendor.cfg | 54,108 |
|  | PrivateMibfile |  |
|  | EdgeCore_ES4704_1.5.22.mib | 568,541 |
| Allocated BootRom | Must use（support vendor.cfg file and ECC function ）1.7.2 version or later’s BootRom， recommended to use （add ROM and IMGfile error-prevent upgrade function ）3.0.8’s or later BootRom。 |  |
| Note | 1. Since there is a change for MRPP’s multicast’s destination address destination address in version 6.1,so cannot use together with previous version。If there is any switch in network use Cluster and upgrade to version 6.1, whole network need to be upgrade. 2. In version 6.1，there is some change in cluster report format ,and cant not compatible to previous report ‘st format，so cannot use with previous version。If there is any switch in network use Cluster and upgrade to version 6.1, whole network need to be upgrade. 3. ES4700_version_nos.img file support GCommon、GWitch、GWitchSlave 、GHoust on board card’s group，these file include 4 small IMGfile。 4. ES4700_ version_LITE_nos.img file support GCommon and GWitch（main board card and board card ）board card group。This version mainly used by 128 MB flash memory (board card) ES4700 switch. 5. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

### 6.1.39.0 Version Release Introduction

### Release date

Release date：8 March 2010

#### Version Mapping Table

##### ES4624(26)-SFP Serials Firmware Mapping Table

| Product Series | ES4624(26)-SFPseries |  |
| --- | --- | --- |
| Model Name | ES4624-SFP、ES4626-SFP |  |
| Files Released | File Name | Size（Bytes） |
|  | IMGfile |  |
|  | ES4624(26)-SFP_6.1.39.0_nos.img | 7,633,285 |
|  | BootRomfile |  |
|  | ES4624(26)-SFP_2.0.11_boot.rom | 2,192,832 |
|  | ES4624(26)-SFP_2.0.11_config.rom | 491,888 |
|  | Vendor.cfgfile |  |
|  | ES4624(26)-SFP-1.1.0-vendor.cfg | 51,685 |
|  | PrivateMibfile |  |
|  | ES4624(26)-SFP_1.5.22.mib | 488,407 |
| Allocated BootRom | Must use（support vendor.cfg file and ECC function’s）version 1.6.2 or later’s BootRom，recommend to use version 1.6.5 or later ‘s BootRom（Add ROM and IMGfile upgrade-error prevent function）,recommend to use version 2.0.8。 |  |
| Note | 1. Since there is a change for MRPP’s multicast’s destination address destination address in version 6.1,so cannot use together with previous version。If there is any switch in network use Cluster and upgrade to version 6.1, whole network need to be upgrade. 2. In version 6.1，there is some change in cluster report format ,and cant not compatible to previous report ‘st format，so cannot use with previous version。If there is any switch in network use Cluster and upgrade to version 6.1, whole network need to be upgrade. 3. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features |  |

##### ES4626(50) Serials Firmware Mapping Table

| Product Series | ES4626(50)series |  |
| --- | --- | --- |
| Model Name | ES4626、ES4650 |  |
| Files Released | File Name | Size（Bytes） |
|  | IMGfile |  |
|  | ES4626(50)_6.1.72.0_nos.img | 7,632,313 |
|  | BootRomfile |  |
|  | ES4626(50)_2.0.11_boot.rom | 2,178,304 |
|  | ES4626(50)_2.0.11_config.rom | 404,464 |
|  | Vendor.cfgfile |  |
|  | ES4626(50)-1.1.0-vendor.cfg | 51,815 |
|  | PrivateMibfile |  |
|  | ES4626(50)_1.5.22.mib | 488,407 |
| AllocatedBootRom | Must use（support vendor.cfg file and ECC function ）1.2.0’s or later BootRom, recommended to use （add ROM and IMG file error-prevent upgrade function ）1.3.0’s or later BootRom, recommended to use 2.0.8 version 。 |  |
| Note | 1. Since there is a change for MRPP’s multicast’s destination address destination address in version 6.1,so cannot use together with previous version。If there is any switch in network use Cluster and upgrade to version 6.1, whole network need to be upgrade. 2. In version 6.1，there is some change in cluster report format ,and cant not compatible to previous report ‘st format，so cannot use with previous version。If there is any switch in network use Cluster and upgrade to version 6.1, whole network need to be upgrade.。 3. ES4626(50) -Lite version img file description ES4626(50)-Lite version img base on ES4626(50)but remove following module： CONFIG_OSPFV3_DISTANCE ENABLE_OSPFD ENABLE_BGPD ENABLE_LDPD ENABLE_PIMD ENABLE_DVMRPD ENABLE_IGMP ENABLE_IGMP_V3 ENABLE_IGMP_PROXY ENABLE_PIM_SSM ENABLE_PIMDM ENABLE_OSPF6D ENABLE_PIM6D ENABLE_PIMDM6 Note：Lite version not support vendor。 4. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features |  |

## 6.0.207.0 Version release introduction

### Release date

Release date ：24.Aug 2009

### Version table

#### ES4700 Serials Firmware Mapping Table

| Product Series | ES4700 series |  |
| --- | --- | --- |
| Model Name | ES4704BD、ES4710BD |  |
| Files released | File Name | Size（Bytes） |
|  | IMG file |  |
|  | ES4700_6.0.207.128_LITE_nos.img | 12,655,445 |
|  | BootRom file |  |
|  | ES4700_2.2.1_boot.rom | 1,353,385 |
|  | Vendor.cfg fiel |  |
|  | ACCTON-ES4704BD-1.0.2-vendor.cfg | 40, 180 |
|  | ACCTON-ES4710BD-1.0.2-vendor.cfg | 40, 180 |
|  | Private Mib file |  |
|  | EdgeCore_ES4704_1.4.27.mib | 488,987 |
|  | EdgeCore_ES4710-1.4.27.mib | 488,975 |
| BootRom combination | When exist hardware Version R 3.5 and higher Version’s board card，must use the 2.1.0 Version and higher Version’s BootRom。 when all board card ‘s hardware Version lower than R 3.5，please use previous 1.7.6 Version’s BootRom。 advise user who upgrade to this release ’s 2.2.1 Version’s BootRom, at least need to use 1.7.6 Version’s BOOTROM detail information reference to following ’s Note。 |  |
| Manual | 《ES4700 Chassis switch installation guide》Version 1.7.2 |  |
| Manual download |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. 2. chasis switch support latest’s R 3.5Version（check label on board card），to this Version’s hardware board card，not support IMG Version before 5.4.123.0 or Bootrom Version before 2.1.0. 3. For new purchase’s racked switch board card, if hardware Version equal or greater than R 3.5，and main-control board card hardware Version lower than R3.5, main control board card’s file need to be upgraded first: IMG Version equal or greater than 5.4.123.0，Bootrom Version equal or greater than 2.1.0。Insert board card after reset the switch。 4. After IMG upgraded to 5.4.123.0 and higher Version，please do not downgrade IMG to 5.4.109.0 or lower Version, system may cannot boot up properly. This operation should be guided by technician. |  |

#### ES4624(26)-SFP Serials Firmware Mapping Table

| Product Series | ES4624(26)-SFP series |  |
| --- | --- | --- |
| Model Name | ES4624-SFP ES4626-SFP |  |
| Files Released | File Name | Size（Bytes） |
|  | IMG file |  |
|  | ES4624(26)-SFP_6.0.207.0_nos.img | 7,482,972 |
|  | BootRom file |  |
|  | ES4624(26)-SFP_1.7.0_boot.rom | 2,548,764 |
|  | ES4624(26)-SFP_1.7.0_config.rom | 514,320 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4624-SFP-1.0.2-vendor.cfg | 37, 810 |
|  | ACCTON-ES4626-SFP-1.0.2-vendor.cfg | 37, 810 |
|  | Private Mib file |  |
|  | ES4626-SFP-1.2.7.mib | 489,015 |
| BootRom in combination | Must use the（supportvendor.cfg file and ECC function）1.6.2 Version and higher Version’s BootRom，it is recommend to use（add ROM and IMG file Upgrade on error-preventive）1.6.5Version and higher Version’s BootRom。 |  |
| Manual in combination | 《ES4624(26)-SFP switch installation guide》Version 1.7.2 |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

#### ES4626(50) Serials Firmware Mapping Table

| Product Series | ES4626(50) series |  |
| --- | --- | --- |
| Model Name | ES4626 ES4650 |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES4626(50)_6.0.207.0_nos.img | 7,465,716 |
|  | BootRom file |  |
|  | ES4626(50)_1.5.1_boot.rom | 2,421,508 |
|  | ES4626(50)_1.5.1_config.rom | 367,528 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4626-1.0.2-vendor.cfg | 37, 810 |
|  | ACCTON-ES4650-1.0.2-vendor.cfg | 37, 810 |
|  | Private Mib file |  |
|  | ES4650-1.4.27.mib | 488,997 |
| BootRom in combination | Must use the（support vendor.cfg file and ECC function）1.2.0 Version and higherVersion’s BootRom，recommend to use（add ROM and IMG file support upgrade on error-preventive）1.3.0Version and higher Version’s BootRom。 |  |
| Manual in combination | 《ES4626(50) switch installation guide》Version1.7.2 |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

## 5.4.128.0 Version Release Introduction

### Release date

Release date：25 August 2008

### Version table

#### ES4700 Serials Firmware Mapping Table

| Product Series | ES4700 series |  |
| --- | --- | --- |
| Model Name | ES4704BD、ES4710BD |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES4700_5.4.128.0_nos.img | 11,180,314 |
|  | BootRom file |  |
|  | ES4700_2.1.0_boot.rom | 1,335,765 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4704BD-1.0.0-vendor.cfg | 42, 788 |
|  | ACCTON-ES4710BD-1.0.0-vendor.cfg | 42, 788 |
|  | Private Mib file |  |
|  | EdgeCore_ES4704-1.2.3.mib | 322, 977 |
|  | EdgeCore_ES4710-1.2.3.mib | 322, 960 |
| BootRom in combination | existed hardware Version R 3.5 and higher Version’s board card，Must use the 2.1.0 Version and higher Version BootRom。 when all board card ‘s hardware Version lower than R 3.5， please use previous ’s 1.7.2 or .1.7.6Version’s BootRom。 detail information reference to following ’s Note。 |  |
| Manual in combination | 《ES4700 switch installation guide》Version1.5.5 |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. 2. chasis switch support latest’s R 3.5 Version（check label on board card），to this Version’s hardware board card，not support IMG Version before 5.4.123.0 or Bootrom Version before 2.1.0 3. For new purchase’s racked switch board card, if hardware Version equal or greater than R 3.5，and main-control board card hardware Version lower than R3.5, main control board card’s file need to be upgraded first: IMG Version equal or greater than 5.4.123.0，Bootrom Version equal or greater than 2.1.0。Inset board card after reset the switch。 4. After IMG upgraded to 5.4.123.0 and higher Version，please do not downgrade IMG to 5.4.109.0 or lower Version, system may cannot bootup properly. This operation should be guided by technician. |  |

#### ES4624(26)-SFP Serials Firmware Mapping Table

| Product Series | ES4624(26)-SFP series |  |
| --- | --- | --- |
| Model Name | ES4624-SFP ES4626-SFP |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES4624(26)-SFP_5.4.128.0_nos.img | 5,992,829 |
|  | BootRom file |  |
|  | ES4624(26)-SFP_1.6.5_boot.rom | 2, 063, 860 |
|  | ES4624(26)-SFP_1.6.5_config.rom | 499, 032 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4624-SFP-1.0.0-vendor.cfg | 40, 418 |
|  | ACCTON-ES4626-SFP-1.0.0-vendor.cfg | 40, 418 |
|  | Private Mib file |  |
|  | ES4626-SFP-1.2.3.mib | 322, 986 |
| BootRom in combination | Must use the（supportvendor.cfg file and ECC function）1.6.2 Version and higher Version’s BootRom，it is recommend to use（add ROM and IMG file Upgrade on error-preventive）1.6.5Version and higher Version’s BootRom。 |  |
| Manual in combination | 《ES4624(26)-SFP switch installation guide》Version1.5.5 |  |
| Manual download fromo |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

#### ES4626(50) Serials Firmware Mapping Table

| Product Series | ES4626(50) series |  |
| --- | --- | --- |
| Model Name | ES4626（MRS3326-S4C、GigaBee-X24）、 ES4650（MRS3350-S4C） |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES4626(50)_5.4.128.0_nos.img | 5,974,962 |
|  | BootRom file |  |
|  | ES4626(50)_1.3.0_boot.rom | 1, 935, 772 |
|  | ES4626(50)_1.3.0_config.rom | 352, 908 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4626-1.0.0-vendor.cfg MRV-MRS3326-S4C-1.0.3-vendor.cfg NTT-GigaBee-X24MK-II-1.0.2-vendor.cfg | 40, 418 26, 236 36, 572 |
|  | ACCTON-ES4650-1.0.0-vendor.cfg MRV-MRS3350-S4C-1.0.5-vendor.cfg | 40, 418 26, 297 |
|  | Private Mib file |  |
|  | ES4650-1.2.3.mib GigaBee-X24MK-II-1.2.3.mib MRS3326(50)-S4C-1.2.3.mib | 322, 968 322,943 323,000 |
| BootRom in combination | Must use the（support vendor.cfg file and ECC function）1.2.0 Version and higher Version BootRom，it is recommend to use（add ROM and IMG file Upgrade on error-preventive）1.3.0Version and higher Version’s BootRom。 |  |
| Manual in combination | 《ES4626(50) switch installation guide》Version1.5.5 |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

#### ES3628EA Serials Firmware Mapping Table

| Product Series | ES3628EA series |  |
| --- | --- | --- |
| Model Name | ES3628EA |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES3628EA_5.4.128.0_nos.img | 4,658,286 |
|  | BootRom file |  |
|  | ES3628EA_1.5.1_boot.rom | 1,507,004 |
|  | ES3628EA_1.5.1_config.rom | 456,988 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES3628EA-1.0.0-vendor.cfg | 40,420 |
|  | Private Mib file |  |
|  | ES3628EA-1-MIB-1.2.3.mib | 322,976 |
| BootRom in combination | Must use the（add ROM and IMG file to avoid error upgrade）1.5.0 Version and higher Version BootRom。 |  |
| Manual in combination |  |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

## 5.4.123.0 Version Release Introduction

### Release date

Release date：2008/06/20

### Version table

#### ES4700 Serials Firmware Mapping Table

| Product Series | ES4700 series |  |
| --- | --- | --- |
| Model Name | ES4704BD、ES4710BD |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES4700_5.4.123.0_nos.img | 11,166,921 |
|  | BootRom file |  |
|  | ES4700_2.1.0_boot.rom | 1,335,765 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4704BD-1.0.0-vendor.cfg | 42, 788 |
|  | ACCTON-ES4710BD-1.0.0-vendor.cfg | 42, 788 |
|  | Private Mib file |  |
|  | EdgeCore_ES4704-1.2.3.mib | 322, 977 |
|  | EdgeCore_ES4710-1.2.3.mib | 322, 960 |
| BootRom in combination | exist hardware Version R 3.5 and higher Version’s board card时，Must use the2.1.0Version and higher Version’s BootRom。 when all board card ‘s hardware Version lower than R 3.5时， please use previous ’s 1.7.2 or .1.7.6Version’s BootRom。 detail information reference to following ’s Note。 |  |
| Manual in combination | 《ES4700 switch installation guide》Version1.5.5 |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. 2. chasis switch support latest’s R 3.5 Version（check label on board card），to this Version’s hardware board card，not support IMG Version before 5.4.123.0 or Bootrom Version before 2.1.0 3. For new purchase’s racked switch board card, if hardware Version equal or greater than R 3.5，and main-control board card hardware Version lower than R3.5, main control board card’s file need to be upgraded first: IMG Version equal or greater than 5.4.123.0，Bootrom Version equal or greater than 2.1.0。Inset board card after reset the switch。 4. After IMG upgraded to 5.4.123.0 and higher Version，please do not downgrade IMG to 5.4.109.0 or lower Version, system may cannot bootup properly. This operation should be guided by technician. |  |

#### ES4624(26)-SFP Serials Firmware Mapping Table

| Product Series | ES4624(26)-SFP series |  |
| --- | --- | --- |
| Model Name | ES4624-SFP ES4626-SFP |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES4624(26)-SFP_5.4.123.0_nos.img | 5,970,626 |
|  | BootRom file |  |
|  | ES4624(26)-SFP_1.6.5_boot.rom | 2, 063, 860 |
|  | ES4624(26)-SFP_1.6.5_config.rom | 499, 032 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4624-SFP-1.0.0-vendor.cfg | 40, 418 |
|  | ACCTON-ES4626-SFP-1.0.0-vendor.cfg | 40, 418 |
|  | Private Mib file |  |
|  | ES4626-SFP-1.2.3.mib | 322, 986 |
| BootRom in combination | Must use the（support vendor.cfg file and ECC function）1.6.2Version and higher Version’s BootRom，it is recommend to use（add ROM and IMG file Upgrade on error-preventive）1.6.5Version and higher Version BootRom。 |  |
| Manual in combination | 《ES4624(26)-SFP switch installation guide》Version1.5.5 |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

#### ES4626(50) Serials Firmware Mapping Table

| Product Series | ES4626(50) series |  |
| --- | --- | --- |
| Model Name | ES4626（MRS3326-S4C、GigaBee-X24）、 ES4650（MRS3350-S4C） |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES4626(50)_5.4.123.0_nos.img | 5,970,626 |
|  | BootRom file |  |
|  | ES4626(50)_1.3.0_boot.rom | 1, 935, 772 |
|  | ES4626(50)_1.3.0_config.rom | 352, 908 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES4626-1.0.0-vendor.cfg MRV-MRS3326-S4C-1.0.3-vendor.cfg NTT-GigaBee-X24MK-II-1.0.2-vendor.cfg | 40, 418 26, 236 36, 572 |
|  | ACCTON-ES4650-1.0.0-vendor.cfg MRV-MRS3350-S4C-1.0.5-vendor.cfg | 40, 418 26, 297 |
|  | Private Mib file |  |
|  | ES4650-1.2.3.mib GigaBee-X24MK-II-1.2.3.mib MRS3326(50)-S4C-1.2.3.mib | 322, 968 322,943 323,000 |
| BootRom in combination | Must use the（support vendor.cfg file and ECC function）1.2.0 Version and higher Version’s BootRom，it is recommend to use（add ROM and IMG file Upgrade on error-preventive）1.3.0Version and higher Version’s BootRom。 |  |
| Manual in combination | 《ES4626(50) switch installation guide》Version1.5.5 |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

#### ES3628EA Serials Firmware Mapping Table

| Product Series | ES3628EA series |  |
| --- | --- | --- |
| Model Name | ES3628EA |  |
| Files Released | File Name | Size（Bytes）（Bytes） |
|  | IMG file |  |
|  | ES3628EA_5.4.123.0_nos.img | 4,653,769 |
|  | BootRom file |  |
|  | ES3628EA_1.5.1_boot.rom | 1,507,004 |
|  | ES3628EA_1.5.1_config.rom | 456,988 |
|  | Vendor.cfg file |  |
|  | ACCTON-ES3628EA-1.0.0-vendor.cfg | 40,420 |
|  | Private Mib file |  |
|  | ES3628EA-1-MIB-1.2.3.mib | 322,976 |
| BootRom in combination | Must use the（add ROM and IMG file Upgrade on error-preventive）1.5.0Version and higher Version’s BootRom。 |  |
| Manual in combination |  |  |
| Manual download from |  |  |
| Note | 1. It is recommended that Internet Explorer 6.0 or above be used when accessing web-related features. |  |

# Bug fixed table

## 6.1.N.0 Version bug fixed table

6.1.N.0 compared with version 6.0.207，fixed following issue：

| BUG-ID | Problem Description |
| --- | --- |
| 41750 | When port NAME is null，SNMP return wrong encoding |
| 41644 | SSL authentication show improper message |
| 41259 | VRRP max support extend to 192 (before was 64) |
| 40222 | 6.0 version , chip 12GT cant manage switch |
| 40124 | Re-distribute multi ospf process,run no router rip，switch zRIPtask mount |
| 40030 | multicast subpression request |
| 39920 | Support 64 multicast PIM port allocate |
| 39904 | request MAC authentication function implement guest vlan and auto vlan function |
| 39861 | Cancel IPv6 enable command |
| 39806 | log signal function. |
| 39712 | Need ECMP support RIP protocol |
| 39312 | specific IP cant communicate to each other，change Ip address work normal |
| 39070 | modify password encryption’s parameter |
| 38849 | Open access to low level use fof show mac and show arp |
| 38070 | debug snmp kernel’s output view’s name maybe wrong encoded |
| 37998 | mstp default name change to NULL（same with cisco） |
| 37878 | Under bootrom,add no password’s command, remove mangement password in img |
| 37876 | Several problem of Window size’s terminal length. |
| 37757 | Add policy route bottom function |
| 37692 | Add SNTP Source function. |
| 37690 | Add SNMP Trap Source function |
| 37656 | use script to run write operation repeat on active board,active/redundant status error in 5 minutes |
| 36731 | When new dot1x user authentication learn，previous user cant Ping to external internet |
| 36285 | temparatue warning always warning，cant disable |
| 35337 | bootrom’s flash delete incomplete ,cause write error |
| 35201 | Built 64 vrrpv3 group，when assign port up/down，redundant selection not correct |
| 35164 | When two switch enable ipv6 vrrp function ，redundant switch will show address conflict message |
| 34560 | Add PIM protocol ‘s port report accouting fucntion |

## 6.0.207.0 Version bug fixed table

6.0.207.0 is new  develop Version,most enchanced function,since there are more function enchanced and bug fix in this version, will not list out  all here.

| BUG-ID | Problem Description |
| --- | --- |
| 30801,30827 | TS01 Change SSH user account password length to 32 char |
| 30839 | TS01 Timezone problem on ES4626-SFP |
| 34970 | ts01:ES4626/50 pause frame display issue(web UI add pause frame count） |
| 35706 | ts01:Snmp trap message’s enterprise and agent addr word value fault |
| 31505 | TS01 ES-4624SFP SNMPwalk problem |
| 31621 | TS01 Add command line show private mib OID |
| 31380 | TS01 SNMP crash with big port mirror problem |
| 31505 | TS01 ES-4624SFP SNMPwalk problem |
| 31249 | TS01 port security problem on ES4626H |
| 31845 | TS01 Modify authentication login command to select next authentication rule |

## 5.4.128.0 Version bug fix list

| BUG-ID | Problem description |
| --- | --- |
| 24080 | TS01-VLAN name only 11 characters Switch(config)#vlan 1 name ? WORD Vlan name <1-11> character |
| 24394 | Module can’t be reset by agent card after hang |
| 23848 | SNMP modified some node’s process method 1.for non index’s parameter Get Next； 2.for index’s parameter ，no index to get-next（ or incomplete index to get-next）； 3.For index’s parameter use minimal reasonable index of get-next； Condition mentioned above will return error。 |

## 5.4.123.0Version bug fix list

| BUG-ID | Problem description |
| --- | --- |
| 22708 | TS01-Show Temperature error display Different’s English console show different message： 1.TeraTerm Pro Temperature: 48.5000 iae 2.Procomm Plus Temperature: 48.5000 !f 3.SecureCRT Temperature: 48.5000 ⊥ |
| 22617 | MP ospf problem（recursive routing） In OSPF learning’s router，there is a router which is local static route，should not be the local 's static route to reach and form recursive routing If a loop formed, can use route map's match ip nexthop to filters. |
| 22613 | vrrp standby switch data forwarding problems did not clear vrrp backup machine 's corresponding arp, leading to vrrp backup group data forwarding problems: 1) s1, s2 and s3allocate Trunk interfaces, are connected to a hub on; 2) s1, s2 to vrrp allocate, vrrp a backup group, the initial s1 for the master, s2 as a backup; 3) s3 ping this vrrp virtual address, success; 4) s2 is also ping this vrrp virtual address, success, show arp to find the virtual address s2 to go to school 's arp; 5) shutdown / no shutdown s1 on 's trunk port, so that s2 into a master; 6) again s3 this vrrp virtual address on a ping, failed; 7) s2 on the show arp found that it retained in the backup state, to learn 's vrrp virtual address arp table entry, so cause of the problem |
| 22566 | Modify the vrrp counterparts' s three-interface ip address will result in vrrp problem, vrrp virtual ip conflict |
| 22565 | MSTP module spanning-tree format command advise default for the auto |
| 22514 | MP 6804 5.4.15.0 Synchronization 48GT Line Card failure (witch) slot2 inserted master MRS-6800-M12GB Version5.4.15.0, line card is the MRS-6800-48GT Version5.3.0.28. After the restart the upgrade master sync 48GT line card software failure. |
| 22405 | TS01-Chassis switch line card IMG-5.4.109.0Version in BootRom-1.7.2Version when the N / A France boot: User Version is the original img-5.4.22.0, bootrom-1.7.2., To provide users with a new Version: img-5.4.109.0, bootrom-1.7.6. Users to upgrade the master card img Version (from 5.4.22.0 -> 5.4.109.0) and bootrom Version (1.7.2-> 1.7.6), found in auto-sync after the img line card N / A France boot. |
| 22295 | ts01: es3628ea (5650-28) _5.4.22.0 as a dhcp server, not all interface addresses can as a relay 's ip helper address. The problem in 5650-28and chasis switch 's MRS-7600-4GX24TX card exists, other cards did not exist the problem. |
| 22231 | 5950-28(52)T’s boot mode need to support rename command |
| 22204 | Will be the same ospf area 's property from nssa changed stub, switch OSPF the task hangs |
| 22203 | snmpv3 repeat allocate snmp-server group d authnopriv read c write <WORD>command show run will have some garbage -error：snmp-server group b auth priv read c write bbbbc notify :啉:啉:? |
| 21968 | TS01-ES3628EA replace Cisco project support64MBand128MB’s Flash |
| 21864 | ts01:01 rmon function only statistics group can be used. 1. rmon design to support statistics, history ,alarm ,event .For statistics and history and ecview Network Management tools mapping is correct ，but alarm and event have compatible problem。 2.alarm’s problem ，after allocate event and alarm， trap and log produced only can view with mib browse，cant showed in ecview。 |
| 21801 | TS01-Some Command line ’s Complete 1.Create multiple Vlan by one command 2.ip helper-address. Can make command to multi-vlan interface at same time. 3.rip network vlan interface Can make command to multi-vlan interface at same time. |
| 21587 | ts01:switchport trunk allowed vlan add’s vlan too many and not continuous，report Vlan IDs parameter is too long! |
| 19848 | VRRP/VRRPv3 support’s max group number adjusted from 32 to 64。 |

# Existing problem and workaround

| BUG-ID | Problem exist | Workaround |
| --- | --- | --- |
| 35786 | When the backup master card or line card software for the 6.0 Version, and activity 5.4Version master card software, the master synchronization software to backup master card or line card, the backup master card or line card crash occurs the task . When the version of backup agent | If the backup master card or line card software needs to 6.0 Version retreated 5.4 Version back when, advise the bootrom mode retreated back to the old Version.。 |

# Version CHANGE NOTICE

## hardware  and IMG functionCHANGE NOTICE

| IMG Version | Release date | Item | Description |
| --- | --- | --- | --- |
| 6.1.40.0 | 2010-3-19 | hardware function upgrade | Compare to 6.0 version ，new support to following board card： MRS- 7608_NEW-MI(38-GWitch) MRS-7604-M1XFP12GX12GT_R4(39-GWitch) MRS-7600-12GX12GT_R4(40-GWitchSlave) MRS-7600-2XFP12GX12GT_R4(41-GWitchSlave) |
|  |  | IMG function upgrade | Compare to 6.0 version，add below function ： 1. System feature . IPFIX 2. L2 feature . ULPP/ULSM . Port-Hybrid . Port-Channel function optimize . Dot1x MAB 3. L3 feature . GREtunnel . ECMP . ND Snooping . DHCPv6 Snooping . Keep alive Gateway 4. Multicast . static multicast MAC 5. QoS/ACL feature . Vlan ACL . Egress ACL . Dynamic ACL ES4700’s LITE version not support DHCPv6 and DVMRP feature ，compare to above complete version only version support below feature： . Port-Hybrid . Port-Channel function optimize . ECMP . Keep alive Gateway Compare to 6.0 version，remove below feature： . not support ipv6 proxy function |
| 6.0.207.0 | 2009-08-24 | hardware function upgrade | N/A |
|  |  | IMG function upgrade | Compare with 5.4Version，6.0Version add specific below： - support IPv6 VRRPv3 - support IPv6 security RA - support DHCPv6 - support IPv6 URPF - support NTP - IP Cluster enhanced - Support LLDP and ULDP - support IMG backup，support multiple allocate file - support RSPAN |
| 5.4.128.0 | 2008-08-25 | hardware function upgrade | N/A |
|  |  | IMG function upgrade | 1.vlan name legal’s length 11 changed from 11 to 64。 |
| 5.4.123.0 | 2008-06-20 | hardware function upgrade | 1. chasis switch support latest’s R 3.5Version，for his Version’s hardware board card， IMG Version before 5.4.123.0 or BootromVersion2.1.0.’s file are not supported。 |
|  |  | IMG function upgrade | 1. Reply ARP Reply report when allocate arp security learn protect command，which source’s ip is local port IP address report， Version before return ARP Reply report。 2. Add Local ARP Proxy function 3. VRRP/VRRPv3 support’s max backup group number change from 32 to 64 4.Increase manually upgrade chasis switch each module ‘s bootrom and IMG file，instead of upgrading bootrom with module’s console port。 |
| 5.4.109.0 | 2008-04-13 | hardware function upgrade | N/A |
|  |  | IMG function upgrade | 1. Radius add NAS IP allocate 2. Add WEB base’s Dot1xauthentication 3. 76/68switch Add fan/power/temperature’s monitor and warnning trapfunction |
| 5.4.22.0 | 2008-02-02 | hardware function upgrade | N/A |
|  |  | IMG function upgrade |  |
| 5.3.0.20 | 2007-07-04 | hardware function upgrade | 1. Add support board cardEM4700BD-4XG-XFP、EM4700BD-48GX-SFP。 2. |
|  |  | IMG function upgrade |  |

## BootRom function CHANGE NOTICE

### ES4700 seriesGCommon and  GZoma board card group  BootRom function CHANGE NOTICE

| Version号 | Release date | Description |
| --- | --- | --- |
| 3.0.11 | 2010-04-30 | 1. add no password command |
| 2.2.1 | 2009-07-10 | - Support multi IMG. - Support multi startup-config. |
| 2.2.0 | 2009-06-17 | 1.Add anti bootrom backward function 2. support76(68)00racked BootRompatch function 3. support new hybrid flash detect ，replace previous hybrid flash detect 4. support auto adjust 100M and133M frequency |
| 2.1.0 | 2008-06-17 | 1.Add bootrom anti-backward function 2. support76(68)00racked BootRom patch function 3. support new’s hybrid flash detect ，replace previous hybrid flash detect 4. support auto adjust 100M and133M frequency |
| 1.7.6 | 2008-03-13 | 1. support ROM and IMG file’s upgrade error-preventive function。 2. Dynamic support128Mand256M flash’s board card。 |
| 1.7.2 | 2007-11-08 | 1. Add Flash ECC debug function。 2. supportVendor.cfg file。 |
| 1.6.10 | 2007-05-10 | 1. Add hybrid flash detect function |

### ES4624(26)-SFP series BootRom function CHANGE NOTICE

| Version号 | Release date | Description |
| --- | --- | --- |
| 2.0.11 | 2010-04-30 | 1. add no password command |
| 1.7.0 | 2009-07-10 | - Support multiple IMG. - Support multiple startup-config. |
| 1.6.5 | 2008-03-21 | 1. Add ROM and IMG file upgrade error-preventive function。 2. Dynamic support128Mand256M flash’s hardware 。 |
| 1.6.2 | 2007-11-16 | 1. Add ECC debug function。 2. supportVendor.cfg file。 |
| 1.5.16 | 2007-05-11 | 1. Add hybrid flash detect function |

### ES4626(50) series BootRom function CHANGE NOTICE

| Version | Release date | Description |
| --- | --- | --- |
| 2.0.11 | 2010-05-05 | 1. add no password command |
| 1.5.1 | 2009-08-12 | - support multiple IMG. - support multiple startup-config. - Support R3.5 hardware 。 |
| 1.3.0 | 2008-05-31 | 1．BootRom hybrid flash detect program。 2．128Mand256M flash auto adjust |
| 1.2.2 | 2008-04-13 | 1. Add BOOTROM’s upgrade error-preventive function |
| 1.2.0 | 2007-10-29 | 1. Add Flash ECC debug function。 2. supportVendor.cfg file。 |
| 1.1.2 | 2007-02-09 | 1. Add LED testing function， 2. Add optimize port testing(include External , PHY , MAC) |

## IMG Command line CHANGE NOTICE

| Version | Item | Description |
| --- | --- | --- |
| 6.1.40.0 | commad added | Add new module，example:IPFIX、Port-Hybrid etc,add some command line 。 |
| 6.0.207.0 | commad added | - cd - delete - dir - format - mkdir - mount - pwd - rename - rmdir - unmount - boot - username （increase cover range to telnet/console/web） - authentication line... login... - authentication security ip(ipv6) - ip dhcp conflict ping-detection enable - ip dhcp ping packets - ip dhcp ping timeout - urpf enable - no mrpp ring primary-port - no mrpp ring secondary-port |
|  | command delete | 1）telnet-user 2）web-user 3）show telnet-user 4）telnet-server security ip 5）telnet-server security ipv6 6）login local |
|  | Upgrade command | - copy - ip urpf enable - multicast destination-control - mrpp ring primary-port - mrpp ring secondary-port |
| 5.4.128.0 | command added | N/A |
|  | command delete | N/A |
|  | Upgrade command | vlan name command：assign name’s can support maximum 64 Bytes。 |
| 5.4.123.0 | command added | 1. Add allocate group multicast software forwarding aging time table command 2. 802.1x related command“aaa-accounting update” 3. Add Local ARP Proxy function related’s command，detail please reference to attached manual。 |
|  | command delete | N/A |
|  | Upgrade command | 1. spanning-tree format command’s default value change to auto 2. trunk allowed vlan command upgrade is switchport trunk allowed vlan { all ¦ {add¦remove¦except} <vlan-list>} 3. chasis switch copy command change to copy <source-url> [slot <slot-id>] <destination-url> [ascii ¦ binary]，added <slot-id>parameter is file or drive path of destination board card where it copy to，default as current active agent card； |
| 5.4.109.0 | command added | 1. show interface command detail 2. bgp inbound-max-route-num <0-500000> no bgp inbound-max-route-num 3. radius nas-ipv4 <ip-address> no radius nas-ipv4 radius nas-ipv6 <ipv6-address> no radius nas-ipv6 Description：Since there are too many command，details description will not list here. Please reference to related attached manual. Document acquisition method please reference to first chapter Version table’s mapping manual。 |
|  | command delete | N/A |
|  | upgradecommand | N/A |
| 5.4.22.0 | commad added |  |
|  | command delete | N/A |
|  | upgradecommand | N/A |
| 5.3.0.20 |  |  |

## BootRom Command line CHANGE NOTICE

### ES4700 seriesGCommon and  GZoma board card group BootRom Command line CHANGE NOTICE

| Version | Item | Description |
| --- | --- | --- |
| 3.0.11 | commad added | 1. no password |
|  | command delete | N/A |
|  | Upgrade command | N/A |
| 2.2.1 | commad added | - boot img set up boot ’s IMG file - boot startup-config set up startup-config file - show boot-files |
|  | command delete | N/A |
|  | Upgrade command | N/A |

### ES4624(26)-SFP seriesBootRom Command line CHANGE NOTICE

| Version | Item | Description |
| --- | --- | --- |
| 2.0.11 | commad added | 1. no password |
|  | command delete | N/A |
|  | Upgrade command | N/A |
| 1.7.0 | commad added | - boot img set up boot ’s IMG file - boot startup-config set up startup-config file - show boot-files |
|  | command delete | N/A |
|  | Upgrade command | N/A |

### ES4626(50) seriesBootRom Command line CHANGE NOTICE

| Version | Item | Description |
| --- | --- | --- |
| 2.0.11 | commad added | 1. no password |
|  | command delete | N/A |
|  | Upgrade command | N/A |
| 1.5.1 | commad added | - boot img set up boot ’s IMG file - boot startup-config set up startup-config file - show boot-files |
|  | command delete | N/A |
|  | upgradecommand | N/A |

# Version function list

## hardware function list

| Item | Description |
| --- | --- |
| ES4700 series support’s board card model | EM4704BD-M12GX-SFP EM4700BD-12GT-RJ45 EM4700BD-2XG-Xenpak EM4700BD-XG-Xenpak EM4704BD-M24TX4GC EM4710BD-Agent EM4700BD-12GX-SFP EM4700BD-8GC16GX2XG EM4700BD-8GC16GX EM4700BD-24TX4GC EM4700BD-12GC12GT2XG EM4700BD-12GC12GT EM4700BD-48GT EM4700BD-4XG-XFP EM4700BD-48GX-SFP |

## IMG function

6.0.207.0 is new developed Version，following function add。

| Item added | Function and Description |
| --- | --- |
| IPv6 VRRPv3 | For IPv6's Virtual Routing Redundancy Protocol |
| IPv6 security RA | To prevent malicious IPv6 router announcement service attacks |
| DHCPv6 | Achieve the DHCPv6 server and relay function, to provide IPv6 host 's stateful address allocation function |
| IPv6 URPF | The group multicast in the 's RPF technology to unicast is used to prevent fraud based on the IPv6 source address' s network attacks |
| NTP | Used on the network all have the clock 's clock synchronization devices |
| IP Cluster enhanced | function to enhance 's switch cluster Network |
| LLDP and ULDP | LLDP is a neighbor discovery protocol for other nodes in the network's own announcement 's existence, and save all the neighboring devices' s discovery information. ULDP used to detect devices' s single-link failures. |
| IMG backup and multiple allocate file | can the switch to store multiple copies of IMG fileandallocate file, and allows users to specify the boot allocate fileand first, second boot IMG file. |
| RSPAN | Remote port mirroring to allow the mirror source port and head 's port on different' s network device |

## BootRom function list

### ES4700 seriesGCommon and  GZoma board card group  BootRom function Table

| Item | Description |
| --- | --- |
| hybrid flash detect | support hybrid flash detect, press CTRL + D implementation of the N / A limit of sub-hybrid flash detect, press CTRL + O to perform a hybrid flash detect, press CTRL + E to perform a simple hybrid flash detect, hybrid flash detect the process, press ESC or CTRL + B may terminate the flash detect |
| Flash ECC debug function | Support ECC debug function |
| Vendor.cfg file support | support |
| BOOTROM upgrade error-preventive function | Support BOOTROM's upgrade error-preventive function, the use of 1.7.5 and above Version's BOOTROM later, BOOTROM only allows upgrade support under the current hardware 's ROM file and IMG file, Note: back to upgrade before Version's ROM file and IMG file, use the back door command: write <filename> / F; directly write <filename> command upgrade will fail. |
| support’s Flash | Support ST Flash、Atmel Flash、Intel Flash |
| support board card’s flash | Dynamic support128Mand256M flash's board card, 1.7.5 and above Version's BOOTROM boot time can be based on the current 's board card's flash's size is set the software parameter, can support256M flash's master card |

### ES4624(26)-SFP series BootRom function list

| Item | Description |
| --- | --- |
| hybrid flash detect | support hybrid flash detect, press CTRL + D implementation of the N / A limit of sub-hybrid flash detect, press CTRL + O to perform a hybrid flash detect, press CTRL + E to perform a simple hybrid flash detect, hybrid flash detect the process, press ESC or CTRL + B may terminate the flash detect. |
| Flash ECC debug function | Support Flash ECC debug function |
| Vendor.cfg file support | support |
| BOOTROM upgrade error-preventive function | Support BOOTROM’s upgrade error-preventive function，after upgrade to 1.6.3 or above Version’s BOOTROM,BOOTROM only allow to upgrade support current hardware ’s ROM file and IMG file Be mentioned: upgrade to older Version’s ROM file and IMG file please use backdoor command：write <filename> /F ;it will failed when use write <filename>command upgrade。 |
| support’s Flash | Support ST Flash、Atmel Flash。 |
| support board card’s flash | Dynamic support 128M and 256M flash’s board card，1.6.3 or above Version’s BOOTROM boot according to current’s board card’s flash’s size setting s/w parameter ，can support 256M flash’s switch。 |

### ES4626(50) Serials BootRom Function List

| Item | Description |
| --- | --- |
| hybrid flash detect | Not support |
| Flash ECC debug function | Support Flash ECC debugging function |
| Vendor.cfg file support | Supported |
| ROM and IMG file’s upgrade error-preventive function | Support BOOTROM’s upgrade’s error-preventive function，after use Version’s BOOTROM after or above1.2.2 ，BOOTROM only allow upgrade support current hardware ’s ROM file，Mentioned： upgrade to older Version’s ROM file please use backdoor command：write <filename> /F ;use write <filename>command upgrade will failed。 |
| Interface Test Function | Include port external loop-back test , PHY and MAC Loop-back test |

# ES4700 series agent card module mapping list

ES4700 series switch support many board card type, this  board card type divide to GCommon、GZoma board card. Switch board card type with ID and board card group mapping list below.

| board card type | board card group |
| --- | --- |
| MRS-7604-M12GB（EM4704BD-M12GX-SFP） | GCommon |
| MRS-7600-12GT（EM4700BD-12GT-RJ45） | GCommon |
| MRS-7600-2-10Gx（EM4700BD-12GT-RJ45） | GCommon |
| MRS-7600-1-10Gx（EM4700BD-XG-Xenpak） | GCommon |
| MRS-7604-M4GX24TX（EM4704BD-M24TX4GC） | GCommon |
| MRS-7608-MI（EM4710BD-Agent） | GCommon |
| MRS-7600-12GB（EM4700BD-12GX-SFP） | GCommon |
| MRS-7600-2XFP8GX16GB（EM4700BD-8GC16GX2XG） | GZoma |
| MRS-7600-8GX16GB（EM4700BD-8GC16GX） | GZoma |
| MRS-7600-4GX24TX（EM4700BD-24TX4GC） | GCommon |
| MRS-7600-2XFP12GX12GT（EM4700BD-12GC12GT2XG） | GZoma |
| MRS-7600-12GX12GT（EM4700BD-12GC12GT） | GZoma |
| MRS-7600-48GT（EM4700BD-48GT） | GZoma |
| MRS-7600-4XFP（EM4700BD-4XG-XFP） | GZoma |
| MRS-7600-48GB（EM4700BD-48GX-SFP） | GZoma |