ES3510MA-FLF-38

Firmware Version 1.3.8.2

INTRODUCTION:

ES3510MA-FLF-38 supports 8 10/100 Base-T ports with 2 Gigabit combo ports, which are comprised of an RJ45 and a SFP.* 10/100/1000 CU built-in associated Mini-GBIC slots

Using IP Clustering for stacking which is a method of grouping switches regardless of geographical location in order to enable centralized management access to the switches through a single management point. This technology provides some of the benefits of stackable switches without requiring direct stacking or connections between switch units. It also allows different kinds of switches to be grouped together, as long as they have the embedded clustering software.

It offers advanced administration through a user-friendly browser interface. QoS and VLAN Supports allows greater scalability and Supports for business-critical applications. Port can be trunked to create a high bandwidth pipe to the network.

It also has comprehensive network management functions, such as Spanning Tree protocol for standard bridging, SNMP, RMON and security features.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Firmware | Version | File Name |
| --- | --- | --- |
| Loader | 1.0.0.5 | es3510ma_ld_V1.0.0.5.bix |
| Software | 1.3.8.2 | es3510ma_v1.3.8.2.bix |

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Phase 3 |  |  |  |
| Patch Version | 1.3.8.2 | Formal release | 2013/07/04 |
| Test Version | 1.3.8.1 | For NTC testing | 2013/06/17 |
| Test Version | 1.3.8.0 | For testing | 2013/05/16 |
| Patch Version | 1.3.7.2 | Formal release | 2013/01/23 |
| Current Version | 1.3.7.0 | Formal release | 2012/11/26 |
| Patch Version | 1.3.6.5 | Patch version | 2012/08/22 |
| Patch Version | 1.3.6.3 | Patch version | 2012/08/01 |
| Patch Version | 1.3.6.2 | Patch version | 2012/07/25 |
| Previous Version | 1.3.6.0 | Formal release | 2012/06/14 |
| Previous Version | 1.3.5.0 | For NTC test | 2012/05/25 |
| Patch Version | 1.3.4.5 | Patch version for BID 3 | 2012/01/20 |
| Patch Version | 1.3.4.4 | Patch version | 2011/11/28 |
| Previous Version | 1.3.4.3 | Formal release | 2011/11/18 |
| Previous Version | 1.3.4.2 | Formal release | 2011/10/21 |
| Previous Version | 1.3.4.0 | Formal release | 2011/10/14 |
| Previous Version | 1.3.2.4 | For NTC test | 2011/10/11 |
| Previous Version | 1.3.2.2 | For NTC test | 2011/09/16 |
| Previous Version | 1.3.2.1 | For Vee Time | 2011/07/22 |
| Previous Version | 1.3.2.0 | Support ECS3550-10PD | 2011/06/23 |
| Previous Version | 1.3.0.8 | Patch version | 2011/06/23 |
| Previous Version | 1.3.0.7 | Patch version | 2011/05/20 |
| Previous Version | 1.3.0.6 | Patch version | 2011/05/12 |
| Previous Version | 1.3.0.3 | Formal test | 2011/04/27 |
| Previous Version | 1.3.0.1 | NTC test | 2011/03/10 |
| Phase 2.5 |  |  |  |
| Current Version | 1.2.4.7 | For Vee Time | 2011/01/10 |
| Previous Version | 1.2.4.2 | Formal release | 2010/12/08 |
| Previous Version | 1.2.4.1 | NTC release | 2010/12/01 |
| Previous Version | 1.2.4.0 | NTC test | 2010/11/16 |
| Previous Version | 1.2.2.0 | NTC test | 2010/08/26 |
| Previous Version | 1.2.1.1 | NTC test | 2010/08/16 |
| Previous Version | 1.2.0.1 | Apply IPv6 ready logo | 2010/06/11 |
| Previous Version | 1.2.0.0 | For demo | 2010/06/04 |
| Phase 2 |  |  |  |
| Current Version | 1.1.4.9 | Formal release | 2010/11/01 |
| Previous Version | 1.1.4.6 | For BigAir | 2010/07/27 |
| Previous Version | 1.1.4.5 | For Vimcom | 2010/06/22 |
| Previous Version | 1.1.4.4 | For Vimcom | 2010/06/03 |
| Previous Version | 1.1.4.3 | Formal release | 2010/04/20 |
| Previous Version | 1.1.4.2 | NTC test | 2010/04/20 |
| Previous Version | 1.1.4.0 | NTC test | 2010/04/13 |
| Previous Version | 1.1.3.x | Engineer test | 2010/3 |
| Phase 1 |  |  |  |
| MP Version | 1.1.2.0 | Formal release | 2010/01/19 |
| Previous Version | 1.1.0.4 | NTC test | 2009/01/16 |
| Previous Version | 1.1.0.0 | NTC test | 2008/12/24 |

HARDware compatibility:

ES3510MA-FLF-38

ES3510MA-DC-FLF-EC

ES3510MA-PD-FLF-EC (ECS3550-10PD)

ECS3510-10PD

Note: Only ECS3550-10PD must use es3510ma_ld_V1.0.0.2.bix (or above) and ECS3510-10PD must use es3510ma_ld_V1.0.0.4.bix (or above), the other models are not necessary to use this version.

BootPROM compatibility:

ALL

Network Management Software Support:

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Accton Product Feature List |  | ES3510MA-FLF-38 |
| --- | --- | --- |
| Product Configuration |  | 8FE + 2 ComboG |
| Switch ASIC |  | Marvell 98DX1005 |
| PHY |  | Marvell 88E3083 + 88E1322 |
| Flash Memory |  | 16MB |
| SDRAM |  | 128MB |
| CPU |  | 98DX1005 embedded CPU |
| L2 MAC Table |  | 8K |
| PoE |  | No |
| Standalone/Stackable |  | Standalone |
| Layer 2 Features |  |  |
| Link Aggregation | 802.3ad with LACP | YES 1) Total 12 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 2 port/trunk GE |
|  | Cisco EtherChannel Liked | YES 1) Total 12 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 2 port/trunk GE |
|  | Unicast / Multicast traffic Balance over Trunking port | YES 1) Unicast : MAC SA/DA |
| VLAN | IEEE 802.1Q Tagged Based | YES |
|  | Port-Based | YES (256 VLANs) |
|  | MAC-Based | YES |
|  | IP-Based | YES |
|  | Private VLAN | YES (Traffic Segmentation per port/VLAN by chip) |
|  | GVRP | YES |
|  | 802.1v Protocol | YES |
|  | IPv6 VLANs | YES |
|  | voice VLAN | YES |
|  | 4K (4092) VLANs | YES (Phase 2) |
|  | VLAN Trunking | YES |
| Spanning Tree | IEEE 802.1D | YES |
|  | IEEE 802.1s | YES (32 instances) |
|  | IEEE 802.1w | YES |
|  | Spanning Tree Fast Forwarding | YES |
|  | Loopback Detection | YES (can select port disable in phase 2) |
|  | Auto Edge Port | YES |
|  | BPDU Filter/Guard | YES |
|  | Root Guard | YES |
| Broadcast/Multicast/UnknownUnicast Storm Control |  | YES (no Broadcast, B+M, B+M+DLF limit) |
| IGMP Snooping | IGMP Snooping v1/v2 | YES |
|  | IGMP v1/v2 querier support | YES |
|  | IGMP Immediate Leave | YES |
|  | IGMP Filtering/Throttling | YES |
|  | IGMP SNP Leave Proxy | YES |
|  | IGMP v1/v2/v3 Proxy | YES |
|  | IGMPV3 Snooping | NO |
| MVR |  | YES (groups - admin: 1024, active: 255 in phase 2) |
| Jumbo Frame |  | 10K |
| QinQ |  | YES |
| select QinQ |  | YES (Phase 2) |
| EAPS |  | NO |
| Auto MDI/MDIX |  | Auto |
| Types of GBIC and SFP support (LX, SX, LH, FX) |  | YES |
| G.8032 (ERPS) |  | YES (add in phase 2.5) |
| Y.1731 |  | YES (add in phase 2.5) |
| 802.1ag (CFM) |  | YES (add in phase 2.5) |
| 802.3ah (OAM) | Customer Premises Equipment (CPE) | YES (add in phase 2.5) |
| QoS Features |  |  |
| Number of priority queues |  | 4 queues/port |
| Scheduling for priority queues | WRR Priority scheduling | YES (Shaped Deficit WRR) |
|  | Strict Priority scheduling | YES |
|  | Hybrid | YES (Shaped Deficit WRR + Strict) |
| COS | 802.1p based COS | YES (per system) |
|  | IP TOS Precedence based COS | NO |
|  | IP DSCP based COS | DSCP Only (per system) |
|  | TCP/UDP Port based COS | YES (no command) |
| Rate Limiting | Ingress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
|  | Egress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
| DiffServ |  | YES |
| Security Features |  |  |
| Static Port Security (MAC-based) |  | YES |
| Dynamic Port Security (MAC-based) |  | YES |
| MAC Limitation per port |  | YES |
| 802.1x (authenticator) | Port-Based | YES |
|  | MAC-Based | YES |
|  | VLAN Assignment | YES |
|  | QoS Assignment (w/o ACL profile) | YES |
|  | QoS assignment w/ ACL profile | YES |
|  | Guest VLAN | YES |
|  | Supplicant Support | YES |
|  | EAPOL frames pass-through | YES |
| MAC Authentication |  | YES |
| WEB Authentication |  | YES |
| ACL | Number of rules | 512 (extended mode 256 rules) |
|  | L2/L3/L4 | YES 1) L2 SA/DA MAC, VLAN based 2) L3 IP SA/DA, subnet based 3) L4 TCP/UDP port 4) IPv6 |
|  | Time-Based (Time Range) | YES |
| RADIUS authentication |  | YES |
| RADIUS authorization and accounting |  | YES |
| TACACS+ Authentication |  | YES |
| TACACS+ authorizaion and accounting |  | YES |
| TACACS+ 3.0 |  | YES |
| HTTPS and SSL (Secured Web) |  | YES |
| SSH 1.5/V2.0 (Secured Telnet session) |  | YES |
| User name password authentication | Local Authentication | YES |
|  | Remote Authentication via RADIUS/TACACS+ | YES |
| Management Interface Access Filtering | SNMP | YES |
|  | WEB | YES |
|  | TELNET | YES |
| Intrusion Lock (Link Detection) |  | YES |
| MAC filter |  | YES |
| Traffic Segmentation |  | YES (support multiple session in phase2) |
| PPPoE Intermediate Agent |  | Yes (add in phase 3) |
| Management Features |  |  |
| Cisco Like CLI |  | YES |
| CLI filtering and terminal setting |  | NO (will add in future) |
| Web Based Management |  | YES |
| Telnet | Client | YES |
|  | Server | YES (original is 4, support 8 in phase 2) |
| Software Download/Upgrade | TFTP | YES (single image) |
|  | Xmodem / Ymodem | YES (Runtime, Loader) |
|  | Diag support TFTP download | NO |
|  | FTP | YES (add in phase 3) |
|  | HTTP | YES (single image) |
| Dual Image |  | NO (hardware limit) |
| Configuration Download/Upload | TFTP | YES (Multiple Copies) |
|  | HTTP | YES (Multiple Copies) |
| Auto upgrade | TFTP | YES |
| Auto upgrade | FTP | YES (add in phase 3) |
| SNMP | v1 | YES |
|  | v2c | YES |
|  | v3 | YES |
| RMON | RMON I (1,2,3,9 group) | YES |
|  | RMON 2 | YES |
| MIB I/II | RFC 1213 | YES |
| BOOTP | Client | YES |
|  | Relay | NO |
| DHCP | Client | YES |
|  | Relay | NO |
|  | Snooping | YES |
|  | Snooping Option 82 | YES |
|  | Dynamic Provision (via Option 66,67) | YES |
|  | Option 82 | YES |
|  | Relay option 82 | YES (add in phase 2.5) |
|  | Server | No |
| IP Source Guard |  | YES (user can config max. binding entry in phase 2) |
| Port Mirroring |  | YES |
| Remote Port Mirroring |  | YES |
| Event/Error Logging | Syslog (Local Flash) | YES 1) 512 entries 2) 256Kbyte Flash |
|  | Remotelog (RFC3164) | YES 1) Cisco-like 2) Support up to 5 Servers |
|  | SMTP (E-mail Notification) | YES |
| DNS | Client | YES |
|  | Proxy | YES |
| Remote Ping |  | YES |
| SNTP |  | YES |
| NTP |  | Yes (add in phase 3) |
| IP Clustering |  | YES (36 members) |
| LLDP(802.1ab) | Link Layer Discovery Protocol | YES |
|  | LLDP-MED | YES (add in phase 3) |
|  | IEEE 802.3at | NO |
| UPnP |  | NO |
| Banner |  | YES |
| MAC Flush |  | YES |
| sFlow |  | NO (chip limitation) |
| Dynamic ARP inspection (DAI) |  | YES |
| VLAN mirror |  | YES |
| MAC Based mirror |  | YES |
| ATC |  | YES |
| Delay reload |  | YES |
| CLI “show debug” |  | NO (will add in future) |
| CLI “show tech” |  | YES |
| CLI “show interfaces brief” |  | YES (support in phase 2) |
| Cable Diagnostics / TDR |  | YES |
| Port Utilization (kbits/sec, Pkts/sec, % Util in recent 300secs) |  | YES |
| IPv6 Management (Telnet Server/ICMP v6) |  | YES |
| Green Ethernet |  | YES (combo GE port only) |
| multiple management ip support |  | YES (phase 2 V1.1.4.6 and Phase 2.5 ) |
| PoE |  | NO |
| Monitor Environment | Power Status | NO |
| IPv6 Features |  |  |
| IPv4/IPv6 Dual Protocol Stack |  | YES |
| IPv6 Address Types | Unicast | YES |
|  | Multicast | This is only used internally. Not configurable. |
| ICMPv6 |  | YES |
| ICMPv6 Redirect (Host) |  | YES |
| IPv6 Path MTU Discovery |  | YES |
| IPv6 Neighbor Discovery | Router Discovery | YES |
|  | Duplicate Address Detection | YES |
|  | Static Cache Entry | NO |
|  | Parameter Discovery | YES |
|  | Prefix Discovery | YES |
|  | Address Resolution | YES |
|  | Unreachable Neighbor Detection | YES |
|  | Redirect (Host) | YES |
| Stateless Autoconfiguration |  | YES |
| Manual Configuration |  | YES |
| Generic Prefix |  | NO |
| SNMP over IPv6 |  | YES |
| HTTP over IPv6 |  | YES |
| SSH over IPv6 |  | YES |
| IPv6 Telnet Support |  | YES |
| IPv6 DNS Resolver |  | YES |
| IPv6 RADIUS+ Support |  | NO |
| IPv6 TACACS+ Support |  | NO |
| IPv6 Syslog Support |  | YES |
| IPv6 SNTP Support |  | YES |
| IPv6 SMTP Support |  | NO |
| IPv6 TFTP Support |  | YES (CLI not support) |
| Remote IPv6 Ping |  | YES |
| IPv6 QoS |  |  |
| IPv6 ACL | Number of rules: |  |
|  | L2/L3/L4 | YES |
| IPv6 DSCP remapping CoS |  | NO |

Installation and Configuration Notes:

In general, the ES3510MA Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ES3510MA Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

To download new firmware form a TFTP server, enter the IP address of the TFTP server, select “opcode” as the file type, then enter the source and destination file names. To start the new firmware, enter the “reload” command or reboot the system.

If your current firmware version is 1.1.2.0, you must setup “Destination file name:” to same as ES3510MA-FLF-38_V1.1.2.0.bix.  After that version, you can setup any file name you want.

CLI commands:

Console#copy tftp file

TFTP server IP address: 192.168.1.23

Choose file type:

1. config:  2. opcode:  : 2

Source file name: opcode.bix

Destination file name: opcode.bix

\Write to flash programming.

Programming flash started.

Success.

Console#reload

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Runtime V1.3.8.2 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00652 | Log Host Port in WEB GUI auto be assigned the invalid value "0" at the first time but can be assign "514" in CLI . |
| --- | --- |
| ES3510MA-FLF-38-00639 | Syslog error Cause DUT exception then system crash |
| ES3510MA-FLF-38-00632 | [ES3528M] Management access |
| ES3510MA-FLF-38-00702 | ES3510MA support mvr proxy-switching, but no "MVR Configure Global" web page |
| ES3510MA-FLF-38-00670 | CoS:If enable strict mode in low priority. and Strict-Wrr mode was incorrect. |
| ES3510MA-FLF-38-00640 | Setting NLM Cause system exception |
| ES3510MA-FLF-38-00592 | PC with Win XP in MAC vlan and dhcp snooping doesn't receive IP address from DHCP server |
| ES3510MA-FLF-38-00649 | WEB port panel:all ports are display link down even there's some ports connected the DUT |
| ES3510MA-FLF-38-00697 | UI - The Port status display are incorrect. |
| ES3510MA-FLF-38-00695 | front pannel display issue in web page |
| ES3510MA-FLF-38-00669 | CoS:Default Priority was support setting in Trunk. and Trust mode was not support. |
| ES3510MA-FLF-38-00650 | Even if Port#9 AND # 10 link with Copper ,the WEB GUI always show SFP link status |
| ES3510MA-FLF-38-00665 | WEB : DAI Not support the ip allow-zeros option |
| ES3510MA-FLF-38-00662 | MAC-BasedVLAN - MAC-Basic VLAN entry did not fully display on Web page. |
| ES3510MA-FLF-38-00693 | (Enhancement) -- IP IGMP Query-drop vlan<VID> |
| ES3510MA-FLF-38-00685 | IPv6 MTU : the size range 1280~65535 is not correct |
| ES3510MA-FLF-38-00672 | IGMPSnooping:"clear ip igmp snooping group dynamic" can't clear dynamic group. |
| ES3510MA-FLF-38-00655 | Cluster:Command "clear cluster candidate table" not work. |
| ES3510MA-FLF-38-00698 | Web UI - "Select all item" can not work well when all items was be select. |
| ES3510MA-FLF-38-00694 | MSTP instances can not be read by SNMP |
| ES3510MA-FLF-38-00663 | MAC-BasedVLAN - When the VLAN does not exist, the MAC-baseVLAN able to created success. |
| ES3510MA-FLF-38-00688 | DAI: DUT cannot save this filter setting to running configuration when we create two ACL rules. |
| ES3510MA-FLF-38-00673 | Diffserv(web): If class map remove 1 choice item, all select item didn't disappear choice auto. |
| ES3510MA-FLF-38-00668 | IP Source Guard: Unselect all interface, the Check Box which select all interface still selected. |
| ES3510MA-FLF-38-00689 | SYSLOG time format is incorrect. |
| ES3510MA-FLF-38-00667 | CoS:Trust mode of CoS was incorrect. |
| ES3510MA-FLF-38-00651 | Log Host : Only setting the IP address of log host in WEB GUI can Not be kept after save and reboot. |
| ES3510MA-FLF-38-00666 | CoS: 2 Queue items can be set vai web UI. |
| ES3510MA-FLF-38-00664 | VLAN - In Web page, the frame of PVID shows no consistency. |
| ES3510MA-FLF-38-00631 | Exception messages will appear if ES3528M telnet to another one, show running. |
| ES3510MA-FLF-38-00654 | DHCP Snooping:When enable DHCP Snooping status, client can not get IP from Server. |
| ES3510MA-FLF-38-00644 | IGMP snooping drop counter didn't increase after IGMP membership report dropped by IGMP filiter |

Runtime V1.3.8.1 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00630 | When telnet from commander to candidate,console will be hank for 15-20 minutes |
| --- | --- |
| ES3510MA-FLF-38-00639 | Syslog error Cause DUT exception then system crash |
| ES3510MA-FLF-38-00629 | Telnet through the switch with wrong IP , the the VTY got freezed |
| ES3510MA-FLF-38-00622 | switch received mutlicast packet came form vlan which disable IGMP Snooping. then shows multicast groups in that vlan |
| ES3510MA-FLF-38-00514 | If we create a ACL by snmp, we can't bind this ACL to port by console and snmp. |
| ES3510MA-FLF-38-00623 | We can't bind acl to port via snmp |
| ES3510MA-FLF-38-00641 | [Enhancement]DHCPSNP ratelimit should be accurate to packet per secound |
| ES3510MA-FLF-38-00632 | [ES3528M] Management access |

Runtime V1.3.8.0 ; Loader V1.0.0.5

|  | Add filter for PVST packets |
| --- | --- |
|  | IGMP RADIUS authentication |
| ES3510MA-FLF-38-00638 | DHCP snooping option 82 checksum calculation problem (checusum error) |
| ES3510MA-FLF-38-00635 | Port security can't work well , when we enable DHCP snooping and port sercurity on combo port. |
| ES3510MA-FLF-38-00634 | A switch does not work after receiving several DHCPSNP attack messages in the syslog |
| ES3510MA-FLF-38-00633 | SNMP(dot1dStpPortDesignatedPort) show wrong |
| ES3510MA-FLF-38-00625 | After copy startup-config tftp procedure on flash appears cfg file, with size 15M |
| ES3510MA-FLF-38-00626 | es3510ma_flf_38 & es3628bt_flf_zz use same mibtreeinit.c |
| ES3510MA-FLF-38-00627 | privilege exec all level 0 test cable-diagnostics doesn't work on simba platform. |
| ES3510MA-FLF-38-00621 | MVR MIB:It didn't set mvr vlan via snmp. |

Runtime V1.3.7.2 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00624 | "ip source-guard mode mac" enabled, but only support 5 dynamic IP Source-guard binding |
| --- | --- |
| ES3510MA-FLF-38-00617 | [dhcp snooping] drop dhcp packet that packet length is less than 300 bytes |
| ES3510MA-FLF-38-00619 | ip allow zeros doesn't save after reload |
| ES3510MA-FLF-38-00582 | Enhancement: ip dhcp snooping limit rate as cisco |

Runtime V1.3.7.0 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00616 | Enhancement: Change DHCPSNP session from 5 to 8 per port |
| --- | --- |
| ES3510MA-FLF-38-00615 | dhcpsnp trust port send untagged dhcp request/lease packet even if it's in trunk mode |
| ES3510MA-FLF-38-00611 | Query won't be forwarded on "no mvr proxy mode" |
| ES3510MA-FLF-38-00614 | After enable ip source-gurad on port, i can't bind service-policy to port |
| ES3510MA-FLF-38-00610 | Enhancement: adding command “ip arp inspection validate dst-mac src-mac ip allow zeros” like in Cisco |
| ES3510MA-FLF-38-00613 | update loader may cause file system crash |
| ES3510MA-FLF-38-00612 | Enhancement: add to running configure |
| ES3510MA-FLF-38-00609 | After config dhcp snooping for specific vlan, and then remove it, save config and reboot. CPU 100% |
| ES3510MA-FLF-38-00608 | IPSG(ACL mode) works incorrectty. |
| ES3510MA-FLF-38-00601 | SNMP: remove switchManagementVlan, because we can use netConfigTable to configure management VLAN & IP address |
| ES3510MA-FLF-38-00583 | Enhancement: modify ACL to work with IP source guard |
| ES3510MA-FLF-38-00602 | Press ctrl+l in telnet/ssh session causes software watchdog timeout and the device will reboot |
| ES3510MA-FLF-38-00596 | SFP Port Shutdown need to TX disable |

Runtime V1.3.6.5 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00607 | IGMPSNP:add show mvr member and show ip igmp snoopoing group more option |
| --- | --- |
| ES3510MA-FLF-38-00606 | switch drops packet which with UDP source port 0 |

Runtime V1.3.6.3 ; Loader V1.0.0.5

|  | MVR: group is not deleted after host leave 2 second. |
| --- | --- |
|  | MVR: immediate-leave check source IP |
| ES3510MA-FLF-38-00605 | switch hanging if inject packet to port with ipv6 and RA guard. enable |

Runtime V1.3.6.2 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00604 | Clinet can't obtain IP from DHCP server if dhcp option 82 is enabled. |
| --- | --- |
| ES3510MA-FLF-38-00603 | command arg[1] is not null when command has no arg |
| ES3510MA-FLF-38-00592 | PC with Win XP in MAC vlan and dhcp snooping doesn't receive IP address from DHCP server |
|  | mvr proxy-query-interval can not save correct value to configure file |

Runtime V1.3.6.0 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00591 | Clock Timezone setting failed |
| --- | --- |
| ES3510MA-FLF-38-00600 | SNMP: Use SNMP shutdown port may cause exception |
| ES3510MA-FLF-38-00598 | IP Source-guard ACL Mode do not set VLAN and Interface range |
| ES3510MA-FLF-38-00597 | Can not set IP Source-guard on range of port |
| ES3510MA-FLF-38-00599 | [DAI] Log interval should be configured when DAI is disabled |
| ES3528MV2-FLF-EC-00244 | DHCP relay and snooping can't work at the same time |

Runtime V1.3.5.0 ; Loader V1.0.0.5

| ES3510MA-FLF-38-00531 | L2Switch: The MAC address entry still exist when the aging time is expired. |
| --- | --- |
| ES3510MA-FLF-38-00587 | command authorization |
| ES3510MA-FLF-38-00594 | during start of the switch is possible to communicate between ports in different VLAN |
| ES3510MA-FLF-38-00572 | Web: update web help pages |
| ES3510MA-FLF-38-00595 | need to clear the command setting by snmp(alias) |
| ES3510MA-FLF-38-00581 | Enhancement: IP-MAC binding is not limit on one port |
| ES3510MA-FLF-38-00593 | igmp max group can not work with mvr |
| ES3510MA-FLF-38-00562 | [ES3510MA_V1.3.4.3] Port description isn't shown in ifMIB in SNMP management. |
| ES3510MA-FLF-38-00588 | mvr profile doesn't use index to delete |
| ES3510MA-FLF-38-00538 | Voice vlan mode auto -> manaul then mac-address-table can't aging out |
| ES3510MA-FLF-38-00586 | Bootp can't binding in the dhcp snooping binding table |
| ES3510MA-FLF-38-00553 | WEB: Security > AAA > Authorization > Configure Server, the WEB page display blank when apply setting. |
| ES3510MA-FLF-38-00569 | Syslog: Coding for memset on SYSLOG_MGR_AddFormatMsgEntry_Smtp |
| ES3510MA-FLF-38-00520 | WEB: The IP page cannot be found when I quickly switch the action menu. |
| ES3510MA-FLF-38-00585 | Diag: wsextlb 1000M test fail for combo port |
| ES3510MA-FLF-38-00573 | show mvr - missing fields "MVR Proxy Switching, MVR Robustness Value" |
| ES3510MA-FLF-38-00570 | correction to spelling/grammer - show power-source-check |
| ES3510MA-FLF-38-00584 | [ES3510MA_V1.3.4.4] ES3510MA can not disable "mvr type source" by SNMP |
| ES3510MA-FLF-38-00579 | Enhancement: reply ARP request from different subnet |
| ES3510MA-FLF-38-00575 | Enhancement: Root change notification |
| ES3510MA-FLF-38-00576 | WebUI : Web GUI connect and change IP address may crash system (side effect from ES4650F-SW-FLF-38-00064) |
| ES3510MA-FLF-38-00580 | DHCP: CPU utilization is 100% which is occupied by DHCP task |
| ES3510MA-FLF-38-00574 | Diag: wsextlb can not configure different speed to test |
| ES3510MA-FLF-38-00568 | Syslog: Coding for memset on SYSLOG_MGR_AddFormatMsgEntry_Smtp |
| ES3510MA-FLF-38-00541 | Security > DoS Protection - corrections to spelling/grammar |
| ES3510MA-FLF-38-00563 | mvr won't reply report when recieved igmp max-resp time is 25s |
| ES3510MA-FLF-38-00567 | Big increase of CPU utilization after using switchport ingress-filtering |
| ES3510MA-FLF-38-00564 | [ES3510MA_V1.3.4.3] High CPU utilization after enabling switchport ingress-filtering function. |
| ES3510MA-FLF-38-00549 | show dos-protection - correction to spelling/grammar |
| ES3510MA-FLF-38-00547 | DoS Protection - corrections to spelling/grammar |
| ES3510MA-FLF-38-00540 | IP > IPv6 > Show Statistics - correction to spelling |
| ES3510MA-FLF-38-00546 | Security > Port Authentication (Configure Global) - correction to spelling/grammar |
| ES3510MA-FLF-38-00545 | Traffic > DiffServ (Configure Policy - Add Rule) - correction to spelling/grammar |
| ES3510MA-FLF-38-00543 | Security > Port Authentication - correction to spelling/grammar |
| ES3510MA-FLF-38-00548 | show watchdog - correction to spelling/grammar |
| ES3510MA-FLF-38-00561 | Add MVR proxy command |
| ES3510MA-FLF-38-00560 | Show wrong information of spanning tree in show tech-support |
| ES3510MA-FLF-38-00517 | L2SW: SFP port link status display abnormal. |
| ES3510MA-FLF-38-00558 | TC BPDU will be sent if we plug on/off the PC on the edge port of the switch |

Runtime V1.3.4.5 ; Loader V1.0.0.4

| ES3510MA-FLF-38-00578 | Enhancement: add new BID 3 for ECS3510-10PD |
| --- | --- |

Runtime V1.3.4.4 ; Loader V1.0.0.3

| Merge from ES4650F-SW-FLF-38 | System may crash when http connect and change IP address |
| --- | --- |

Runtime V1.3.4.3 ; Loader V1.0.0.3

| ES3510MA-FLF-38-00559 | ES3510MA http server isn't workable after switch bootup with "no ip http sercure-server" |
| --- | --- |
| ES3510MA-FLF-38-00554 | Enhancement: when startup by default configure and DHCP can not get IP, we will use one fixed IP |
| ES3510MA-FLF-38-00386 | add Mac address info in the Trap message of port security when there is MAC violation |
| ES3510MA-FLF-38-00557 | Enhancement: Simplify ARP Inspection setting in Config file Format |
| ES3510MA-FLF-38-00556 | there are duplicate port configuration in config file, unwanted configration is printout after the "end" |
| ES3510MA-FLF-38-00503 | [ES3510MA_V1.3.0.8] BPDU-filter lost after rebooting DUT |
| ES3510MA-FLF-38-00502 | aaa accounting: use command "no aaa accounting command 15" will crash the system |
| loader | Enhancement: When dual firmware master is failed, boot from backup firmware (for ECS3550-10PD) |

Runtime V1.3.4.2 ; Loader V1.0.0.2

| ES3510MA-FLF-38-00555 | SNMP: system will crash by SNMP shutdown |
| --- | --- |
| ES3510MA-FLF-38-00515 | AutoUpgrade: After opcode autoupgrade, DUT can't auto reload when reload status is enabled. |
|  | Remove DHCP option 82 duplicate node from private MIB |
| ES3510MA-FLF-38-00551 | Web: System->Console & Memory Status don't display any information |

Runtime V1.3.4.0 ; Loader V1.0.0.2

| ES3510MA-FLF-38-00552 | WEB&AAA: security-->AAA-->authorization-->configure service can't display |
| --- | --- |

Runtime V1.3.2.4 ; Loader V1.0.0.2

| ES3510MA-FLF-38-00544 | Web: update help pages |
| --- | --- |
| ES3510MA-FLF-38-00512 | UDLD: Sent 100 UDLD entries several times, CLI print debug message. |
| ES3510MA-FLF-38-00505 | Enhancement: SNMP OID for "show interface transceiver" |
| ES3510MA-FLF-38-00522 | WEB: disable port admin will cause CLI print a lot of strange log and WEB disconnect. |
| ES3510MA-FLF-38-00525 | WEB: ACL group can't bind to ethernet port via the WEB. |
| ES3510MA-FLF-38-00518 | AutoUpgrade: Can't perform auto upgrade on DUT port9 and port10. |
| ES3510MA-FLF-38-00521 | L2SW: The counter table of interface statistics can't cleared via the WEB. |
| ES3510MA-FLF-38-00504 | Enhancement: Display Port 1~8 Power source status |
| ES3510MA-FLF-38-00511 | Cluster: [show cluster candidate] cann't work. |
| ES3510MA-FLF-38-00535 | SNTP: sntp timezone name's range is 29 strings in WEB help description, but setting 30 strings is OK. |
| ES3510MA-FLF-38-00534 | SNTP: manually sntp setting is different from WEB help and CLI. |
| ES3510MA-FLF-38-00506 | DiffServ Class Name can input over 16 characters |
| ES3510MA-FLF-38-00527 | WEB: The maximum destination file name of opcode is 32 characters, but the help page description is 31 characters. |
| ES3510MA-FLF-38-00459 | LLDP: Disable LLDP admin status automatically on the port when the port is administratively disabled. |
| ES3510MA-FLF-38-00508 | We can't enable "DHCP snooping information option" by snmp |
| ES3510MA-FLF-38-00487 | Enhancement: add aaa accounting command |
| ES3510MA-FLF-38-00510 | UDLD: Device print exception message after change hostname to 255 characters. |
| ES3510MA-FLF-38-00413 | Enhancement: add RA Guard function |
| ES3510MA-FLF-38-00491 | [ACL] command tree issue |
| ES3510MA-FLF-38-00496 | [enhancement] support rate limit by rule when the chip cannot support accurately with register |

Runtime V1.3.2.2 ; Loader V1.0.0.2

| ES3510MA-FLF-38-00489 | ES3510MA can't set sfp-forced 100M on Web. |
| --- | --- |
| ES3510MA-FLF-38-00495 | [enhancement] support auto upgrade finish not reboot automatically |
| ES3510MA-FLF-38-00492 | [CLI] copy running-config issue |
| ES3510MA-FLF-38-00427 | Delay reload display wrong reload date in "reload in ..." |
| ES3510MA-FLF-38-00493 | [enhancement] Extend the command for logging on the remote host or not what can choose the destination udp port. |
| ES3510MA-FLF-38-00441 | system:It will display error message and didn't forwarding data when rebooting one DUT. |
| ES3510MA-FLF-38-00488 | MIB: diffServActionPktNewDscp and diffServActionStatus are in wrong node |
| ES3510MA-FLF-38-00416 | show lldp info remote-device - correction to spelling |
| ES3510MA-FLF-38-00466 | LLDP: Memory leaks occur when receiving a PDU which has many Port and Proto VLAN ID TLV of VLAN id 0. |
| ES3510MA-FLF-38-00501 | aaa accounting : use command " no aaa accounting exec 'ABC' " on global mode,DUT was hang up |
| ES3510MA-FLF-38-00500 | 45.2.11 LLDP capabilities test |
| ES3510MA-FLF-38-00497 | DHCP: doesn't work dhcp snooping with "information option" |
| ES3510MA-FLF-38-00429 | Enhancement: add private mib of DHCPV6SNP |
| ES3510MA-FLF-38-00490 | Wrong spelling of DHCPSNP rogue server attack |
| ES3510MA-FLF-38-00496 | [enhancemnt] support rate limit by rule wehn the chip cannot support accurately with register |
| ES3510MA-FLF-38-00503 | [ES3510MA_V1.3.0.8] BPDU-filter lost after rebooting DUT |

Runtime V1.3.2.1 ; Loader V1.0.0.2

| ES3510MA-PD-FLF-EC-00008 | Add CLI commands to enable/disable PSE check on ports with PD capability |
| --- | --- |
| ES3510MA-FLF-38-00480 | MVR: settop-box will send v3 report and won't reply g-s query |
| ES3510MA-FLF-38-00477 | LLDP:Can't show correct Neighbor Entries Dropped Count from Web page when receive 4 different LLDPs per port repeatedly. |
| ES3510MA-FLF-38-00482 | Port status can know the port shutdown is caused by BPDU guard. |
| ES3510MA-FLF-38-00484 | [ph2] StaticTrunk: The DUT displayed twice error message while configured TLVs of LLDP on the trunk interface. |
| ES3510MA-FLF-38-00486 | memory leakage when other querier exist and enable quereir |
| ES3510MA-FLF-38-00483 | [ph2] SNMP: The DUT could not get the value of "lldpXdot3LocPortTable" object on MIB browser. |
| ES3510MA-FLF-38-00479 | Enhancement: Add UDLD function |
| ES3510MA-FLF-38-00487 | Enhancement: add aaa accounting command |

Runtime V1.3.2.0 ; Loader V1.0.0.2

| ES3510MA-FLF-38-00476 | LLDP: DUT send LLDP PDU with wrong Aggregation status on IEEE 802.3 - Link Aggregation TLV for a trunk. |
| --- | --- |
| ES3510MA-FLF-38-00473 | LLDP:When configure LLDP port status of Static LACP as Disabled, peer static LACP can't clear LLDP information. |
| ES3510MA-FLF-38-00450 | ACL: When we bind service-policy ACL and ip source-guard on a port,the ACL won't work well. |
| ES3510MA-FLF-38-00470 | Enable remote log by web, CLI display some message as log ram. |
| ES3510MA-FLF-38-00469 | Web: System > Time > Configure Time Server - nothing displays. |
| ES3510MA-FLF-38-00456 | XSTP: BPDU filtering shall be operation disabled when oper edge is changed to disabled |
| ES3510MA-FLF-38-00437 | System:The clinets didn't receive multicast data of all groups when the traffic is over 2000p/s. |
| ES3510MA-FLF-38-00478 | [Diag] Enhancement request from MFG test engineer to add SFP test |
| ES3510MA-FLF-38-00471 | [Driver] Do not allow PHY link up if the connected link partner is not PoE PSE port |
| ES3510MA-FLF-38-00472 | [Flash] Need to support 32MB flash on ES3510MA-FLF-PD board(board id=2) |
| ES3510MA-FLF-38-00475 | Add BID for ES3510MA-PD and revise its name in web |

Runtime V1.3.0.8 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00367 | [DHCPSNP] Failed to write binding and forward dhcp ack when processing dhcp relay packets |
| --- | --- |
| ES3510MA-FLF-38-00471 | [enhance] support NTP (web page) |
| ES3510MA-FLF-38-00480 | MVR: settop-box will send v3 report and won't reply g-s query |

Runtime V1.3.0.7 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00468 | The port type name need to change for web UI. |
| --- | --- |
|  | Fix software watchdog problem when some functions are disable |
|  | DHCPSNP option remote-id doesn't work |

Runtime V1.3.0.6 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00422 | [ES3510MA_V1.2.4.7] Switch is changing DSCP marks without any configuration |
| --- | --- |
| ES3510MA-FLF-38-00459 | LLDP: Disable LLDP admin status automatically on the port when the port is administratively disabled. |
| ES3510MA-FLF-38-00458 | LLDP:When the port is removed from Trunk or LACP, the LLDP configuration on the port is not same as that on trunk or LACP. |
| ES3510MA-FLF-38-00464 | lldp protocol block the port of the SMC8126L2 |
| ES3510MA-FLF-38-00460 | LBD: can not function correctly |
| ES3510MA-FLF-38-00454 | LLDP:LLDP TLV configuration can't be saved. |
| ES3510MA-FLF-38-00462 | ES3528M - ARP packets go back through entering port when AI enabled |

Runtime V1.3.0.3 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00436 | DUT can upload new firmware via FTP , but system didn't reboot automatically . |
| --- | --- |
| ES3510MA-FLF-38-00443 | IPsubnet-based vlan : after add subnet base vlan and can't ping to pc successful |
| ES3510MA-FLF-38-00359 | corrections to grammar/spelling - show ip igmp snooping |
| ES3510MA-FLF-38-00440 | MIB: igmpsnoopfilterstatus and igmpsnoopprofilestatus didn't get-next |
| ES3510MA-FLF-38-00439 | Web: The word "TLVs" misspell by web. |
| ES3510MA-FLF-38-00395 | MIB: voiceVlanMgt & diffServMgt should add to MIB file |
| ES3510MA-FLF-38-00411 | Enhancement: add PPPoE Intermediate Agent function |
| ES3510MA-FLF-38-00382 | ACL :There are some function problems of ACL's precedence |
| ES3510MA-FLF-38-00433 | CLI: remove unuse command "test loop internal..." |
| ES3510MA-FLF-38-00424 | CLI command has error in negotiation |
| ES3510MA-FLF-38-00280 | BACKDOOR: support MARVELL Galtis shell |
| ES3510MA-FLF-38-00444 | [SNMP] Failed to get dhcpSnoopVlanEnable value |
| ES3510MA-FLF-38-00445 | Auto upgrade is failed when it need to pass through default-gateway |
| ES3510MA-FLF-38-00449 | MSTP: Get the MIB object xstInstancePortPathCost, it always is zero. |
| ES3510MA-FLF-38-00452 | [ES3528M_V1.4.14.1] crash problem |
| ES3510MA-FLF-38-00457 | [ES3528MO_V1.4.14.1] DHCPSNP will block packets which the IP-in-IP packets related to DHCP |
| ES3510MA-FLF-38-00447 | CLI: remove useless command "interface loopback 0" |
| ES3510MA-FLF-38-00446 | the HTTPS certificate is not for ES3510MA |
| ES3510MA-FLF-38-00402 | Enhancement: add Network Time Protocol (NTP) function |

Runtime V1.3.0.1 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00372 | ES3510MA can not local authentication when tacacs server disconnect |
| --- | --- |
| ES3510MA-FLF-38-00395 | MIB: voiceVlanMgt & diffServMgt should add to MIB file |
| ES3510MA-FLF-38-00407 | Enhancement: add software watchdog function |
| ES3510MA-FLF-38-00409 | Enhancement: add DHCP V6 snooping |
| ES3510MA-FLF-38-00371 | LLDP: add LLDP-MED function |
| ES3510MA-FLF-38-00413 | Enhancement: add RA Guard function |
| ES3510MA-FLF-38-00411 | Enhancement: add PPPoE Intermediate Agent function |
| ES3510MA-FLF-38-00370 | LAN: add rx_is_tagged flag to indicate if rx packet is tagged or untagged |
| ES3510MA-FLF-38-00406 | Enhancement: add Denial of Service (DoS) protection |
| ES3510MA-FLF-38-00374 | corrections to grammer/spelling - show interfaces switchport: 801.Q-tunnel |
| ES3510MA-FLF-38-00375 | PortUtil: change unit of octets rate from bytes/s to kbits/s |
| ES3510MA-FLF-38-00376 | (Enhance) "show interfaces status" provides the Port Upttime - the elapsed time since it got to linkup. |
| ES3510MA-FLF-38-00418 | Enhancement: CLI show more than one page, need to provide guideline |
| ES3510MA-FLF-38-00389 | add port seepd and duplex info into log when interface links up |
| ES3510MA-FLF-38-00396 | MIB: rateLimitMgt & portMgt nodes should be removed, atcMgt node should revise |
| ES3510MA-FLF-38-00424 | CLI command has error in negotiation |
| ES3510MA-FLF-38-00425 | NIC driver may block the system |
| ES3510MA-FLF-38-00366 | BPDU tunneling |
| ES3510MA-FLF-38-00280 | BACKDOOR: support MARVELL Galtis shell |
| ES3510MA-FLF-38-00328 | auto option supported by speed command in cli, but not in the web |
| ES3510MA-FLF-38-00414 | Enhancement: add "show spanning-tree brief" function |
| ES3510MA-FLF-38-00430 | no ip telnet server will cause CPU utilization become 100% |
| ES3510MA-FLF-38-00353 | corrections to grammar/spelling - show tacacs-server |
| ES3510MA-FLF-38-00358 | corrections to grammar/spelling - show qos map trust-mode interface ethernet |
| ES3510MA-FLF-38-00332 | corrections to grammar/spelling - show qos map trust-mode interface |
| ES3510MA-FLF-38-00295 | Non-STP Loopback detection implementation |
| ES3510MA-FLF-38-00431 | BPDU-guard may cause system crash |
| ES3510MA-FLF-38-00377 | Change users privileges like in 3528M:privilege exec level 0 show mac and Privilege Management |
| ES3510MA-FLF-38-00398 | Web UI: There are no web pages of ERPS, CFM, and OAM help |
| ES3510MA-FLF-38-00327 | ftp in copy command for cli, but not in web |
| ES3510MA-FLF-38-00401 | Enhancement: add FTP auto upgrade function |
| ES3510MA-FLF-38-00426 | Enhancement: Change some LLDP default value from false to true which is same as MO |
| ES3510MA-FLF-38-00423 | SFP only check present, don't need to check RX_LOSS |
| ES3510MA-FLF-38-00399 | RSTP strange behavior |
| ES3510MA-FLF-38-00384 | clear arp-cache via SNMP on ES3528M priority low |
| ES3510MA-FLF-38-00352 | corrections to grammar/spelling - show ip interface |
| ES3510MA-FLF-38-00397 | MIB: some igmpSnoopMgt nodes should be removed |
| ES3510MA-FLF-38-00419 | Some nodes can't work well by snmp.(igmpfilter,igmpprofile.filemgt) |
| ES3510MA-FLF-38-00421 | [ES3510MA_V1.2.4.7] MSL-ERROR after ip igmp snooping querier enabled/disabled |
| ES3510MA-FLF-38-00420 | [DHCPSNP] Binding entries will disappear sometimes |
| ES3510MA-FLF-38-00387 | DHCP packets wil modify Vlan after trapped to CPU if Mac Valn is enabled |
| ES3510MA-FLF-38-00388 | block IGMP query packets |
| ES3510MA-FLF-38-00415 | [CLUSTER] remove command's output modifier in simba platform |
| ES3510MA-FLF-38-00359 | corrections to grammar/spelling - show ip igmp snooping |
| ES3510MA-FLF-38-00393 | Support multiple mvr domain and mvr profile |
| ES3510MA-FLF-38-00383 | mvr query always carry null src ip |
| ES3510MA-FLF-38-00408 | Enhancement: Need the DAI objects in the private MIB to manage DAI via SNMP. |
| ES3510MA-FLF-38-00378 | [CLUSTER] cluster packet will not be traped to cpu after enable cluster |
| ES3510MA-FLF-38-00326 | Syslog:The command "(no) logging facility" didn't work. |
| ES3510MA-FLF-38-00275 | (155) [SNMP] - [Trap] Invalid Trap output. |

Runtime V1.2.4.7 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00390 | ImageDownload:TFTP download opcode with snmp will cause DUT crash. |
| --- | --- |
| ES3510MA-FLF-38-00391 | Web:HELP can't browse on the WEB. |
| ES3510MA-FLF-38-00392 | PortTrunk:Create port trunk and join member but 3rd party is default setting,then coneect DUT to 3rd party |
| ES3510MA-FLF-38-00394 | PortTrunk:It can't forward packet by load balance,and it has packet loss issue. |
| ES3510MA-FLF-38-00383 | mvr query always carry null src ip |
| ES3510MA-FLF-38-00294 | auto recovery feature for BPDU Guard on ES3528M priority low |
| ES3510MA-FLF-38-00373 | Mac address still aging out by "mac-address-table aging-time 0" |
| ES3510MA-FLF-38-00367 | [DHCPSNP] Failed to write binding and forward dhcp ack when processing dhcp relay packets |
| ES3510MA-FLF-38-00356 | corrections to grammar/spelling - show mac-address-table count interface ethernet |
| ES3510MA-FLF-38-00331 | corrections to grammar/spelling - show mac-address-table count interface |
| ES3510MA-FLF-38-00368 | add mvr group range command |
| ES3510MA-FLF-38-00369 | PortSecurity: when mac count is less than configured max mac count, port security should be disabled automatically |
| ES3510MA-FLF-38-00312 | [DHCP&DHCPSNP] Option 82 code enhancement |
| ES3510MA-FLF-38-00347 | Web: The filed 'Type' displayed in Security->IP Source Guard->Static Binding is different with CLI |
| ES3510MA-FLF-38-00365 | Web: update new wen help files |
| ES3510MA-FLF-38-00364 | WebUI add G.8032, OAM & CFM function and web pages |
| ES3510MA-FLF-38-00363 | NetworkAccess: MAC Filter ID range is 1-64, maximum MAC Filter is 65. |
| ES3510MA-FLF-38-00362 | corrections to grammar/spelling - web interface |
| ES3510MA-FLF-38-00325 | IPv6: CLI can be set invaild ipv6 address. |
| ES3510MA-FLF-38-00329 | correction to grammar/spelling - show interfaces status |
| ES3510MA-FLF-38-00330 | corrections to grammar/spelling - show auto-traffic-control |
| ES3510MA-FLF-38-00354 | corrections to grammar/spelling - show interfaces status ethernet |
| ES3510MA-FLF-38-00357 | corrections to grammar/spelling - show spanning-tree |
| ES3510MA-FLF-38-00351 | [DHCPSNP] support "clear ip dhcp snooping binding" command |
| ES3510MA-FLF-38-00335 | corrections to grammar/spelling - show lldp info remote-device detail |

Runtime V1.2.4.2 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00285 | Traffic Segmentation : The traffic-segmentation for uplink port can't be configured successfully. |
| --- | --- |

Runtime V1.2.4.1 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00348 | network access - mac filter id - can't be bound to port using web |
| --- | --- |
| ES3510MA-FLF-38-00349 | System - IP (Add) page is missing "Gateway IP Address" |
| ES3510MA-FLF-38-00338 | System:Use 3 DUT to do system test but all disable spanning tree then CLI displays exception. |
| ES3510MA-FLF-38-00339 | [DHCPv6] It will fail to send broadcast packet after getting ip from dhcpv6 server |
| ES3510MA-FLF-38-00340 | System:Use mib browser to "walk" ,CLI displays exception message and DUT crash. |
| ES3510MA-FLF-38-00341 | System:Use mib browser over ipv6 to "walk" , DUT will display exception message. |
| ES3510MA-FLF-38-00342 | [DHCPSNP] dhcp snooping process packet's speed is slow |
| ES3510MA-FLF-38-00344 | GVRP: When vlan id is greater than 256, the port enabled gvrp can't dynamic join vlan automatically. |
| ES3510MA-FLF-38-00345 | (simba)MVR forward report cost to much time |
| ES3510MA-FLF-38-00346 | add CLI command to setup algorithm for trunk by Poncat capability |

Runtime V1.2.4.0 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00288 | Add DDM function to get SFP information |
| --- | --- |
| ES3510MA-FLF-38-00324 | Console: remove error message in startup console |
| ES3510MA-FLF-38-00281 | Ping6 : The description for Ping6 count and size range will go wrong with "FE80::X%ZoneID" type of destination. |
| ES3510MA-FLF-38-00286 | Multicast : The Max number for static Multicast groups is mismatch between Web description and real implementation. |
| ES3510MA-FLF-38-00319 | STP: STP topology change include blocked port information in log |
| ES3510MA-FLF-38-00323 | [Cluster] CLUSTER_MGR_SendClusterPacket() doesn't free memory before function return |
| ES3510MA-FLF-38-00290 | Web_UI : DUT did not set password type (encrypted password or not ) via Web UI. |
| ES3510MA-FLF-38-00315 | CPU utilization can't be update when use https. |
| ES3510MA-FLF-38-00276 | CLI: It didn't have the ipv6 format to set when setting sntp server/syslog server/hosts in CLI, but allow to set. |
| ES3510MA-FLF-38-00322 | DHCP: "ip dhcp snooping information option remote-id " didn't save in running configure |
| ES3510MA-FLF-38-00277 | CLI:It display exception when no ipv6 address autoconfig |
| ES3510MA-FLF-38-00318 | STP: add block & shutdown functions in STP |
| ES3510MA-FLF-38-00320 | STP: can setup release time for release-mode |
| ES3510MA-FLF-38-00309 | [DHCPv6] It will fail to restart DHCPv6 on L2 switch |
| ES3510MA-FLF-38-00237 | Enhancement: Refine the ping backdoor. |
| ES3510MA-FLF-38-00313 | FXC3110(V1.1.0.7)-Issue#42: DUT doesn't send to IPv6 router solicitation on time with "ipv6 address autoconfig" |
| ES3510MA-FLF-38-00306 | UI/Ping: owner and test name, array size forgot to "+ 1" |
| ES3510MA-FLF-38-00292 | use SYS_ADPT_MAX_NBR_OF_PRIORITY_QUEUE_FOR_CPU_PORT in |
| ES3510MA-FLF-38-00297 | (IT2R-0035)[E-OAM] - [CFM] set lowestAlarmPri to 4, when rx a macstatus ccm, it still set its transmitting ccm's |
| ES3510MA-FLF-38-00314 | Customerinfo:Loader version error. |
| ES3510MA-FLF-38-00302 | (ST-0143)Use macro running VLAN&shutdown&noshutdown&#169; running_config, the CLI freeze |
| ES3510MA-FLF-38-00308 | Enhancement: VLAN behavior consistency |
| ES3510MA-FLF-38-00311 | [DHCPSNP] It will not forward packets to trust port |
| ES3510MA-FLF-38-00310 | [DHCPSNP] DHCP client may get ip but DUT doesn't write binding sometimes |
| ES3510MA-FLF-38-00304 | CLI on ECS4610-26T : port description doen't accept the description with SPACE |
| ES3510MA-FLF-38-00300 | SNMP walk won't stop |
| ES3510MA-FLF-38-00303 | ES3528M-SFP-38 after create vlan 311 and copy startup,it will stop after reboot |
| ES3510MA-FLF-38-00293 | ip dhcp snooping issue in show running |
| ES3510MA-FLF-38-00307 | DUT will crash when use HTTPS URL to connect it. |
| ES3510MA-FLF-38-00247 | FXC3110(V1.1.0.7)-Issue#46: There is no setting available for IPv6 ACL |
| ES3510MA-FLF-38-00301 | Check-in Marvell appDemo build environment for Galtis |
| ES3510MA-FLF-38-00296 | [DHCP] dhcp send request fail in syslog |
| ES3510MA-FLF-38-00298 | (IT2R-0043)[E-OAM] - [CFM] the port status and interface status of show ethernet cfm maintenance-points |
| ES3510MA-FLF-38-00278 | [enhnace]protocol vlan support priority |
| ES3510MA-FLF-38-00291 | UI Enhacement: access mode, access vlan, multiple vlan setting |
| ES3510MA-FLF-38-00289 | When System boot (A2 chip), the error message of DEV_SWDRV_CreateTrunk is occurred |
| ES3510MA-FLF-38-00287 | FXC3326(V1.1.0.1)-Issue#92: Operation Speed-Duplex problem after disabling and then enabling negotiation |
| ES3510MA-FLF-38-00273 | Web:The mail adress link of contact us is incorrect. |
| ES3510MA-FLF-38-00283 | Web : The Email contact info for Edgecore Products shows brandsupport@accton.com. |

Runtime V1.2.2.0 ; Loader V1.0.0.0

Runtime V1.2.1.1 ; Loader V1.0.0.0

| ES3510MA-FLF-38-00198 | ACL: Add the ACL into the DUT cause that webUI hang up. |
| --- | --- |
| ES3510MA-FLF-38-00241 | CLI: the system mtu jumbo command should be removed |
| ES3510MA-FLF-38-00239 | FXC3110(V1.1.0.1)-Issue#15: Rate limit setting problems |
| ES3510MA-FLF-38-00226 | MAC Aging-Time : There should not be two MAC aging-time values when configure the aging-time as 0. |
| ES3510MA-FLF-38-00245 | FXC3110(V1.1.0.7)-Issue#60: Queue weight setting is missing after reload |
| ES3510MA-FLF-38-00203 | When we use clustering, we can’t access some menus of candidate switch |
| ES3510MA-FLF-38-00231 | Web: Only the IP Address 1 can be configured on the DNS Static Host Table web page. |
| ES3510MA-FLF-38-00255 | Web-Auth: After disable/enable global configuration, web-auth has incorrect behavior. |
| ES3510MA-FLF-38-00232 | Web : The DNS Static Host IP address can be set as illegal IP addresses. |
| ES3510MA-FLF-38-00225 | Web : The Log Remote Server IP can be set as illegal IP addresses. |
| ES3510MA-FLF-38-00259 | IPv6-ReadyLogo-HostMode: address autoconfig was disabled after link-local address changed. |
| ES3510MA-FLF-38-00262 | CLI: change default exec. timeout of console from 0 to 600 |
| ES3510MA-FLF-38-00253 | [IP Statistics] ICMP sent echo request counter always zero in "show ip traffic" command |
| ES3510MA-FLF-38-00252 | Cable diagnostic incorrect test procedure may cause CLI crashed |
| ES3510MA-FLF-38-00240 | Web UI, Traffic->DiffServ is unnecessary to setup Action in Add Rule |
| ES3510MA-FLF-38-00248 | FXC3110(V1.1.0.6)-Issue#40: IP source-guard filter cannot be changed in port 10 if trunk port is present |
| ES3510MA-FLF-38-00246 | FXC3110(V1.1.0.7)-Issue#53: QoS trust mode setting problem in Web if there is 100M port added to trunk |
| ES3510MA-FLF-38-00224 | CLI : CLI will be no response after a long duration real-world testing. |
| ES3510MA-FLF-38-00268 | ES3510MA: Switchport mode access is not stored into running config |
| ES3510MA-FLF-38-00242 | CLI: Change the default value of Inactive timeout to 600 seconds |
| ES3510MA-FLF-38-00269 | fs.c may not free memory or close file in error status. |
| ES3510MA-FLF-38-00270 | [DHCP option82] It should unicast reply packet if RID is matched |
| ES3510MA-FLF-38-00249 | FXC3110(V1.1.0.6)-Issue#39: IP source-guard show incomplete information if there is trunk port on switch |
| ES3510MA-FLF-38-00264 | ES3510MA tacacs+ issue |
| ES3510MA-FLF-38-00235 | Web: tacacs-serve timeout can be set on CLI, but can't be set on Web. |
| ES3510MA-FLF-38-00271 | Traffic Segmentation: wrong CLI command: uplink-to-uplink |
| ES3510MA-FLF-38-00265 | G.8032 support hardware interrupt to improve the performance |
| ES3510MA-FLF-38-00201 | backdoor passwd should be encrypted |
| ES3510MA-FLF-38-00202 | At a reload, logging entries in RAM are cleared and 'coldStart' is relogged. |
| ES3510MA-FLF-38-00250 | FXC3110(V1.1.0.6)-Issue#33: mac-learning cannot be disabled in both CLI and WebUI |

Runtime V1.2.0.1 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00236 | ipv6readylogo: ping6 to multicase address maybe halt the CLI |
| --- | --- |
| ES3510MA-FLF-38-00234 | Radius:Timeout of Radius setting have different range in Web UI and Console |

Runtime V1.2.0.0 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00207 | [Enhancement---P2.5] 802.3ah |
| --- | --- |
| ES3510MA-FLF-38-00206 | Enhancement---P2.5] 802.1ag |
| ES3510MA-FLF-38-00204 | [Enhancement---P2.5] G.8032 |
| ES3510MA-FLF-38-00205 | [Enhancement---P2.5] Y.1731 |
| ES3510MA-FLF-38-00228 | In auto mode, insert 100 SFP, port; type is 100. unplug 100 SFP and insert 1000 SFP, port type still is 100 |
| ES3510MA-FLF-38-00223 | Web : The word "to to" should be only one "to" |
| ES3510MA-FLF-38-00222 | Web : The word "importsuser" should be separated to "imports user". |
| ES3510MA-FLF-38-00221 | Web : The word "definesthe" should be separated to "defines the". |
| ES3510MA-FLF-38-00220 | Web : The word "clock" spelling goes wrong. |
| ES3510MA-FLF-38-00219 | Web : Go through System->Memory Status page and press "Help" link (on the upper right corner), the help |
| ES3510MA-FLF-38-00216 | SSL and Port security causing web management problems and STP instability |
| ES3510MA-FLF-38-00227 | Client can’t get multicast packet from certain multicast group address |
| ES3510MA-FLF-38-00210 | How to manage the LLDP via SNMP for ES3528M |
| ES3510MA-FLF-38-00215 | CLI:no any port or trunk can join RSPAN VLAN, so the column"Ports/Port Channels:" should be removed" |
| ES3510MA-FLF-38-00214 | [DHCPv6] Repeated enable and disable IPv6 address autoconfig cause exception |
| ES3510MA-FLF-38-00153 | telnet can not enter runtime backdoor |
| ES3510MA-FLF-38-00125 | [Enhance-p2] Traffic segmentation support multiple session |
| ES3510MA-FLF-38-00076 | support port isolation per VLAN in ES3510MA |
| ES3510MA-FLF-38-00196 | Accton:LN-2010-01-02 Link failure issue - WSS 2380 & HP Server |
| ES3510MA-FLF-38-00213 | ipv6ready: For IPv6 Ready Logo Host Mode Test |
| ES3510MA-FLF-38-00212 | CLI: the "size" and "count" variables cannot be specified when ping6 link-local addresses |
| ES3510MA-FLF-38-00211 | CLI:The description of error message about RSPAN should express obviously |
| ES3510MA-FLF-38-00200 | LLDP don't send out packet when port shutdown |
| ES3510MA-FLF-38-00208 | UnknownUnicastStorm: should use "switchport unknown-unicast" instead of "switchport unicast" |
| ES3510MA-FLF-38-00199 | IGMPSNP:It should be able to create 64 static multicast group. not 16. |

Runtime V1.1.4.6 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00254 | [Enhancement] Multiple management ip support for L2 product |
| --- | --- |

Runtime V1.1.4.5 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00238 | ES3510MA: Traffic-segmentation command is not correctly stored into running/startup config |
| --- | --- |

Runtime V1.1.4.4 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00233 | In auto mode, insert 100 SFP, port; type is 100. unplug 100 SFP and insert 1000 SFP, port type still is 100 |
| --- | --- |
| ES3510MA-FLF-38-00217 | The CLI will show lots of message and hang after configuring the media-type to sfp-forced mode |
| ES3510MA-FLF-38-00230 | [combo] sfp-prefer does not work while connecting copper & plugging gbic without fiber line |

Runtime V1.1.4.3 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00193 | web: Web UI did not set phb queue value in interface. |
| --- | --- |
| ES3510MA-FLF-38-00197 | port mirroring error may cause system freeze |

Runtime V1.1.4.2 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00181 | After user configures DAI setting then inject ARP packet to switch then exception will happen |
| --- | --- |
| ES3510MA-FLF-38-00183 | Trunk:Create a static trunk,commmand "sh run" ,it displays "lacp" word in running config. |
| ES3510MA-FLF-38-00142 | (Enhance) Add a hot-key in "a" so that "show running" will dispaly all output wihtout pauses. |
| ES3510MA-FLF-38-00134 | FR: "clear ip igmp snooping groups", to delete IGMP snooping multicast groups |
| ES3510MA-FLF-38-00167 | Port 1/10 counter is incorrect |
| ES3510MA-FLF-38-00115 | AccessModePVID: Can set PVID to non-existing VLAN member. |
| ES3510MA-FLF-38-00182 | Vlan Trunking: Can't check per trunk "vlan trunking" status via "show int status port-channel" command. |
| ES3510MA-FLF-38-00185 | 802.1X : 802.1x client(for windows) Authenticating fail via DUT. |
| ES3510MA-FLF-38-00186 | ssh : DUT can not disconnect over session ID 4. |
| ES3510MA-FLF-38-00187 | ssh : when ssh version 1.Clien can not open 8 account. |
| ES3510MA-FLF-38-00184 | ImageDownload: The automatic opcode upgrade operation can't work properly. |
| ES3510MA-FLF-38-00188 | Log:The log time is not the same with current time when setting time zone |
| ES3510MA-FLF-38-00189 | Log-in:The web must re-login when change forced speed |
| ES3510MA-FLF-38-00052 | The automatic Opcode Upgrade Path description on SNMP description is incorrect. |
| ES3510MA-FLF-38-00191 | CLI:it should display correct warning message when no rspan session |
| ES3510MA-FLF-38-00192 | RSPAN:Operation status is down when it only set port 9 and port 10 as destination/intermediate uplink port |
| ES3510MA-FLF-38-00194 | Cos : When DUT queue weight set as 1 :1:8:8. The package have forward incorrect ratio. |
| ES3510MA-FLF-38-00190 | ARP: DUT will crash and didn't access web when learning arp packets |
| ES3510MA-FLF-38-00195 | tftp copy file error, but no any error message |

Runtime V1.1.4.0 ; Loader V0.0.1.5

| ES3510MA-FLF-38-00125 | [Enhance-p2] Traffic segmentation support multiple session. |
| --- | --- |
| ES3510MA-FLF-38-00153 | telnet can not enter runtime backdoor |
| ES3510MA-FLF-38-00062 | Traceroute: The output of the traceroute command is not correct when the destination cannot be reached. |
| ES3510MA-FLF-38-00079 | MAC-Auth: if the MAC-auth port receives a large amount of traffic, the DUT will not be reachable |
| ES3510MA-FLF-38-00078 | [XSTP]To implement "13.25.1 allSynced" according to IEEE Std 802.1Q-2005. |
| ES3510MA-FLF-38-00077 | (Enhance) Compatible with Cisco prestandard MST implementation in IOS Software earlier than V12.2(25)SED. |
| ES3510MA-FLF-38-00050 | DUT doesn't display any better error message to explain the operation failure when Opcode file not created in flash |
| ES3510MA-FLF-38-00056 | Vlan Trunking:Can't configure "vlan trunking function" on FE port and trunk interface. |
| ES3510MA-FLF-38-00088 | LGN: Web "Interface > Port > Cable Test", add a note that reminds users to wait several seconds and click the refresh button |
| ES3510MA-FLF-38-00087 | (Enhance) In order to have better EMI margin for FE models, it is to update MAC Register 0x48 value. |
| ES3510MA-FLF-38-00081 | ARP Cache: The actual timeout is measured around 2 minutes, however, it shall be 1200 seconds at default |
| ES3510MA-FLF-38-00089 | System: Inject line speed DA as CPU MAC packets to port with mac-authentic mode will result to corruption of rx packet |
| ES3510MA-FLF-38-00059 | (Enhance) Dump stack frame and log messages on task exception |
| ES3510MA-FLF-38-00084 | Receiving VRRP packets from the HP switch in the topology cause ping loss on ES3510MA |
| ES3510MA-FLF-38-00082 | Accton: LN-2009-12-01 VRRP operation problem for ping loss. |
| ES3510MA-FLF-38-00094 | It always fails to delete default gateway |
| ES3510MA-FLF-38-00069 | DHCP relay: it can't find is in CLI and web. : DHCP relay information option 82 needed |
| ES3510MA-FLF-38-00070 | Cluster : cluster bug of 4093 vlan. |
| ES3510MA-FLF-38-00085 | (Enhance) Web "IP > ARP" for ARP Cache timeout and entries. |
| ES3510MA-FLF-38-00090 | WebUI: The PING result is different on the CLI and the WebUI. |
| ES3510MA-FLF-38-00091 | WebUI: Both combo port link up even only connect copper cable. |
| ES3510MA-FLF-38-00083 | Single image use ram.bix in the file system for same file name issue |
| ES3510MA-FLF-38-00080 | Port linkdown is possibly caused by cable dis-/re-connections when power-saving is enabled. |
| ES3510MA-FLF-38-00096 | LGN: (Enhance) Cable Test result: change "Test failed" to "Not supported". |
| ES3510MA-FLF-38-00095 | extend the length of username and password to 32 |
| ES3510MA-FLF-38-00086 | WebUI: The flow control status is incorrect when auto-negotication be enabled. |
| ES3510MA-FLF-38-00098 | controls the SSSMII interface, with 0x1f8007e3, the waveform looks better and has better EMI result. |
| ES3510MA-FLF-38-00100 | MVR: MVR will send out v3 query and don't accespt TO_EX{} |
| ES3510MA-FLF-38-00099 | ES3510MA cant save in flash for R01 units |
| ES3510MA-FLF-38-00093 | (Enhance) No need for the "POST Result: " caption on the CLI for Diagnostics Fast mode. |
| ES3510MA-FLF-38-00102 | IGMP_SNP_Filter:Send join report which include isn't configured in profile will cause error message in CLI. |
| ES3510MA-FLF-38-00068 | TCP Friendly : the rate-limit result is not closed for the CLI setup. |
| ES3510MA-FLF-38-00106 | MVR:Client use VLC to receive multicast data has packet loss and lag issue. |
| ES3510MA-FLF-38-00110 | LLDP: cli read management address fail |
| ES3510MA-FLF-38-00107 | IP Source-Guard:it will display error message and drop all packets whatever it match when it change filter type |
| ES3510MA-FLF-38-00105 | IP Source-Guard:it will display error message and drop all packets whatever it match when it change filter type |
| ES3510MA-FLF-38-00113 | TCP Friendly : the rate-limit result is not closed for combo port |
| ES3510MA-FLF-38-00109 | MVR: jion report on reciever port, but reprot won't be forward to source port |
| ES3510MA-FLF-38-00112 | Web: Traffic->Priority->Queue, Queue mode sould no has DRR |
| ES3510MA-FLF-38-00108 | Click the customer logo, the browser didn't open a new session to connect to customer's web site. |
| ES3510MA-FLF-38-00116 | LLDP Remote Device Information - web |
| ES3510MA-FLF-38-00117 | lldp - remote device info (remote max frame size) - web |
| ES3510MA-FLF-38-00118 | Spanning Tree - Configure Global (MSTP) error on web page |
| ES3510MA-FLF-38-00111 | Web may crash in Spanning Tree > STA (Configure Global - Configure) => Set to MSTP, then Select "Add Member" |
| ES3510MA-FLF-38-00119 | Web: "Spanning Tree > MSTP, Add Member" will cause memory dump (web_proc) on the console. |
| ES3510MA-FLF-38-00103 | IGMP_SNP_Filter:When profile ID is the maximum value:4294967295, can't bind profile on a port by web. |
| ES3510MA-FLF-38-00120 | (Enhance) "show memory" - Used plus Free will be equal to the Total DRAM size on the board. |
| ES3510MA-FLF-38-00114 | CLI: no "show memory" & "show access-list tcam-utilization" and should remove "show power" |
| ES3510MA-FLF-38-00121 | no interface vlan 1 system hang or interface vlan 2 ping will fail |
| ES3510MA-FLF-38-00124 | [Enhance-p2] support MVR groups from 255 to 1024 |
| ES3510MA-FLF-38-00101 | MVR:It doesn't display receiver port join group information correctly. |
| ES3510MA-FLF-38-00046 | IGMP Throttling: When max multicast group is higher or equal 255, throttling status is still "False". |
| ES3510MA-FLF-38-00158 | MLDSNP: Dynamic joined group doesn't be cleared when timer expired. |
| ES3510MA-FLF-38-00156 | ES3510MA config issue |
| ES3510MA-FLF-38-00138 | (Enhance) "show running-config" can be specified in interface, {ehternet ¦ port-channel ¦ vlan }. |
| ES3510MA-FLF-38-00141 | (Enhance) IP Cluster: The 3-octet OUI shall be moved to SysInclude. |
| ES3510MA-FLF-38-00140 | (DCR) DHCPSNP: Rouge server attack, log/trap in its IP address instead of the MAC. |
| ES3510MA-FLF-38-00155 | Enhancemane: L2 ipcfg and rif active notification |
| ES3510MA-FLF-38-00133 | FR: EAPOL Forwarding/Transparent |
| ES3510MA-FLF-38-00146 | [Enhance-p2] sh ip igmp snooping group instead of use sh mac-address-table multicast |
| ES3510MA-FLF-38-00145 | IGMP snooping new feature : Leave-Proxy and Immediate-Leave |
| ES3510MA-FLF-38-00150 | (Enhance) Add priority option at configuring MVR/IGMPSNP |
| ES3510MA-FLF-38-00154 | [Enhance-p2] CLI: MVR group setting can be seperated |
| ES3510MA-FLF-38-00143 | (Enhancement) Enlarge ACL/DiffServ OM capacities in order to fit user applications. |
| ES3510MA-FLF-38-00122 | [Enhance-p2] support 4K VLAN |
| ES3510MA-FLF-38-00131 | [XSTP]To implement "13.25.1 allSynced" according to IEEE Std 802.1Q-2005. |
| ES3510MA-FLF-38-00123 | [Enhance-p2] Spanning tree loopback detection can configure port disable |
| ES3510MA-FLF-38-00072 | ACE , ACL and other setting need to follow ES3528MO |
| ES3510MA-FLF-38-00144 | DCR: One service policy per port for ES3552M product (Current 32 policy maps in total) |
| ES3510MA-FLF-38-00137 | (Enhance) "show spanning-tree" can be specified to only display stp-enabled ports. |
| ES3510MA-FLF-38-00135 | Enlarge the max length of VLAN name from 32 to 128 |
| ES3510MA-FLF-38-00148 | (Enhance) DHCP: DHCP option 82 remote ID can be configured to be Mac or IP |
| ES3510MA-FLF-38-00127 | [Enhance-p2] IP source guard: let user can configure max. binding entry |
| ES3510MA-FLF-38-00157 | FXC3110(V1.1.0.1)-Issue#11: Console shows crash messages if SNMP-server host is configured and manage through webUI |
| ES3510MA-FLF-38-00139 | Enlarge the class statement in policy-map command |
| ES3510MA-FLF-38-00097 | LGN: (Enhance) DSCP marking/remarking |
| ES3510MA-FLF-38-00151 | WEB interface: Administartion>LLDP>Local Device Information: should not has router |
| ES3510MA-FLF-38-00161 | ComboSFP linkup/linkdown every 127sec and cause traffic break awhile |
| ES3510MA-FLF-38-00130 | [Enhance-p2] CLI "show interfaces brief" |
| ES3510MA-FLF-38-00092 | (Enhance) CLI "show interfaces brief" - brief in one line per interface. |
| ES3510MA-FLF-38-00132 | (Enhance) CLI: At a time configure static ip address of the switch, and optionally ip address of the default gateway. |
| ES3510MA-FLF-38-00159 | ES3510MA packets loss |
| ES3510MA-FLF-38-00136 | Enhance BPDU forwarding |
| ES3510MA-FLF-38-00149 | [Enhance-p2] show System resources, CPU utilization and memory status, on the WebGUI and SNMP |
| ES3510MA-FLF-38-00147 | [Enhance-p2] combo port un-plug & plug which will cause some setting reset (change) |
| ES3510MA-FLF-38-00160 | CLI: provide speed 100fx/1000sfp setting for SFP |
| ES3510MA-FLF-38-00128 | [Enhance-p2] support Delay reload |
| ES3510MA-FLF-38-00152 | CLI: ES3510MA doesn't has PoE function, so it should not has "power" command |
| ES3510MA-FLF-38-00066 | Need to support show techsupport and debug |
| ES3510MA-FLF-38-00052 | The automatic Opcode Upgrade Path description on SNMP description is incorrect. |
| ES3510MA-FLF-38-00166 | show runing mrd config always exist and query-interval confige command is wrong |
| ES3510MA-FLF-38-00168 | cli_runcfg, always dispaly enble mrd and proxy-query-inteval |
| ES3510MA-FLF-38-00169 | MVR won't reply report when received query |
| ES3510MA-FLF-38-00174 | ES3528M: help with diagnosis of management freezing |
| ES3510MA-FLF-38-00171 | IP source guard: configuration from snmp doesn't work |
| ES3510MA-FLF-38-00162 | [DHCPSNP information option, DHCP relay option]It should release mref if add/remove option82 fail. |
| ES3510MA-FLF-38-00104 | Help:IGMP Snooping max group range description is not correct on web help. |
| ES3510MA-FLF-38-00175 | copy file run the second time will failed |
| ES3510MA-FLF-38-00170 | Cluster: It display invalid data when diable cluster via web |
| ES3510MA-FLF-38-00165 | WebUI: show memory size from 123456789 to 123,456,789 |
| ES3510MA-FLF-38-00180 | LGN: DiffServ: policy-map does not have packets set ip dscp correctly. |
| ES3510MA-FLF-38-00126 | [Enhance-p2] support telent server from 4 to 8 session |
| ES3510MA-FLF-38-00164 | Enhancement: IP: "show ip interface" may remove "via XX-XX-XX-XX-XX-XX" and "Bandwidth: 1g" |
| ES3510MA-FLF-38-00163 | CLI: remove useless messages in "sh ip in" |
| ES3510MA-FLF-38-00179 | FXC3110(V1.1.0.1)-Issue#21: Show running config shows jumbo frames is 10240 which is not equal to user manual description |
| ES3510MA-FLF-38-00173 | SNMP: Can not setup ACL in MIB prioAclToCosIfIndex |
| ES3510MA-FLF-38-00129 | [Enhance-p2] support select QinQ |
| ES3510MA-FLF-38-00178 | IPConfiguration:Set invalid subnet mask:0.0.255.255 without error message,it will be changed to 255.255.0.0 automatically. |
| ES3510MA-FLF-38-00176 | XSTP: loopback detection does not work |
| ES3510MA-FLF-38-00172 | Enhancement: Check invalid gateway address for cmd: "ip address X X default-gateway X" |
| ES3510MA-FLF-38-00177 | IPConfiguration:Change DUT's IP via mib will cause user can't connect to mib and web. |
| ES3510MA-FLF-38-00181 | After user configures DAI setting then inject ARP packet to switch then exception will happen |
| ES3510MA-FLF-38-00183 | Trunk:Create a static trunk,commmand "sh run" ,it displays "lacp" word in running config. |
| ES3510MA-FLF-38-00142 | (Enhance) Add a hot-key in "a" so that "show running" will dispaly all output wihtout pauses. |
| ES3510MA-FLF-38-00134 | FR: "clear ip igmp snooping groups", to delete IGMP snooping multicast groups |
| ES3510MA-FLF-38-00167 | Port 1/10 counter is incorrect |
| ES3510MA-FLF-38-00115 | AccessModePVID: Can set PVID to non-existing VLAN member. |

Runtime V1.1.2.0 (1.1.0.4) ; Loader V0.0.1.5

| ES3510MA-FLF-38-00099 | can't save config. file in flash for R01 units |
| --- | --- |
| ES3510MA-FLF-38-00098 | HW request, controls the SSSMII interface, MAC reg 0x48 with 0x1f8887e3, the waveform looks better and has better EMI result. |
| ES3510MA-FLF-38-00095 | extend the length of username and password to 32 |
| ES3510MA-FLF-38-00084 | Receiving VRRP packets from the HP switch in the topology cause ping loss on ES3510MA |

Known Issue

Image V1.1.2.0 can only upgrade opcode by same file name in runtime. So, you must use the same file name as ES3510MA-FLF-38_V1.1.2.0.bix.

Runtime V1.1.0.1 ; Loader V0.0.1.4

| ES3510MA-FLF-38-00059 | (Enhance) Dump stack frame and log messages on task exception |
| --- | --- |
| ES3510MA-FLF-38-00061 | LGN: (Enhance) VLAN: switchport access mode - one single untagged |
| ES3510MA-FLF-38-00062 | Traceroute: The output of the traceroute command is not correct when the destination cannot be reached |
| ES3510MA-FLF-38-00063 | Show IP interface can show many interface, but this is L2 Switch, do we need to show Loopback 0 AND vlan 2 |
| ES3510MA-FLF-38-00064 | No show processor in console ? There is CPU utilization chart in web interface |
| ES3510MA-FLF-38-00065 | Please support auto edge, otherwise PC connect to switch will take 30 sec to become forwarding. |
| ES3510MA-FLF-38-00067 | Need to support copy and paste configure file to console |
| ES3510MA-FLF-38-00071 | MVR : Multicast vlan(source) port in the other (receriver) port is untagged. but the set is tagged. So the VLC |
| ES3510MA-FLF-38-00073 | Rule: Add relocation PCL code for the fail-safe when allocating PCE. |
| ES3510MA-FLF-38-00074 | DAI: can't forward arp packet to mac-based vlan, protocol-vlan |
| ES3510MA-FLF-38-00075 | GVRP issue sync from ES3528MO |
| ES3510MA-FLF-38-00079 | MAC-Auth: if the MAC-auth port receives a large amount of traffic, the DUT will not be reachable |
| support | show techsupport |

Known Issue

Image older than V1.1.2.0 (loader is 0.0.1.4) can not save configure file and upgrade opcode from runtime. Please contact our technical support to help you.

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

- Ingress TCP Rate limit: If you need accurate ratio of ingress TCP rate limit, you can use ACL rule plus DiffSev way to manage the bandwidth of input.

- The jumbo frame size is 10240 which is different with manual and html help file. User can not change the size. (ES3510MA-FLF-38-00179)

- Vlan trunking can be supported on all ports. The manual is incorrect on “VLAN trunking can only be enabled on Gigabit Ethernet port or trunks”

- VLAN mirroring or MAC-based mirroring can not coexist with Port mirroring. While VLAN mirroring and MAC-based mirroring can be coexisted, but they must use the same target port.

- MAC Chip limit: unicast, multicast, broadcast and storm control all use one same threshold, although each one can enable/disable independent. If user need to use different threshold for each one, he must use rule (diffserv) to do it.

- PHY chip limit: Cable Test on Power Saving is highly inaccurate. If you want to get accurate cable test, please turn off power saving mode first.

- PHY chip limit: GE port can do cable diagnostic in linkup, but FE & GE ports can only do cable diagnostic in linkdown without partner

- PHY chip limit: Power saving (green Ethernet) is work for GE port only.

- MAC chip limit: There is no HW loopback detection without STP (loop-free detection)

- MAC chip limit: Can not support multiple uplink ports on one session

- IGMPSNPv3 ASM: we only recognize IGMP V3 report without src_ip_list

- No mac-learning command for CLI and webUI, the manual is incorrect.

- An ACL can contain 64 rules, but the value in manual is wrong

- Web UI only support IE 7 and above version.